3GPP TSG-RAN WG4 Meeting #114	R4-2501383

Athens, Greece, 17-21 Feb 2025

Title: 	TP for TR 36.719-01-01, addition of CA_3A-8A-20A-28A-38A

Agenda Item:	6.4.2

Source: 	Nokia

Document for:	Approval

# Introduction

This TP for TR 36.719-01-01 adds CA_3A-8A-20A-28A-38A.

****************************************** Start of TP ******************************************

## 5.2	LTE-A inter-band CA for x (x>1) bands DL with 1 band UL

### 5.2.x	CA_3A-8A-20A-28A-38A

#### 5.2.x.1	Channel bandwidths per operating band for CA

Table 5.2.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

Table 5.2.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-8A-20A-28A-38A | - | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 8 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

#### 5.2.x.2	Co-existence studies

Not applicable for this CA configuration.

#### 5.2.x.3	∆TIB and ∆RIB values

Table 5.2.x.3-1: IB,c

| CA_3-8-20-28-38 | 3 | 0.6 |
| --- | --- | --- |
|  | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |
|  | 38 | 0.5 |

Table 5.2.x.3-2: R IB,c

| CA_3-8-20-28-38 | 3 | 0.2 |
| --- | --- | --- |
|  | 8 | 0.2 |
|  | 20 | 0.1 |
|  | 28 | 0.2 |
|  | 38 | 0.2 |

#### 5.2.x.4	REFSENS requirements

Not applicable for this CA configuration.

****************************************** End of TP ******************************************