# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501448

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	(NR_NTN_enh-Core) Discussion and draft replied LS on the FR2-NTN UE capabilities for 200Mhz channel bandwidth and 2 MIMO layers

Agenda Item:	5.21.1

Document for:	Approval

Introduction

In this meeting, RAN4 received the LS [1] from RAN2 and was asked whether UE capabilities for 200Mhz channel bandwidth and 2 MIMO layers are mandatory for FR2-NTN bands as below.

At RAN2#128 it has been discussed UE capabilities for FR2-NTN bands. RAN2 would like to check with RAN4 whether UE capabilities for 200Mhz channel bandwidth and 2 MIMO layers are mandatory for FR2-NTN bands.

In this paper, we’d like to discuss the issue raised by RAN2 and provide the draft replied LS.

Discussion

In the Rel-18, RAN4 introduced the Ka bands as FR2-NTN features. According to the NOTEs specified in table 5.1-1 of TS 38.101-5 [2], it means that FR2-NTN bands can refer to the features / capabilities specified for TN FR2-1 bands unless otherwise stated.

- 5 Operating bands and channel arrangement - 5.1 General The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present Release of specifications. NOTE: Other operating bands and channel bandwidths may be considered in future Releases. Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NTN satellite can operate according to this version of the specification are identified as described in Table 5.1-1. Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN (Note 1); 410 MHz – 7125 MHz. FR2-NTN (Note 2); 17300 MHz – 30000 MHz. NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated..

For TN FR2-1 bands, it’s mandatory for UE to support the DL two MIMO layers in FR2 and UL/DL 200MHz in FR2 according to the following perCCperFS capabilities specified in TS 38.306.

Observation 1: For TN FR2-1 bands, it’s mandatory for UE to support the DL two MIMO layers in FR2 and UL/DL 200MHz in FR2 according to the following perCCperFS capabilities specified in TS 38.306.

For FR2-NTN Ka band VSAT, there is no issue observed to mandatorily support the UL/DL 200MHz channel bandwidth as the similar baseband capabilities for TN can be reused for FR2-NTN implementation.

Proposal 1: UE capabilities for 200Mhz channel bandwidth are mandatory for FR2-NTN bands.

For FR2-NTN scenarios, most of them are for direct path communications between SAN and VSAT. Thus, it’s very hard for VSAT to achieve the communication with one satellite by using two DL MIMO layers. Apart from that, UE need to afford the additional implementation costs for two DL MIMO layers, e.g. two simultaneous receiving beams. In total, the situation is different between FR2 TN UE and FR2-NTN Ka band VSAT for two layers DL MIMO. Thus, it’s better to consider not mandatory support of two layers DL MIMO for FR2-NTN Ka band VSAT.

Proposal 2: UE capabilities for 2 MIMO layers are not mandatory for FR2-NTN bands.

Summary

Observation 1: For TN FR2-1 bands, it’s mandatory for UE to support the DL two MIMO layers in FR2 and UL/DL 200MHz in FR2 according to the following perCCperFS capabilities specified in TS 38.306.

Proposal 1: UE capabilities for 200Mhz channel bandwidth are mandatory for FR2-NTN bands.

Proposal 2: UE capabilities for 2 MIMO layers are not mandatory for FR2-NTN bands.

- References

[1] R2-2411195, LS on UE Capabilities for FR2-NTN, RAN2

[2] R4-2420477, (NR_NTN_enh-Core)Draft CR for TS 38.101-5, Correction on FR2-NTN frequency range, CATT