# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501459

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rx requirements for NR NTN HPUE

Agenda Item:	7.10.2.2.2

Document for:	Approval

Introduction

In the RAN4#112 meeting, RAN4 reached the following consensus on the Rx requirements for NR NTN HPUE in the Way forward [1].

Issue 4-1: General considerations - Agreement - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class. Issue 4-2: RSD for NR-NTN - Agreement - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE. - Further discuss the following aspects: - Use similar way in TN spec by introducing a new table of RSD compared to the PC3 - Whether default Tx-Rx frequency separation is used for test point

In the RAN4#112bis meeting, RAN4 reached the following consensus on the Rx requirements for NR NTN HPUE in the Way forward [2].

Issue 3-1: RSD form in spec Agreement: - Use similar way in TN spec to derive RSD, if needed. Issue 3-2: RSD requirements for PC2 Agreement: - For n256: [0dB] RSD for PC2 NR-NTN for 1Tx and 2Tx - FFS on the duplexer isolation and PA interference Issue 3-4: REFSENS/RSD testing Agreement: - For PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration with PC3 NR NTN UE as starting point.

In the RAN4#113 meeting, the following way forwards were captured in the Tdoc [3].

Issue 3-1-2: RSD Requirements for PC2 on n254 Agreement: For PC2 on n254: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx Issue 3-1-3: RSD requirements for PC2 on n255 Way forward: For PC2 on n255: Further discuss the follow assumptions - Option 1: Duplex isolation as 50~52; PA interference as -107 dBm/Hz (P2/R4-2419073) - Option 2: Duplex isolation as 51; PA interference as -117 dBm/Hz (O1/R4-2419540) Issue 3-1-4: RSD requirements for PC2 on n256 Way forward: For PC2 on n256: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx as starting point - To be confirmed by further discuss the following assumptions: - Option 1: Duplex isolation as 50~52; PA interference as -117.8 dBm/Hz - Option 2: Duplex isolation as 51; PA interference as -124 dBm/Hz Issue 3-4: Others general proposals w.r.t RSD Way forward: - Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.

RAN4 has reached some consensus on assumption of duplexer isolation and PA non-linearity interference. In this paper, we’d like to share our views on how to select the suitable parameters for NR NTN HPUE.

Discussion

Referring to the current spec TS 38.101-5, both default Tx-Rx separation and flexible Tx-Rx frequency separation were specified for REFSENS verification.

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN) NTN Satellite Operating Band; TX – RX carrier centre frequency separation. n256; 190 MHz1 165 to 215 MHz2. n255; -101.5 MHz1 -72.5 to -130.5 MHz2. n254; 862 – 885 MHz. NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2.

In the WF [1] of RAN4#112 meeting, RAN4 still has an open issue whether the default Tx-Rx frequency separation is only used for HPUE RSD evaluation.

For different assumption of Tx-Rx frequency separation ( default vs variable ), different companies have different views about PA interference.

For band n256 and n255, the UL / DL frequency configuration can be defined below.

|  | default Tx-Rx frequency separation | variable Tx-Rx frequency separation |
| --- | --- | --- |
| n256 | UL: 1990~2010MHz, DL: 2180~2200MHz (170MHz Gap) | UL: 1990~2010MHz, DL: 2170~2190MHz (160MHz Gap) |
| n255 | UL 1626.5~1646.5MHz, DL 1525~1545MHz (-81.5MHz Gap) | UL 1626.5~1646.5MHz, DL 1539~1559MHz (-67.5MHz Gap) |

Observation 1: for band n256, no matter default or variable Tx-Rx frequency separation is assumed, the frequency gap between UL and DL is equal to or larger than 160MHz (8th ~ 9th adjacent channel).

If RAN4 takes the assumption of PA interference as -124 dBm/Hz for band n256 PC2 RSD evaluation for either default or variable Tx-Rx frequency separation, both 1Tx/2Rx and 2Tx/2Rx architectures don’t need to specify the RSD requirements as the evaluation in Annex A.1.

Proposal 1: for PC2 n256, the PA interference can be assumed as -124 dBm/Hz for both default or variable Tx-Rx frequency separation, so the RSD can be zero for both 1Tx/2Rx and 2Tx/2Rx RF architectures.

For band n255, the different assumptions about Tx-Rx frequency separation will lead to the different PA non-linearity performance. Based on the WF in last meeting, we have two options:

Issue 3-1-3: RSD requirements for PC2 on n255 Way forward: For PC2 on n255: Further discuss the follow assumptions - Option 1: Duplex isolation as 50~52; PA interference as -107 dBm/Hz (P2/R4-2419073) - Option 2: Duplex isolation as 51; PA interference as -117 dBm/Hz (O1/R4-2419540)

Proposal 2: before RAN4 discuss the assumption of PA interference for PC2 n255, it’s better to discuss whether only default Tx-Rx frequency separation is used for RSD evaluation. If only default Tx-Rx frequency separation is used for RSD evaluation, we can assume PA interference as -117 dBm/Hz for PC2 n255 RSD evaluation.

Summary

Observation 1: for band n256, no matter default or variable Tx-Rx frequency separation is assumed, the frequency gap between UL and DL is equal to or larger than 160MHz (8th ~ 9th adjacent channel).

Proposal 1: for PC2 n256, the PA interference can be assumed as -124 dBm/Hz for both default or variable Tx-Rx frequency separation, so the RSD can be zero for both 1Tx/2Rx and 2Tx/2Rx RF architectures.

Proposal 2: before RAN4 discuss the assumption of PA interference for PC2 n255, it’s better to discuss whether only default Tx-Rx frequency separation is used for RSD evaluation. If only default Tx-Rx frequency separation is used for RSD evaluation, we can assume PA interference as -117 dBm/Hz for PC2 n255 RSD evaluation.

- References

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung

[2] R4-2417122, WF on co-existence study and HPUE for NR-NTN, Samsung

[3] R4-2420403, WF on NR NTN HPUE, Samsung

- Annex

- A.1 The 20MHz PC2 MSD calculations for 1Tx and 2Tx in band n256

Table 1 20MHz MSD calculation for 1Tx in band n256

|  | parameters | n256 main path | n256 diversity path |
| --- | --- | --- | --- |
| transmit power for n256, dBm |  | 26 |  |
| Tx band n256 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Duplexer isolation at n256 uplink freq，dB | 52 |  |  |
| antenna isolation, dB | 10 |  |  |
| noise figure dB | 9.5 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -164.5 |  |  |
| Rx band n256 BW(MHz, NRB) | 18 |  |  |
| Thermal noise, dBm |  | -91.95 | -91.95 |
| n256 PA noise PSD at PA output port, dBm/Hz | -124 |  |  |
| n256 PA noise PSD at PA output port, dBm/RxBW |  | -51.45 |  |
| n256 Tx filter rejection at 2170~2200MHz, dB | 50 |  |  |
| n256 PA noise power at Rx n256 ant port at 2170~2200MHz, dBm |  | -101.45 | -111.45 |
| Total noise level at ANT port |  | -91.48 | -91.90 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -92.48 | -92.90 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(20MHz BW), dBm | -93.71 |  |  |
| MSD | -0.41 |  |  |

Table 1 20MHz MSD calculation for 2Tx in band n256

|  | parameters | n256 main path | n256 diversity path |
| --- | --- | --- | --- |
| transmit power for n256, dBm |  | 23 | 23 |
| Tx band n256 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Duplexer isolation at n256 uplink freq，dB | 52 |  |  |
| antenna isolation, dB | 10 |  |  |
| noise figure dB | 9.5 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -164.5 |  |  |
| Rx band n256 BW(MHz, NRB) | 18 |  |  |
| Thermal noise, dBm |  | -91.95 | -91.95 |
| n256 PA noise PSD at PA output port, dBm/Hz | -124 |  |  |
| n256 PA noise PSD at PA output port, dBm/RxBW |  | -51.45 | -51.45 |
| n256 Tx filter rejection at 2170~2200MHz, dB | 50 |  |  |
| n256 PA noise power at Rx n256 ant port at 2170~2200MHz, dBm |  | -101.45 | -101.45 |
| Total noise level at ANT port |  | -91.49 | -91.49 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -92.49 | -92.49 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(20MHz BW), dBm | -93.50 |  |  |
| MSD | -0.20 |  |  |

- A.2 The 20MHz PC2 MSD calculations for 1Tx and 2Tx in band n255

TBA after RAN4 confirm the PA interference for PC2 n255.