3GPP TSG-RAN WG4 Meeting #114                                   R4-2501478

Athens, Greece, 17th – 21st February, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on power domain enhancement for NR single carrier

Agenda Item:	7.1.1.2.1

Document for:	Approval

- Introduction

In RAN#113, the WF was agreed upon in [1], and there are still some FFS issues. In this paper, we will propose our views and discuss these issues.

- Status from RAN4#113

- Issue 1-1-1: Approaches of converting outer RB allocation to inner RB allocation

Agreement in main session:

- Extended CBW based approach

  - where the extended CBW or aggregated CBWs for the DL-only contiguous intra-band CA case is signalled from network to UE

  - Both scenario 1 and 2 should be covered

  - The network should ensure the co-existence be met when signaling the extended CBW.

The following points need further discussion:

- Issue 1-1-2: Approaches of calculating the new inner RB region

- Issue 1-1-3: Width/RB numbers of extended CBW at each side of UE CBW

- Issue 1-2-1: Where to use extended IBE in the larger BS CBW

  - It appears there was a common view to use IBE between UE CBW edge and extCBW (shifted OOBE) edge, but the details were left FFS.

- Issue 1-2-2: How to derive the extended IBE: FFS but should cover both cases for partial RB allocation and full RB allocation in the UE CBW.

- Issue 1-2-3: Boundary to apply ACLR and SEM: ACLR and SEM should be applicable from the edge of extended UE CBW. FFS whether based on UE CBW or based on extended CBW.

- Issue 1-2-4: Boundary to apply SE

- Signalling and other aspects

- Summary of our Proposed Concept

The following is proposed as the concept to be specified in Rel-19:

- Both Symmetric and Asymmetric shifted OOBE are specified.

- Both Symmetric and Asymmetric shifted OOBE apply to modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms.

- Shifted OOBE requirement specified for the following:

  - 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on both sides for Symmetrical shift

  - 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on only one side for Asymmetrical shift

Larger values than 0.5*UE CBW are not specified. If smaller values other than 0.5* UE CBW are specified, they should be limited to a few discrete points.

- Shifted OOBE defined for any existing specified UE CBW up to the maximum CBW supported by the UE.

- Where shifted OOBE corresponds to an extended CBW, extended IBE applies up until the starting frequency of the shifted OOBE, for RBs outside of NRB, a “virtual extended NRB” term applies.

- Define OOBE integrated according to UE CBW (in order to achieve Spurious Emission ITU recommendation) with Spurious Emission boundary start from end of shifted OOBE.

- Define RRC signalling from network to UE to permit UE to apply “shifted OOBE + MPR improvement”, to allow mobile operators the choice of whether to allow it or not.

- If the mobile operator has decided to allow the shifted OOBE + MPR improvement, specify rules for the UE to identify whether to apply Symmetric shifted OOBE, Asymmetric shifted OOBE, or no relaxation at all, based on the distance from its UE CBW edge to the CarrierBandwidth edge.

NOTE: We change the term from “extended CBW” to “shifted OOBE” because in our view the OOBE requirements apply based on the original UE CBW, and we select both asymmetrical and symmetrical approaches. Therefore, it seems easier to view it from the perspective of a shift of OOBE requirements.

- 4. Our detailed comments and justification for the above

Width/RB numbers of extended CBW at each side of UE CBW

We believe that defining both Symmetric and Asymmetric extensions of 0.5 * CBW on both sides or one side of the UE CBW is sufficient to cover most cases. This provides sufficient flexibility in case the UE is located at the edge of the spectrum block. Given the moderate gains of the feature, the additional complexity of defining a granular extension vs the complexity both in terms of specification, device testing and signalling does not seem very well justified. Furthermore, there appears to be NO benefit to MPR performance of extending beyond 0.5 * UE CBW on either side.

Regarding whether to apply this for just the maximum CBW or smaller CBWs, we are open minded. There may be cases where the relaxed emission may apply in guardband, where the UE cannot be configured with a wider bandwidth, so it may make sense here to allow for smaller CBWs.

Proposal 1: Both Symmetric and Asymmetric shifted OOBE are specified.

Proposal 2: Shifted OOBE requirement is specified for:

  - 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on both sides for Symmetrical shift

  - 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on only one side for Asymmetrical shift

Proposal 3: Larger values than 0.5*UE CBW are not specified. If smaller values other than 0.5* UE CBW are specified, they should be limited to a few discrete points.

Proposal 4: Shifted OOBE can be defined for any existing UE CBW up to the maximum CBW supported by UE.

Whether ACLR/SEM are integrated over UE CBW or extended CBW

Taking into account the ITU-R recommendation of Spurious Emission starting from 250% from the centre of the necessary bandwidth (CBW), it appears that this can only be fulfilled naturally if the ACLR/SEM ends before this point. With a 0.5 * UE CBW extension to the edge of the UE CBW, this would require the ACLR/SEM to be integrated over the UE CBW, and not over the extended CBW.

Proposal 5: Define OOBE integrated according to UE CBW (in order to achieve Spurious Emission ITU recommendation).

Boundary to apply Spurious Emission

Based on the above, the Spurious Emission should start where SEM/ACLR finishes, and such a way forward can comply with ITU-R recommendations (ITU REC SM.329/328).

Proposal 6: Spurious Emission boundary starts from end of shifted OOBE.

InBand Emissions (IBE) between UE CBW edge and extended edge

In terms of how to define the extended IBE requirement, we believe that applying an extension of the NRB term (NRB_EXT) would be sufficient. The following that the following approach would work:

- For Symmetrical extension (0.5 * UE CBW on each side), NRB_EXT = NRB of CBW corresponding to UE CBW*2 (for same SCS)

- For asymmetrical extension (0.5 * UE CBW on one side), NRB_EXT = NRB of CBW corresponding to UE CBW*1.5 (for same SCS)

- In case no defined CBW corresponds to UE CBW*2, NRB_EXT = floor(UE CBW*2 / (SCS * 12)) RBs.

- In case no defined CBW corresponds to UE CBW*1.5, NRB_EXT = floor(UE CBW*1.5 / (SCS * 12)) RBs.

Proposal 7: Where shifted OOBE corresponds to an extended CBW, extended IBE applies up until the starting frequency of the shifted OOBE, for RBs outside of NRB, a “virtual extended NRB” term applies.

Proposal 8: Use the approach proposed in this document to derive NRB_EXT.

Why to specify the extension as “shifted OOBE” rather than “extended CBW”

When defining the SEM/ACLR requirements as integrated according to UE CBW, but where the starting point is shifted from the channel edge, this is not the same as the approach taken for scaling SEM/ACLR integration over a wider CBW.

It cannot be assumed that the existing MPR rules can be applied for any combination of UE CBW and wider overlaid CBW in the same way as for a partial RB allocation operating in such a wider CBW. This would need all of the cases (including all UE CBW locations within each wider CBW) to be simulated for every combination of CBW and wider CBW, which appears to be a lot of effort for very limited gain.

Proposal 9: Define the term in the specifications as “shifted OOBE” rather than “extended CBW”

What we have done is to simulate the Symmetrical and Asymmetrical shifted OOBE assuming 0.5 * CBW shift applying the SEM/ACLR according to UE CBW. The 20MHz results below are for 15kHz SCS which is a worst case than 30kHz SCS in terms of NRB and spectrum utilization. Therefore, the equivalent ratios can apply for 30kHz SCS too.

Table 1a: Evaluation results for Symmetric shifted OOBE (20MHz CBW with 0.5* CBW shift both sides

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {0, 100}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {0, 106}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {0, 100}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {0, 106}, {0, 50}, {0, 25}, {0, 1} | ALL PASS | ALL PASS |

Table 1b: Evaluation results for Asymmetrical shifted OOBE (20MHz CBW with 0.5 * CBW shift right side)

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, QPSK, 21.5dBm | {35, 70}, {105, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |
| CP-OFDM, 16QAM, 21dBm | {35, 70}, {105, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, QPSK, 23dBm | {91, 182}, {272, 1} | ALL PASS | ALL PASS |
| DFT-s-OFDM, 16QAM, 22dBm | {50, 25}, {105, 1} | ALL PASS | ALL PASS |

Table 1c: Evaluation results for Asymmetrical shifted OOBE (100MHz CBW with 0.5 * CBW shift right side)

| Waveform, Mod, Tx power | RB allocation (RBstart, LCRB) | ACLR Pass/fail | SEM Pass/fail |
| --- | --- | --- | --- |
| DFT-s-OFDM, QPSK, 23dBm | {91, 182}, {272, 1} | ALL PASS | ALL PASS |

Proposal 10: Both Symmetrical and Asymmetrical shifted OOBE apply to modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms.

For Symmetrical case, it can be observed that the maximum inner RB allocation is LCRB = NRB.

For the Asymmetrical case, it can be observed that the maximum inner RB allocation is LCRB = NRB*2/3, as also shown in the figure 1 below.

Figure 1: Asymmetric shifted OOBE

Proposal 11: Agree that the maximum inner RB allocation is LCRB = NRB with symmetrical shift of 0.5 * UE CBW on both sides, and that maximum inner RB allocation is LCRB = ceil(2*NRB/3) with asymmetrical shift of 0.5 * UE CBW on one side.

The following shows example modifications to the MPR formula to cover both Symmetrical and asymmetrical OOBE shifts, applying only for shifts of 0.5 * UE CBW.

RBStart,Low = min(max(1, floor(LCRB/2)), OOBExt,Low),

where OOBExt,Low:

= 0, if OOBE shift from lower UE CBW edge applied,

= ∞ otherwise

RBStart,High = NRB – min(max(1, floor(LCRB/2)), OOBExt,High) – LCRB,

where OOBExt,High:

= 0, if OOBE shift from upper UE CBW edge applied,

= ∞ otherwise

The RB allocation is an inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High

LCRB ≤ ceil(NRB/2 * LCRB_ Ext) , where LCRB_ Ext:

= 2, if OOBE shifted by 0.5 * UE CBW on both upper and lower edge

= 4/3, if OOBE shifted by 0.5 * UE CBW on only one of either upper or lower edge

= 1 otherwise

Proposal 12: Consider the example spec changes in this section as a starting point for spec drafting for modification to the MPR section of TS 38.101-1.

Network-UE signalling aspects

There are some different potential motivations for network to UE signalling, and in past meetings they have been somewhat bundled together in discussions. We believe it would be best to separate them.

Potential Motivations (1,2,3) are:

- To indicate to the UE that the MNO permits the application of relaxed OOBE in its spectrum block. This may be needed due to any geographically specific regulatory conditions not allowing the relaxed OOBE, or it may just be because the operator has some concern about the UE applying IBE requirement levels instead of OOBE outside of its CBW but inside the operator’s spectrum block.

Our view: This makes sense and de-risks deployment of the feature.

- To avoid the UE from having to check whether it is in a frequency location within the spectrum block that would be acceptable in terms of relaxing emissions. It is true that if the network provided this information to the UE then it would make the life simpler for the UE.

Our view: Due to concerns expressed by network infra vendors, we can instead accept not to rely on this, and instead require the UE to identify by itself whether its frequency location would allow it to apply relaxed OOBE. We would also be willing to define such rules in the RAN4 specifications. Existing parameters in SIB1 and/or dedicated RRC signalling can be used to allow the UE to identify this (carrierBandwidth, offsetToPointA, offsetToCarrier, and UE BWP size and location info).

- To indicate exactly what is the level of extension permitted by the operator. This would assume that there is some network configurability of this relaxation.

Our view: This would seem to complicate the feature, complicate the network and UE implementation, and require more testing, so recommend constraining any configuration flexibility.

Proposal 13: Define RRC signalling from network to UE to permit UE to apply “shifted OOBE + MPR improvement”, in order to allow mobile operators the choice of whether to allow it or not.

Potential rules to be specified instead of using network signalling for “Motivation 2” above

- UE shall obtain the SIB1 and/or dedicated RRC signalling information to identify the distance from UE CBW edge to CarrierBandwidth edge

- Shifted OOBE starts at 0.5 * UE CBW from UE CBW edge

- M = number of RBs between UE CBW lower edge and start of lowest RB of CarrierBandwidth

- N = number of RBs between UE CBW upper edge and end of highest RB of CarrierBandwidth

- NRB = maximum number of RBs of UE CBW

- If both M and N ≥ ceil(NRB/2), shifted OOB applies from both lower and upper UE CBW edges

- If M < ceil(NRB/2) and N ≥ ceil(NRB/2), shifted OOB applies only from UE CBW upper edge

- If M ≥ ceil(NRB/2) and N < ceil(NRB/2) : shifted OOB applies only from UE CBW lower edge

- Otherwise: shifted OOB not applied and legacy requirements approach applies instead

Proposal 14: If the mobile operator has decided to allow the shifted OOBE + MPR improvement, specify rules for the UE to identify whether to apply Symmetric shifted OOBE, Asymmetric shifted OOBE, or no relaxation at all, based on the distance from its UE CBW edge to the CarrierBandwidth edge.

- 5. Conclusion & Proposals

Proposal 1: Both Symmetric and Asymmetric shifted OOBE are specified.

Proposal 2: Shifted OOBE requirement is specified for:

  - 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on both sides for Symmetrical shift

  - 0.5 * UE CBW shift of OOBE starting point from UE CBW edge on only one side for Asymmetrical shift

Proposal 3: Larger values than 0.5*UE CBW are not specified. If smaller values other than 0.5* UE CBW are specified, they should be limited to a few discrete points.

Proposal 4: Shifted OOBE can be defined for any existing UE CBW up to the maximum CBW supported by UE.

Proposal 5: Define OOBE integrated according to UE CBW (in order to achieve Spurious Emission ITU recommendation).

Proposal 6: Spurious Emission boundary starts from end of shifted OOBE.

Proposal 7: Where shifted OOBE corresponds to an extended CBW, extended IBE applies up until the starting frequency of the shifted OOBE, for RBs outside of NRB, a “virtual extended NRB” term applies.

Proposal 8: Use the approach proposed in this document to derive NRB_EXT.

Proposal 9: Define the term in the specifications as “shifted OOBE” rather than “extended CBW”

Proposal 10: Both Symmetrical and Asymmetrical shifted OOBE apply to modulations of QPSK and 16QAM and for both DFT-S-OFDM and CP-OFDM waveforms.

Proposal 11: Agree that the maximum inner RB allocation is LCRB = NRB with symmetrical shift of 0.5 * UE CBW on both sides, and that the maximum inner RB allocation is LCRB = ceil(2*NRB/3) with asymmetrical shift of 0.5 * UE CBW on one side.

Proposal 12: Consider the example spec changes in this section as a starting point for spec drafting for modification to the MPR section of TS 38.101-1.

Proposal 13: Define RRC signalling from network to UE to permit UE to apply “shifted OOBE + MPR improvement”, in order to allow mobile operators the choice of whether to allow it or not.

Proposal 14: If the mobile operator has decided to allow the shifted OOBE + MPR improvement, specify rules for the UE to identify whether to apply Symmetric shifted OOBE, Asymmetric shifted OOBE, or no relaxation at all, based on the distance from its UE CBW edge to the CarrierBandwidth edge.

- 6. References

- R4-2420527, “WF on power domain enhancements”, Huawei