3GPP TSG-RAN WG4 Meeting #113	R4-2501481

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	The impacts of T310 suspension on RLM requirements in NB-IoT over NTN

Agenda item:	5.28.2

Document for:	Discussion

Introduction

This document discusses maintenance on Clause 7.23A.2.2 about Radio Link Monitoring requirements for NB-IoT operation in NTN.

The maintenance is needed to make the TS 36.133 in line with the UE operation, as described in TS 36.331 and TS 36.300. And the reason behind the modification is that most of the times, UEs are not required to suspend a timer related operation when a timer is running.

Discussion

The TS 36.300 defines the UE suspends AS operations including RLM timers such as T310, when performing GNSS acquisition. The UE will resume the timers, when the GNSS acquisition is completed. This can be found in Clause 23.21.2:

TS 36.300 Clause 23.21.2.2 […] In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation. The UE can be triggered to perform, or configured to autonomously perform, GNSS acquisition. In connected mode, upon outdated ephemeris and common Timing Advance, the UE shall acquire the broadcasted parameters. Upon failed GNSS acquisition, the UE shall move to idle mode if the GNSS position is outdated and uplink transmission extension is not active. Upon outdated GNSS position the UE shall move to idle mode, unless GNSS acquisition was triggered or uplink transmission extension is active. Upon completing the GNSS acquisition, the UE shall trigger remaining validity duration reporting (see TS 36.321 [13]). NOTE: The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is performing GNSS acquisition and resumed when the GNSS acquisition is completed. The UEs may be configured to report Timing Advance at initial access or in connected mode. In connected mode, event-triggered reporting of the Timing Advance is supported.

The current TS 36.133 defines the NB-IoT NTN UE shall “monitor the link for recovery” upon start of T310 and until the expiry or stop of T310. Thus, the current 36.133 does not account for the potential suspension of T310, during which suspension the UE shall not monitor the link for recovery (because it is not possible due to GNSS and 3GPP modems not being able to operate concurrently).

TS 36.133 current phrasing does not account for potential suspension of T310

The TS 36.133 section 7.23A.2.2 is updated to reflect the timer T310 is suspended during GNSS acquisition and that the corresponding monitoring of the link for recovery is not required during the GNSS acquisition. A similar change shall be made to section 7.19A.2.2 (UE Cat M1 for satellite access)

Conclusion

The paper has analyzed the need for maintenance of RLM requirements to account for the suspension of RLM timers, the following observations were made:

Observation 1: TS 36.133 current phrasing does not account for potential suspension of T310

Proposal 1: The TS 36.133 section 7.23A.2.2 is updated to reflect the timer T310 is suspended during GNSS acquisition and that the corresponding monitoring of the link for recovery is not required during the GNSS acquisition. A similar change shall be made to section 7.19A.2.2 (UE Cat M1 for satellite access)

- References

- 3GPP, TS 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”, Release