3GPP TSG-RAN WG4 Meeting #114	R4-2501513

# Athens, Greece, 17th – 21st Feb, 2025

Title: 	Discussion on 8Rx performance requirements with MMSE-IRC

Source: 	Huawei, HiSilicon

Agenda item:	7.18.2

Document for:	Discussion

Background

According to WF R4-2419784 [1], some open issues on 8Rx performance requirements with MMSE-IRC were left. This contribution provides our views.

- Discussions

General

Definition of MMSE-IRC receiver capability for 8Rx UE

One open issue is whether to refine the wording of 8Rx receiver, which was defined in R18:

Definition of MMSE-IRC receiver capability for 8Rx UE - Proposals - Option 1: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE supporting SU-MIMO 8Rx receiver. - Option 2: Update the existing Rel-18 UE capability definition as a maintenance part of the R18 FR1 RF enhancement WI. - Option 3: Introduce new Rel-19 UE capabilities for MU-MIMO 8Rx receiver. And clearly state that the UE supporting ‘MU-MIMO 8Rx receiver’ are also required to support ‘SU-MIMO 8Rx receiver’ capability. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

MU-MIMO scenario is missing in R18 receiver capability discussion. However, it has been agreed that R19 MU-MIMO test coverage should cover both receiver types, so it is reasonable to refine the simplified and baseline receiver definitions to cover both SU-MIMO and MU-MIMO scenario. Option4 can be considered as baseline.

Proposal 1: Update simplified and baseline receiver definition in TS 38.308 and TS 38.101-4 as following:

Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

8Rx UE MMSE-IRC receiver coverage

How to define the requirements for baseline receiver and simplified receiver are FFS:

- Proposals - Option 1: RAN4 to define two sets of requirements (baseline 8Rx receiver and simplified 8Rx receiver). - Option 2: Set the criteria for defining two sets of requirements as: If the span after removing a reasonable number or outliers (at least 5 results should be left) is >2.5dB, define two sets of requirements. The evaluation of the span shall be done for each configuration agreed for requirement definition. - Option 3: Postpone defining criteria for decision of one or two sets of requirements until all test cases are finalized.

It is common understanding that number of sets of requirements depends on performance difference between such two receivers. According to our results shown in Appendix, performance difference under inter cell interference is about 2.3dB for rank1, 3.3dB~3.7dB for Rank2 and 6.6dB for Rank4, performance difference under MU-MIMO scenario is about 2.0dB~3.4dB. Unless the gap is negligible such as <0.5dB, two set of requirements shall be defined for two receivers. Adding additional margin to cover different UE implementations will relax the requirements for baseline receiver and make the test unfair.

Observation 1: The gap between baseline receiver and simplified receiver is significant for all cases.

Proposal 2: RAN4 to define two sets of requirements for baseline 8Rx receiver and simplified 8Rx receiver.

PDSCH requirements with inter cell interference

Scenarios

In last meeting, following agreements were made:

Number of interference cells - Background: - We have an existing agreement from RAN#112bis, to reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx, with TBD for applicability rules to reduce UE efforts [R4-2416555] - Proposals: - Option 1: Both 1 interference cell and 2 interference cells

We don’t have agreements on scenarios in previous meeting, the agreement on reuse the same interference profiles doesn’t mean scenarios should also be reused. Firstly, we should discuss the scenario(s), if the scenario is agreed, the corresponding interference profiles defined in Rel-17 for 2/4Rx can be reused. Based on our understanding, HomNet scenario is worse case with higher interference power level compared to HetNet scenario. So we don’t think that if a UE which has passed the severe test still needs to pass the optimistic test since UE baseband behaviour will not be changed.

Proposal 3: RAN4 to only consider HomNet scenario.

One key issue is Rank and MCS, in last meeting, we had following candidates for evaluation:

Rank and MCS Agreement: - Use the following for evaluation. TBD on requirements: - Rank1: MCS17 - Rank2: MCS17 - Rank4: MCS13 - 2nd priority simulations can be removed for next meeting for both baseline and simplified receiver.

Rank

It is mentioned that R17 2Rx requirements for inter-cell interference with HomNet scenario is 15.4dB, which could be a reference when judging the reasonability of 8Rx test SNR points.

Table 5.2.2.1.15-3: Minimum performance for PDSCH with rank 1 and with inter-cell interference

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | 15.4 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 12.5 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |

To make target SNR points reasonable, with lower operating SNR points for 8Rx, higher rank should be selected. Based on our simulation results, given rank4 with MCS13, target SNR is 8.6dB for baseline receiver and 14.2dB for simplified receiver, which is suitable for cell edge scenarios. Some companies question about the feasibility of rank4+MCS13, it is mentioned that such configurations have been used for R15 PDSCH requirements definition (Table 5.2.3.1.1-6 in 38.101-4). Hence, we propose to choose Rank4+MCS13.  Furthermore, interference estimation processing with Rank 4 is more complicate than rank2. For Rank4, number of DMRS CDM groups without data is 2, i.e. two CDM groups are allocated with DMRS, which means two CDM groups can be used for interference estimation. However, for rank1/2, the number of CDM groups is 1, which means only one CDM group can be used for interference estimation. Therefore, introducing rank4 can comprehensively verify the interference estimation algorithm by utilizing all potential CDM groups.

Observation 2: Rank4+MCS13 also be used in R15 PDSCH requirements definition.

Observation 3: Legacy PDSCH tests with ICI only have one number of DMRS CDM group without data configured, which cause the limitation that only one CDM group can be used for interference estimation. However, Rank4 allows UE to estimate interference utilizing two CDM groups.

Proposal 4: RAN4 to choose Rank4 with MCS13.

CQI requirements with inter cell interference

Rank

One key issue is rank for serving cell. Rank1 and 2 are candidate options:

Rank for serving cell Agreement: - For initial study use: - Rank 1 - Interested companies can also provide results with Rank 2.

Rank1 is too optimistic for 8Rx receiver, even with inter-cell interference, we don’t think it’s possible to schedule 1 layer for a 8Rx UE. Hence we prefer to choose Rank2.

Proposal 5: RAN4 to choose Rank2 for CQI definition.

CQI calculation algorithm

Same as demodulation processing, there could be two different CQI calculation assumptions:

- Baseline CQI calculation: UE perform CQI calculation with 8Rx joint processing

- Simplified CQI calculation: UE perform CQI calculation with separate 4Rx processing

Based on our understanding, CQI calculation algorithm should be consistent with demodulation algorithm, otherwise, performance degradation may occur. For example, if demodulation is based on 8Rx joint processing but CSI calculation is based on two separate 4Rx joint processing, CQI derived by CSI-RS estimation may be underestimated and absolute throughput may be lower than it should be. Therefore, CQI calculation algorithm is expected to match to demodulation algorithm. We did some simulations for both two receiver types: Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing. Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing. It’s observed that in most SNR points γfor baseline receiver is higher than simplified receiver. The reason is that baseline receiver has higher interference suppression capability, given the same total power of interference and white noise, baseline receiver can cancel more interference. Therefore, we propose to define CSI requirements for such two receiver types.

Figure 2-2: CQI test results for baseline receiver and simplified receiver.

Proposal 6: RAN4 to define CQI requirements for two receiver types:

Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing.

Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing.

PDSCH requirements under MU-MIMO scenario

Rank and MCS

Following candidate options for evaluation were made in last meeting:

Agreement: - Define requirements for: - Rank 2+2 - FFS: MCS17/19 - TBD: Rank 4+4: MCS13

For Rank4+4, 70% of max throughput is unachievable for simplified receiver based on our simulation results in clause 4, which can’t accommodate the agreements that test coverage should cover simplified receiver.

Proposal 7: RAN4 to only consider Rank 2+2 and preclude Rank 4+4.

We did simulations for both MCS17 and MCS19, performance gain between MCS17 and 19 is smaller than 2dB, both MCS can verify the MMSE-IRC receiver. However, according to Table A.3.2.1.1-3(PDSCH FRC 64QAM), MCS19 is the most common MCS index in 64QAM and existing FRC R.PDSCH.1-2.1 could be reused directly. If MCS17 is selected, RAN4 have to create a new FRC. Hence, it’s better to choose MCS19.

Observation 4: FRC R.PDSCH.1-2.1 could be reused directly for 8Rx MCS19 test. RAN4 have to create a new FRC if MCS17 is used.

Proposal 8: RAN4 to choose MCS19 for PDSCH requirements under MU-MIMO scenario.

Applicability rule and release independent

Test applicability

We have following options for test applicability rules:

Test applicability for 8Rx UEs Agreement: - New requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs. - TBD: additional test applicability rule. Test applicability to skip 2/4Rx requirements - Proposals - Option 1: Defining applicability rules to skip 2/4Rx requirements if 8Rx tests are passed. - Option 1a: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 1b: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Table 5.1.1.3-1 (Requirements applicability for optional UE features) should be updated to capture the applicability rules that which requirement is applicable for which 8 Rx receiver. The other aspect is how to handle 2Rx/4Rx tests if 8Rx UE has passed 8Rx tests. We prefer that if 8Rx UE passes 8Rx test, 2Rx/4Rx test can be skipped regardless the test configurations, since Ruu calculation is main step of MMSE-IRC receiver and 8x8 Ruu calculation is much complicate than 4x4 or 2x2, UE doesn’t need to pass test cases corresponding to simper implementation if has passed cases corresponding to complicate implementation.

Proposal 9: The new applicability rules are needed to specify that which requirement is applicable for which 8 Rx receiver, meanwhile,

Proposal 10: Introduce the applicability rules that if UE passes 8Rx tests, all 2Rx/4Rx tests shall be skipped, no matter same test configurations as 2Rx/4Rx are defined or not.

Release independency

We have following options for release independency:

Release independent - Proposals - Option 1: Discuss release independence requirements at a later stage in the WI. - Option 1a: RAN4 start discussing the issue of release independent when RAN4 agree on baseline assumptions, test parameters and test metrics, i.e., after RAN4#113. - Option 2: The new requirements for 8Rx should be release independent from Rel-18. - Option 2a: The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced. - Option 3: The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

Noted that supporting 8Rx is introduced in Rel-18 but release independent from Rel-17. Considering there is short period between Rel-18 and Rel-19 and MMSE-IRC receiver is basic feature which is always be assumed as baseline receiver for demodulation requirements. It’s reasonable to define the new requirements release independent from Rel-18. To be conservative, the requirements could be optional for Rel-17.

Proposal 11:  The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

- Conclusion

- In this paper, we provide our views on 8Rx requirements with interference. The proposals are:

Proposal 1: Update simplified and baseline receiver definition in TS 38.308 and TS 38.101-4 as following:

Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Observation 1: The gap between baseline receiver and simplified receiver is significant for all cases.

Proposal 2: RAN4 to define two sets of requirements for baseline 8Rx receiver and simplified 8Rx receiver.

Proposal 3: RAN4 to only consider HomNet scenario.

Observation 2: Rank4+MCS13 also be used in R15 PDSCH requirements definition.

Observation 3: Legacy PDSCH tests with ICI only have one number of DMRS CDM group without data configured, which cause the limitation that only one CDM group can be used for interference estimation. However, Rank4 allows UE to estimate interference utilizing two CDM groups.

Proposal 4: RAN4 to choose Rank4 with MCS13.

Proposal 5: RAN4 to choose Rank2 for CQI definition.

Proposal 6: RAN4 to define CQI requirements for two receiver types:

Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing.

Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing.

Proposal 7: RAN4 to only consider Rank 2+2 and preclude Rank 4+4.

Observation 4: FRC R.PDSCH.1-2.1 could be reused directly for 8Rx MCS19 test. RAN4 have to create a new FRC if MCS17 is used.

Proposal 8: RAN4 to choose MCS19 for PDSCH requirements under MU-MIMO scenario.

Proposal 9: The new applicability rules are needed to specify that which requirement is applicable for which 8 Rx receiver, meanwhile,

Proposal 10: Introduce the applicability rules that if UE passes 8Rx tests, all 2Rx/4Rx tests shall be skipped, no matter same test configurations as 2Rx/4Rx are defined or not.

Proposal 11:  The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

- Appendix

Table 4-1 and Figure 4-1 provides simulation results for 8Rx with inter-cell interference.

- Table 4-1: Summary of simulation results for 8Rx with inter-cell interference (HomoNet)

| Propagation conditions | Rank | MCS |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance difference (Baseline receiver vs Simplified receiver) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| TDLC300-100 | Rank1 | MCS17 |  | 5.5 | 9.8 | 4.3 | 7.8 | 11.3 | 3.5 | 2.3 |
| TDLA30-10 | Rank2 |  | MCS17 | 7.3 | 12.1 | 4.8 | 10.8 | 14.2 | 3.4 | 3.5 |
|  | Rank4 |  | MCS13 | 8.6 | 13.7 | 5.1 | 15.2 | 17.2 | 2.0 | 6.6 |

- Table 4-2: Summary of simulation results for 8Rx with inter-cell interference (HetNet)

| Propagation conditions | Rank | MCS |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance difference (Baseline receiver vs Simplified receiver) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| TDLA30-10 | Rank1 | MCS17 |  | 3.6 | 7.1 | 3.5 | 5.3 | 8.5 | 3.2 | 1.7 |
|  | Rank2 |  | MCS17 | 6.4 | 10.7 | 4.3 | 8.9 | 12.8 | 3.9 | 2.5 |
|  | Rank4 |  | MCS13 | 7.8 | 11.9 | 4.1 | 13.4 | 15.9 | 2.5 | 5.6 |

Table 4-3 and Figure 4-1 provides simulation results for 8Rx with MU-MIMO.

- Table 4-3: Summary of simulation results for 8Rx with MU-MIMO

|  |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance loss |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| Rank2+2 | MCS17 | 9.3 | Inf | Inf | 13.3 | Inf | Inf | 4.0 |
|  | MCS19 | 10.8 | Inf | Inf | 15.2 | Inf | Inf | 4.4 |
| Rank4+4 | MCS13 | 12.4 | Inf | Inf | Inf | Inf | Inf | Inf |

Figure 4-1: Simulation results for PDSCH requirements with MU-MIMO

- Reference

[1]  R4-2419784 WF on NR demodulation performance: Phase 5. Nokia