3GPP TSG-RAN WG4#114	R4-2501578

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.5.2

Source: 	Qualcomm France

Title: 	Intra-band Non-collocated NR CA/EN-DC requirements

Document for:	Approval

# Introduction

Considerations for Non-collocated NR CA/EN-DC UE are provided in this contribution.

- Background

WF was agreed in RAN4#113 [1]:

# Topic #1: Rel-18 Type 2

  - Sub-topic 1-1 : UE RF requirements for Type 2 EN-DC/NR-CA

### <Issue 1-1:  UE Capability Type definition for EN-DC and NR-CA>

Way forward:

- Continue further discussion on following options and conclude it in the next meeting, so that RAN4 can begin Rel-19 38.101-1/101-3 CR work in April meeting.

- Proponents of Option 2 need to propose specific examples in Feb. meeting.

- Option 1:

Add the table into TS 38.101-3: 7.10B	Power imbalance for EN-DC and 38.101-1: 7.10A	Power imbalance for NR-CA.

- Option 2:

Not adding the table in 38.101-1/3 and refer to 38.331 and its sections.

# Topic #2: Rel-19 Type 4

  - Sub-topic 2-1 : UE RF requirements for Type 4 EN-DC/NR-CA

### <Issue 2-1-1:  How to capture the power imbalance and REFSENS requirements for Big CR>

Way forward:

- In the previous meeting RAN4 agreed no rush to make decision on Rel-19 CR. Discuss it as Rel-19 big CR.

- The CR leader take companies’ inputs of R4#112-bis meeting into account when drafting actually.

### <Issue 2-1-2:  Whether to introduce term “Type 2” for pre Rel-19 specifications>

Way forward:

- Wait for the conclusion of Issue 1-1: UE Capability Type definition for EN-DC and NR-CA.

  - Sub-topic 2-2 : UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA

### <Issue 2-2-1:  New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)>

Way forward:

- WF on this Issue 2-2-1 is consolidated into one WF of Issue 2-2-2.

### <Issue 2-2-2:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated)>

Way forward:

- For NR-CA, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

- For EN-DC, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

### <Issue 2-2-3:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b>

Way forward:

  - When maxMIMO-layers = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

NOTE: This is specified Rel-18 behavior. No need to further discuss.

  - When maxMIMO-layers ＞4

- FFS in the next meeting.

  - When maxMIMO-layers = 4

- FFS in the next meeting.

NOTE: maxMIMO-layers means the existing BS signaling.

  - Sub-topic 2-3 : Other aspects (incl. clarification of contiguous LTE CCs)

### <Issue 2-3-1:  UE Capability Type definition for EN-DC and NR-CA>

Way forward:

- Continue further discussion in Issue 1-1.

### <Issue 2-3-2:  Reference UE architecture>

Agreement:

- No need to discuss the table in the future RAN4 meetings.

The table in WID is not reference point for RAN4.

- Discussion

### <Issue 1-1:  UE Capability Type definition for EN-DC and NR-CA>

### <Issue 2-1-2:  Whether to introduce term “Type 2” for pre Rel-19 specifications>

<Issue 2-3-1:  UE Capability Type definition for EN-DC and NR-CA>

We have co-signed CR’s to make the current wordings more readable without adding the controversial tables into NR CA/EN-DC specifications for different types.

Proposal 1: Agree CR’s improving the current wording for Intra-band Non-collocated CA/EN-DC requirements, without adding new tables.

### <Issue 2-2-2:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated)>

Way forward:

- For NR-CA, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

- For EN-DC, one new Rel-19 BS signaling is needed from Type 4 to Type 1 4L or Type 1 larger than 4L per CC.

  - Type 4 to Type 1 4L or Type 1 larger than 4L per CC relies on BS reconfiguration.

To clarify this matter, UE indicates maxMIMO-layers to NW as usual. UE indicating support of 6L/8L per CC is optional. If UE indicates >4L then switching between Type 1 < - > Type 4 could happen accordingly in NR CA. The group should discuss if this can be done using one BS signaling, or if two BS signalings are needed; one to switch between Type 1 < - > Type 4 with Max 4L per CC in Type 1, and ; one to switch between Type 1 < - > Type 4 with >4L per CC in Type 1.

For EN-DC, there is no need to have >4L even for type 1.

Additionally, RAN4 needs to think how to formulate the agreements into understandable format so that everyone understand them the same way. Initial connection and switching between Types go hand-in-hand.

Proposal 2: For NR-CA, new Rel-19 BS signaling to switch between Type 4 and Type 1 accounts the UE indicated maxMIMO-layers capability. RAN4 to consider if these can be done using single BS signaling, or two separate BS signaling

-If UE does not indicate maxMIMO-layers >4 then switching between Type 4 and Type 1 can be done accordingly, i.e. between Type 4 and Type 1 4L

-If UE indicates optional 6L/8L support per CC then switching between Type 4 and Type 1 can be done accordingly, i.e. between Type 4 and Type 1 6L/8L

Proposal 3: For EN-DC, new Rel-19 BS signaling to switch between Type 4 and Type 1 accounts the UE indicated maxMIMO-layers capability.

-Switching between Type 4 and Type 1 can be done accordingly, i.e. between Type 4 and Type 1 4L

### <Issue 2-2-3:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b>

Way forward:

  - When maxMIMO-layers = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

NOTE: This is specified Rel-18 behavior. No need to further discuss.

  - When maxMIMO-layers ＞4

- FFS in the next meeting.

  - When maxMIMO-layers = 4

- FFS in the next meeting.

NOTE: maxMIMO-layers means the existing BS signaling.

Proposal 4: UE behavior during initial connection

  - When maxMIMO-layers = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

NOTE: This is specified Rel-18 behavior. No need to further discuss.

  - When maxMIMO-layers ＞4

- UE is Type 1

  - When maxMIMO-layers = 4

- ALT1:

  - UE is Type 1 or Type 4 depending on present Rel-19 BS signaling element value

  - When Rel-19 BS signaling is absent UE follows Rel-18 BS signaling, i.e. is Type 1

- ALT2:

  - Rel-19 BS signaling is present UE is Type 4

  - When Rel-19 BS signaling is absent UE follows Rel-18 BS signaling, i.e. is Type 1

In Rel-18, the approach was somewhat different, but there also the difference is that element can be present only when maxMIMO-layers is up to two, and with that depending on it being present/absent UE is collocated/non-collocated with 2L. When maxMIMO-layers is more than two, UE is collocated.

In R19, assuming the field can be present only when maxMIMO-layers is up to four, we kind of lose the possibility to ignore the field by changing maxMIMO-layers configuration for 4 as UE can be collocated/non-collocated with maxMIMO-layers =4, unlike in Rel-18 UE was non-collocated only with maxMIMO-layers =2, and collocated only with maxMIMO-layers =4. In other words, having R19 BS field absent for non-collocated would mean that for primary use case for 4Rx bands, one would need to always have field present for collocated with 4L.

ALT2 would change the logic how Type 2 UE is controlled, as it would basically be along these lines below.

nonCollocatedTypeNR-CA-R19

This field is only present for a UE configured with maxMIMO-Layers with value more than 2 but less than or equal to 4 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present absent, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is present absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

NOTE: maxMIMO-layers means the existing BS signaling.

Additionally, RAN4 needs to think how to formulate the agreements into understandable format so that everyone understand them the same way. Initial connection and switching between Types go hand-in-hand.

Proposal 5: “UE behavior during initial connection” basically means how the BS signaling field contents are written and in which conditions UE is non-collocated/collocated. “BS signaling to switch between Type 1 and Type 4” means change in the state/presence of BS signaling field

Proposal 6: All agreements must be collected and formulated in such a way that everyone understand them the same way. For instance, UE behavior in initial connectivity and switching from the respective state go hand-in-hand.

- Conclusion

Considerations for Non-collocated NR CA/EN-DC UE were provided with the following proposals.

Proposal 1: Agree CR’s improving the current wording for Intra-band Non-collocated CA/EN-DC requirements, without adding new tables.

Proposal 2: For NR-CA, new Rel-19 BS signaling to switch between Type 4 and Type 1 accounts the UE indicated maxMIMO-layers capability. RAN4 to consider if these can be done using single BS signaling, or two separate BS signaling

-If UE does not indicate maxMIMO-layers >4 then switching between Type 4 and Type 1 can be done accordingly, i.e. between Type 4 and Type 1 4L

-If UE indicates optional 6L/8L support per CC then switching between Type 4 and Type 1 can be done accordingly, i.e. between Type 4 and Type 1 6L/8L

Proposal 3: For EN-DC, new Rel-19 BS signaling to switch between Type 4 and Type 1 accounts the UE indicated maxMIMO-layers capability.

-Switching between Type 4 and Type 1 can be done accordingly, i.e. between Type 4 and Type 1 4L

Proposal 4: UE behavior during initial connection

  - When maxMIMO-layers = 2

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1.

- If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2.

NOTE: This is specified Rel-18 behavior. No need to further discuss.

  - When maxMIMO-layers ＞4

- UE is Type 1

  - When maxMIMO-layers = 4

- ALT1:

  - UE is Type 1 or Type 4 depending on present Rel-19 BS signaling element value

  - When Rel-19 BS signaling is absent UE follows Rel-18 BS signaling, i.e. is Type 1

- ALT2:

  - Rel-19 BS signaling is present UE is Type 4

  - When Rel-19 BS signaling is absent UE follows Rel-18 BS signaling, i.e. is Type 1

NOTE: maxMIMO-layers means the existing BS signaling.

Proposal 5: “UE behavior during initial connection” basically means how the BS signaling field contents are written and in which conditions UE is non-collocated/collocated. “BS signaling to switch between Type 1 and Type 4” means change in the state/presence of BS signaling field

Proposal 6: All agreements must be collected and formulated in such a way that everyone understand them the same way. For instance, UE behavior in initial connectivity and switching from the respective state go hand-in-hand.

- Reference

[1] R4-2420399, “WF on non-collocated intra-band EN-DC and NR-CA”, KDDI