# 3GPP TSG-RAN WG4 Meeting #114  		R4-2501599

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on other RRM requirement impacts in R19 MIMO

Agenda Item:	7.22.3.2

Document for:	Discussion

Introduction

In RAN4 #113, the WF [1] is approved. Moreover, an incoming LS from RAN1 is received [2].

We have no strong view so far on CJT calibration reporting. In this contribution, we analyse the RRM requirement impacts from UL-only TRP.

Discussion

<On asymmetric DL sTRP/UL mTRP >

In RAN4 #112bis, the follow issue was discussed and the agreements are achieved as follows.

Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting.

There are generally two issues regarding above agreements.

The first issue is related to the LS in [2]. RAN4 has asked one question for this objective and RAN1’s reply is as follows.

RAN4 is discussing the ‘asymmetric DL sTRP/UL mTRP deployment scenario’ objective of the WI for FR2 deployments. In one proposed scenario, the UL TRP(s) have full UL reception functionality but downlink functionality limited to SSB transmission. Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID. The answer to the Question 1 in LS R1-2409353 is: - From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

Based on RAN1 reply, there are generally two modes:

1) UE does not expect to receive SSB from UL TRP(s)

2) UE may expect to receive SSB from UL TRP(s)

For Mode 1) above, in our understanding, completely UL-only transmission would cause a lot of spec impact to UE RF requirements and testing in FR2-1. In our view R19 TU might not be enough to deal with such big impact. Therefore, it is slightly preferred to deprioritize this case for FR2-1. For FR1, RAN4 can also discuss whether to specify 2-TA timing reference for this case.

On the other hand, for Mode 2) above, UE may expect to receive SSB from the UL-only TRP. In this case, RAN4 may consider to Re-use R18 conclusions for 2-TA.

Proposal 1  RAN4 does not specify the RRM requirements for UL-only TRP where UE does not expect to receive SSB for FR2-1 in Rel-19.

Proposal 2  For the case where UE may expect to receive SSB for the UL-only TRP, RAN4 reuse R18 RRM requirements for 2TA.

For FR1 where UE does not expect to receive SSB from the UL-only TRP, RAN1 has discussed this issue with good progress. RAN1 #118bis agreements are cited as follows.

Agreement in RAN1 #118bis Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

R18 2-TA is based on two separate downlink timing references, however, in R19, one downlink reference timing is agreed to be applied to both TAGs. Therefore, RAN1 also had the discussion regarding whether UE autonomous TA adjustment can be applied to both TAGs. In our view this is RAN4 issue and shall be discussed in RAN4.

Observation 1  RAN1 has agreed that one downlink timing reference is applied to both TAGs.

Proposal 3  For the case where UE does not expect to receive SSB for the UL-only TRP, RAN4 to further discuss whether UE autonomous TA adjustment is only applicable to the first TAG.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  RAN4 does not specify the RRM requirements for UL-only TRP where UE does not expect to receive SSB for FR2-1 in Rel-19.

Proposal 2  For the case where UE may expect to receive SSB for the UL-only TRP, RAN4 reuse R18 RRM requirements for 2TA.

Observation 1  RAN1 has agreed that one downlink timing reference is applied to both TAGs.

Proposal 3  For the case where UE does not expect to receive SSB for the UL-only TRP, RAN4 to further discuss whether UE autonomous TA adjustment is only applicable to the first TAG.

- References

[1]  R4-2420108  WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek.  RAN4 #113

[2]  R4-2417117  Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement, RAN1.  RAN4 #113