# 3GPP TSG-RAN WG4 Meeting #114  		R4-2501602

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on RRM requirements for common signal or channel adaptation in R19 NES

Agenda Item:	7.25.2.2

Document for:	Discussion

Introduction

In RAN4 #112b and RAN4 #113, the WFs [1], [2] are approved.

Based on all above information, we provide our views on RRM requirements for common signal or channel adaptation in R19 NES.

Discussion

<On impacts to L1 measurements>

Here L1 measurements refers to the first 4 bullets above, i.e. L1-RSRP for serving cell, L1-SINR, BFD, CBD.

Firstly, our understanding is that CBD is only performed after beam failure is detected. Also note that there is no legacy CBD transition period requirement defined in TS 38.133. Therefore, we prefer to follow the legacy way, i.e. not to define RRM requirements if the adaptation of SSB periodicity happens during the time UE performs CBD.

Proposal 1  RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs CBD.

For BFD, RAN1 has only agreed to adapt SSB on SCells. In [1], the following is agreed based on RAN1 progress.

Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism)

In TS 38.133, the following is captured.

TS 38.133 v18.8.0 ### 8.5.1 Introduction The UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set q0 as specified in TS 38.213 [3] in order to detect beam failure on: - PCell in SA, NR-DC, or NE-DC operation mode, - PSCell in NR-DC and EN-DC operation mode, - SCell in SA, NR-DC, NE-DC or EN-DC operation mode, - Deactivated PSCell in NR-DC and EN-DC operation mode The RS resource configurations in the set q0 on PCell, PSCell or deactivated PSCell (if configured with bfd-and-RLM with value true) can be periodic CSI-RS resources and/or SSBs. RS resource configuration in the set q0 on SCell shall be periodic CSI-RS.

Therefore, based on existing RAN1 agreements, there is no need to discuss SSB adaptation for BFD.

Proposal 2  RAN4 NOT to discuss the SSB adaptation impact on BFD, since the SCell BFD requirements are only applicable when the RSs are periodic CSI-RSs. RAN4 may further discuss impacts on RLM/BFD requirements if RAN1 agrees SSB adaptation on SpCell.

Another issue would be whether to consider inter-cell L1 measurement for this SSB adaptation. In Rel-17, ICBM with inter-cell L1 measurement is supported by R17 MIMO WI. So far there is no discussion in RAN1 yet regarding whether SSB adaptation also needs to consider the inter-cell scenario. In our view, support of SSB adaptation under inter-cell scenario is possible considering intra-DU deployment, but the corresponding signalling and mechanism need to be agreed in RAN1/2 first.

Proposal 3  The support of SSB adaptation under inter-cell scenario needs to be discussed and agreed in RAN1/2 first. RAN4 postpone corresponding discussion until RAN1/2 make conclusions.

<On impacts to TCI activation and SCell activation>

This is similar to the issue of CBD. In legacy, change of SSB measurement period due to change of DRX cycle length is also not considered in TCI activation and SCell activation. It would be a corner case if this happens. The rule shall be followed when we discuss SSB periodicity adaptation.

Proposal 4  RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation.

<On impacts to L3 measurements>

In previous meeting, RAN4 has sent LS to RAN2 regarding the whether there is any SMTC impacts. Therefore, we think RAN4 shall wait for RAN2 conclusion first. RAN4 may stop the corresponding discussion if RAN2 agrees no impact to SMTC.

Proposal 5  For impacts to L3 measurements, RAN4 to wait for RAN2 conclusion regarding whether there is impact to SMTC due to SSB adaptation.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs CBD.

Proposal 2  RAN4 NOT to discuss the SSB adaptation impact on BFD, since the SCell BFD requirements are only applicable when the RSs are periodic CSI-RSs. RAN4 may further discuss impacts on RLM/BFD requirements if RAN1 agrees SSB adaptation on SpCell.

Proposal 3  The support of SSB adaptation under inter-cell scenario needs to be discussed and agreed in RAN1/2 first. RAN4 postpone corresponding discussion until RAN1/2 make conclusions.

Proposal 4  RAN4 to follow legacy rule that no RRM requirement is defined if the adaptation of SSB periodicity happens during the time UE performs TCI switch or SCell activation.

Proposal 5  For impacts to L3 measurements, RAN4 to wait for RAN2 conclusion regarding whether there is impact to SMTC due to SSB adaptation.

- References

[1]  R4-2416860  WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei.  RAN4 #112bis

[2]  R4-2420242  WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei.  RAN4 #113