3GPP TSG-RAN WG4 Meeting #114								    R4-2501624

Athens, GR, 17th – 21st February, 2025

Agenda Item: 9

Source: MediaTek Inc., Orange, T-Mobile USA, Vodafone, Verizon

Title: Introducing Tx switching for 3Tx UEs in Rel-19 TEI

Document for: Discussion

# 1	Introduction

New requirements for 3Tx UEs has been introduced since Rel-18, e.g., in the non-spectrum WI 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC and spectrum WI R18_3Tx_NR_CA_ENDC. It provides an opportunity of concurrent UL-MIMO and UL CA, which is one key technology in boosting UL throughputs.

In Rel-19, this 3Tx evolution continues. New WIs were approved. NR_ENDC_RF_Ph4 has one objective to specify HPUE with CA for 3Tx, while the corresponding spectrum-related works are distributed in multiple basket WIs, such as DC_R19_xBLTE_yBNR, NR_CADC_SUL_R19 for PC3 and HPUE_DC_LTE_NR_R19, HPUE_NR_CADC_SUL_R19 for HPUE.

Based on the good foundation of above works and a very strong demand in the commercial market, we believe it is the time to further introduce the Tx switching mechanism for 3Tx UE in TEI-19 for even higher UL throughputs.

In this paper, we provide the motivation as well as the analysis of system benefit and required spec effort to introduce Tx switching for 3Tx UEs.

# 2	Discussions

Market demand

Strong demand of introducing Tx switching for 3Tx UE is shown in the contribution [1] which is co-sourced by 14 companies, including 12 operators from different areas. The reason is that it can bring significant system benefit with very limited spec effort, which we will provide the analysis in the following.

Observation 1: There is a very high market demand to introduce Tx switching of 3Tx UEs.

System benefit

Considering the case of FDD+TDD CA with 3Tx UE, without Tx switching the 3Tx antennas needs to be distributed into the 2 CCs according to the RRC configuration, i.e., either {TDD 2T + FDD 1T} or {TDD 1T + FDD 2T}. In Figure 1, we provide a rough calculation to show that allowing Tx switching can further enhance the UL throughput around 25%. To avoid too detail throughput calculations, we use a “simplified throughput metric”. The assumptions are listed below:

- TDD UL-DL pattern: 3D-1S-2U-4D

- FDD: SCS 15KHz, BW 20MHz (106 RBs)

- TDD: SCS 30KHz, BW 100MHz (273 RBs)

- Simplified throughput metric: (# of RBs) x (# of Tx) x (# slots)

As can be seen from Figure 1,

- For {TDD 2T + FDD 1T}, the 2 Tx can only be used during TDD UL slots. In other slots (most of the time), only 1 Tx is used in FDD. This is definitely a waste of UE Tx antenna resource. After aggregating TDD and FDD, the simplified throughput metric is 1622.

- For {TDD 1T + FDD 2T}, FDD carrier can enjoy 2Tx all the time. However, FDD carrier only has 106 RBs and 5 slots in a cycle. During TDD UL slot, only 1Tx can be used, which cannot fully utilize the wide BW benefit (273 RBs) of the TDD carrier. The simplified throughput metric is 1606.

- With Tx switching, we can enjoy both the benefit of FDD (more opportunity of transmission in time domain) and TDD (larger BW). The idea is extremely simple.

  - During TDD DL slots, it is very obvious that keeping 2Tx at TDD carrier brings zero benefit. Therefore, a wise choice is to allow 2Tx to FDD carrier.

  - During TDD UL slots, the best strategy is to allow 2Tx in TDD carrier to get the benefit of wide TDD BW.

The simplified throughput metric is 2046, which provides 26% and 27% gain over the 2 previous cases, respectively.

Figure 1. Simplified throughput metrics for {TDD 2T + FDD 1T}, {TDD 1T + FDD 2T} and Tx switching

Based on above calculation, we can see the UL throughput gain is significant. Different TDD UL-DL pattern or different FDD/TDD BW may lead to different gains. By changing these configurations (including UE power class), the gain could be lower or even higher. Note that this gain can only be achieved by DCI, which is fast enough to adopt the TDD UL-DL pattern. The gain cannot be achieved through RRC reconfiguration.

Observation 2: Allowing Tx switching of 3Tx UEs can bring significant UL throughput over UEs configured with either {TDD 2T + FDD 1T} or {TDD 1T + FDD 2T}.

Spec effort

Based on above analysis, we can see already significant gain under 2CC CA. Therefore, to control the spec impact, we believe it is sufficient to start from Tx switching only within 2 CCs. Also, even if UE has 3Tx, there is no intention to either use all 3Tx on the same CC or distribute 3Tx into 3 CCs. We found that the scope we want for 3Tx switching well fits that of Rel-17 Tx switching. Therefore, we suggest the 3Tx switching mechanism can be built on top of Rel-17 Tx switching foundation, instead of Rel-18 which has more than 2 CCs.

Thanks to the very good foundation that has been built in Releases 16 and 17, many of the UE capabilities and RRC parameters can be directly re-used. We list some of the examples that are used in current RAN4 specs:

- UE capability

  - uplinkTxSwitchingPeriod2T2T-r17: Since UE still uses up to 2Tx in a band, this UE capability can be directly reused.

  - uplinkTxSwitching-DL-Interruption-r16: This UE capability has been reused in Rel-17 and Rel-18. We do not see a problem to further reuse it here.

  - uplinkTxSwitching-OptionSupport2T2T-r17: This UE capability indicates whether UE can do switchedUL or dualUL. In our understanding, 3Tx UE should anyway be able to do both. Thus, this capability can still be re-used with a certain limitation to 3Tx UEs.

- RRC signaling

  - uplinkTxSwitchingPeriodLocation: This is to allow network to control where to case interruption. Since UE can already support this from Rel-16, there is no problem to inherit it in Rel-19.

Observation 3: The UE capabilities and RRC signalling introduced in Rel-16 and Rel-17 can be directly re-used for Tx switching of 3Tx UEs.

With the understanding of what can be reused, now it becomes easier to discuss the spec impact. Based on our analysis, the spec impacts are mainly in TS38.214, TS38.101-1 and TS38.133.

  - TS 38.214: Text updates to add switching case between {2T+1T} and {1T+2T} in clause 6.1.6.2

  - TS 38.101-1: New section for ON-OFF mask requirements in clause 6.3A.3.3.

  - TS 38.133: New section for interruption requirements in clause 8.2.2.2.10

The required changes are very simple. The draft CRs are attached below for reference.

Observation 4: The spec effort of introducing Tx switching of 3Tx UEs is very limited.

Based on above discussions, it is obvious that allowing Tx switching for 3Tx UEs bring obvious system benefit with very limited spec effort. Therefore, we suggest introducing Tx switching for 3Tx UEs in Rel-19 TEI.

Proposal 1: Introduce Tx switching for 3Tx UEs in Rel-19 TEI.

We expect that RAN1 TEI-19 will be closed after RAN1#121 (May’25). This means RAN1 cannot wait for RAN4 agreement in Aug. Late conclusions in RAN4 would be problematic for those RAN4 TEIs with RAN1 impact. Therefore, we suggest RAN4 to have some early TEI discussions and allowing endorsing/agreeing CRs in RAN4#114b (Apr’25) and RAN4#115 (May’25) meetings. RAN1 can then proceed based on RAN4 agreements. Therefore, key commercial features will not miss the chance to be introduced.

Proposal 2: RAN4 to have early TEI-19 discussions and endorse CRs in RAN4#114b (Apr’25) and RAN4#115 (May’25) meetings.

# 3	Summary

In this paper, we provide the motivation, system benefit and spec impact to introduce Tx switching of 3Tx UEs. We have the following observations and proposals

Observation 1: There is a very high market demand to introduce Tx switching of 3Tx UEs.

Observation 2: Allowing Tx switching of 3Tx UEs can bring significant UL throughput over UEs configured with either {TDD 2T + FDD 1T} or {TDD 1T + FDD 2T}.

Observation 3: The UE capabilities and RRC signalling introduced in Rel-16 and Rel-17 can be directly re-used for Tx switching of 3Tx UEs.

Observation 4: The spec effort of introducing Tx switching of 3Tx UEs is very limited.

Proposal 1: Introduce Tx switching for 3Tx UEs in Rel-19 TEI.

Proposal 2: RAN4 to have early TEI-19 discussions and endorse CRs in RAN4#114b (Apr’25) and RAN4#115 (May’25) meetings.

# 7	References

- [1] RP-243003, Tx switching for 3Tx UEs, MediaTek Inc., BT plc, China Telecom, China Unicom, CHTTL, CKH IOD UK LIMITED, ISSDU, OPPO, Softbank, Telecom Italia, Telstra, T-Mobile USA, Verizon, Vodafone