TSG-RAN Working Group 4 (Radio) meeting #114	R4-2501649

Athens, EU, 17th to 21th February 2025

Source:	Ericsson

Title:	TP to TS 38.141-2: Addition of spatial emission requirement to protect FSS UL within band n104

Agenda item:	7.13.4.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#113 in Orlando) extensive discussion resulted in a working document to be used as baseline for coming text proposals.

According to the WID [1], the work related to the introduction of new protection requirement for FSS UL within band n104 is planned to end at this meeting.

In this contribution we provide additional information to be captured in the conformance test specification with respect the introduction of EEIRP requirement for band n104. At the end of this contribution a text proposal is attached and presented for approval.

# Discussion

In this contribution we provided missing information related to:

- Manufacturer declarations required for EEIRP requirement

- Description of test requirement and links between local coordinate system and global coordinate system

- EIRP pattern resolution

- Test beam set description

Based on the provided information addition to draft specification text is created in a text proposal TS 38.141-2 attached at the end of this contribution.

The changes added by the text proposal can be summarized as:

- In subclause 4.6, addition of information for EEIRP for declaration D.10 (Steering range).

- In subclause 4.6, improvement of description for declaration of mechanical down-tilt range D.64.

- In subclause 4.6, removal of NOTE since its redundant.

- In subclause 6.9.4.1, addition of initial condition for mechanical down-tilt.

- In subclause 6.9.4.2, Removal of redundant information with respect to time duration required for measured RMS sample.

- In subclause 6.9.4.2, addition on reference to Annex O with description to test beam directions.

- In subclause 6.9.5, addition of relation between local coordinate system and global coordinate system.

- In Annex N, addition of missing information on EIRP sampling grid and maximum allowed sampling resolution.

- Addition of Annex O, with description of test beam directions.

Proposal 1: Include information from text proposal attached at the end of this contribution to TS 38.141-2.

  - Manufacturer declarations

In Rel-13/15 manufacturer declarations for AAS base stations was defined for OTA requirements applicable for BS type 1-H and BS type 1-O. For the EEIRP requirement the BS steering range capability needs to be declared. This declaration is already defined for other OTA requirements with declaration D.10.

For EEIRP it is foreseen that different steering ranges can be used for different mechanical tilt settings. Therefore, it is important to develop a concept that allows for declaration of multiple steering ranges.  BS manufacturer can therefore declare multiple set of D.10 if the base station is capable to support multiple configurations, as needed indifferent deployment situations.

In addition, a new declaration of mechanical down-tilt range is required for a specific steering range (D.10). Therefore, a new declaration, D.64 is introduced. This declaration is tied to D.10 for the EEIRP requirement. The mechanical down-tilt declarations are needed when the measured EEIRP in local coordinate system is compared to the requirement limit formulated in the global coordinate system referring to the horizon.

An example with multiple steering ranges and associated declaration of OTA peak directions set, maximum steering direction and mechanical down-tilt range is visualised in Figure 2.1-1.

# Figure 2.1-1: Relation between D.10 and D.64 for the EEIRP requirement

The addition to current declaration framework is listed in Table 2.1-1. New information unique for the EEIRP requirement is marked in red text.

Table 2.1-1: Declarations needed for the EEIRP requirement

| Declaration identifier | Declaration | Description | Applicability |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H | BS type 1-O | BS type 2-O |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax:The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). For the EEIRP requirement in subclause 6.9, the OTA peak direction set maximum steering direction(s) describes the BS steering range(s) for a declared mechanical tilt range (D.64). | x | x | x |
| D.64 | Mechanical down-tilt range | For BS required to meet EEIRP requirement in subclause 6.9 the mechanical down-tilt range as maximum (θmech,max) and minimum (θmech,min) supported mechanical down-tilt angle should be declared. | x | x | n/a |

Observation 1: For EEIRP conformance testing, we do not need to alter previous declaration related to supported beams. The essential part for EEIRP relates to the beam steering range capabilities.

Observation 2: For the EEIRP requirement the declaration of beam steering range capability is essential. This information will be needed to determine where test beams should be directed. RAN4 need to establish links between declaration D.10 and new declaration D.64 for mechanical down-tilt range.

  - Test requirement

In RAN4 BS RF specification, the inclusion of the test requirement is typically a copy of the RF core requirement. For EEIRP the challenge is that the limits are defined on a global coordinate system referring to the horizon. This coordinate system is currently not defined in the BS test specification. Hence, a relation between the local coordinate system declared by the manufacturer, and the global coordinate system used by ITU-R, needs to be related with respect to eventual mechanical down-tilt used in the deployment at the site. For situations where mechanical tilt is considered, the supported steering range might be affected.

This can be handled by shifting the measured EEIRP before comparing to the requirement profile using the following approximate transformation between the local and the global coordinate system.

θH=-θ+θMT

, where

θH is the angle with respect to the horizon defined between +90° and -90°. Above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

 is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°.

MT is the down-tilt angle representing the mechanical tilt defined between -90° and +90°.

This approximation is valid for reasonable values of mechanical down-tilt. The exact expression coordinate transformation used for simulation purposes can be found in TR 36.814, Annex A.2.1.6.2.

An alternative approach could be to express the requirement limit in the local coordinate system as function of mechanical tilt.

Observation 3: Since the WRC-23 condition relates to a global coordinate system referring to the horizon, RAN4 need to establish links to the local coordinate system used in AAS specifications.

  - EIRP pattern resolution

Considering the requirement for OTA spatial emission above the horizon there are two aspects to consider in relation to how to define the EIRP pattern spatial resolution required to guarantee accurate measurement of EEIRP:

- The sampling resolution required to capture the radiation above the horizon. For this, sufficient number of spatial samples are needed to sufficiently capture emission generated in the upper hemisphere including side lobes and grating lobes.

- The size of the elevation bins varies over the elevation range from 5 degrees at low elevations to 30 degrees at high elevation angles. This means that the number of vertical samples is limited by the bin size.

The first aspect can be resolved by taking enough spatial samples required by the spatial sampling Nyquist criteria.

Considering the minimum bin size of 5 degrees in the WRC-23 outcome, it would be safe to set the maximum vertical sampling resolution to 1 degree.

For the horizontal sampling resolution, the maximum sampling resolution can be determined by the physical size of the array antenna panel. The largest physical dimension is often used to characterize the beamforming capability of an antenna. Instead of adding a new declaration of beamwidth characteristics, it is more straightforward to use the physical largest dimension of the array antenna. A declaration of beamwidth would require a characterization of beamwidth to find the most narrow supported beamwidth. Such characterization would require substantial measurement effort.

The maximum EIRP sampling resolution in degrees required for the EIRP pattern measurement is given by:

∆θ=1∆φ=180πλD

, where l is the wavelength and D is the physical length of the array antenna diagonal. The physical length of the array antenna diagonal can be determined by the following expression:

D=MMsubdel2+Ndaz2

, where M is the number of sub-array rows, Msub is the number of elements in the sub-array, del is the vertical element separation, N is the number of sub-array columns and daz is the azimuth element separation. All these parameters are design parameters known by the manufacturer. For external testing, this information is typically provided in product datasheets. If not, the test lab can easily find out by physical inspection.

The physical size of the array antenna will determine the narrowest beam width supported by the base station. There are expressions in antenna literature to relate physical size and minimum beamwidth. The challenge with beamwidth as such is that it should be measured before declared, which is more complex than just declaring physical size based on general parameters.

Observation 4: The horizontal EIRP pattern sampling resolution is set by the physical size of the BS array antenna.

  - Test beam set description

Based on the declaration of the steering range capability, the test beams are located within the steering range. Different options on how to place the test beams and on how many test beams are required to measure accurate EERIP profiles, have been discussed for a long time. There are some fundamental options related to how to capture the test beam constellation in the conformance test specification. Two straightforward approaches to further consider are listed below as Case A and Case B.

- A table holding description of test beam steering direction as a function of D.10 parameters. A table would need a row per test beam direction with an equation with D.10 parameters as input.

- For certain test beam constellations with uniform beam distribution, the test beam direction can be described with a set of equations for qa,b and ja,b separately.

For Case B, uniformly distributed test beams are considered. The steering range is divided into uniformly distributed sub-areas. The test beam direction is located in the center of each sub-area. Based on the declared steering range capability (part of D.10), the test beam constellation consisting of K=AB test beams is determined as:

θa,b=aAθmax-θmin-θmax-θmin2+θminφa,b=bBφmax-φmin-φmax-φmin2+φmin

for a=1..A rows and b=1..B columns, where parameters jmin, jmax, qmin and qmax is provided from D.10 declaration.

If other test beam constellations are considered, Case A may be more suitable with the additional complexity to develop and maintain separate mathematical expression per each test beam.

In Figure 2.4-1, some examples produced based on Case B assuming A=4, 6 and B=4, 6.

# Figure 2.4-1: Examples of test beam constellations

Observation 5: Considering a situation with the steering range divided into uniformly distributed rectangles. It is from a specification drafting perspective beneficial to adopt a concept where the test beams are located in the middle of declared steering range sub-box.

# Conclusion

In this contribution we provide some essential information not captured by the working document aiming for a draft TS 38.141-2.

At the end of this contribution a text proposal for TS 38.141-2 is attached for approval. The text proposal adds missing information presented in this contribution.

With respect to the conformance test specification discussion, we have observed following:

Observation 1: For EEIRP conformance testing, we do not need to touch previous declaration related to supported beams. The essential part for EEIRP relates to the beam steering range capabilities.

Observation 2: For the EEIRP requirement the declaration of beam steering range capability is essential. This information will be needed to determine where test beams should be directed. RAN4 need to establish links between declaration D.10 and new declaration D.64 for mechanical down-tilt range.

Observation 3: Since the WRC-23 condition related to a global coordinate system referring to the horizon, RAN4 need to establish links to the local coordinate system used in AAS specifications.

Observation 4: The horizontal EIRP pattern sampling resolution is set by the physical size of the BS array antenna.

Observation 5: Considering a situation with the steering range divided into uniformly distributed rectangles. It is from a specification drafting perspective beneficial to adopt a concept where the test beams are located in the middle of declared steering range sub-box.

Based on the observation we propose following proposals to progress the work:

Proposal 1: Include information from text proposal attached at the end of this contribution to TS 38.141-2.

# References

[1]	RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC

[2]	R4-2511648, “TP to TR 38.908: Addition of technical background for EEIRP”, Ericsson

TEXT PROPOSAL:

## 4.6	Manufacturer's declarations

The following BS manufacturer's declarations listed in table 4.6-1, when applicable to the BS under test, are required to be provided by the manufacturer for radiated requirements testing for BS type 1-H, BS type 1-O and BS type 2-O.

For the BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3], clause 4.6.

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 3) | x | x | x |
| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Supported bands declared for every beam (D.3). (Note 4) | c | x | x |
| D.5 | BS requirements set | Declaration of one of the NR base station requirement's set as defined for BS type 1-H, BS type 1-O, or BS type 2-O. | c | x | x |
| D.6 | BS class | Declared as Wide Area BS, Medium Range BS, or Local Area BS. | c | x | x |
| D.7 | BS channel band width and SCS support | BS supported SCS and channel bandwidth per supported SCS. Declared for each beam (D.3) and each operating band (D.4). | c | x | x |
| D.8 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | x | x | x |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) Φmax:The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) Φmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) θmax: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) θmin: The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). For the EEIRP requirement in subclause 6.9, the OTA peak direction set maximum steering direction(s) describes the BS steering range(s) for a declared mechanical tilt range (D.64). | x | x | x |
| D.11 | Rated beam EIRP | The rated EIRP level per carrier (Prated,c,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared for every beam (D.3). (Note 12, 14, 18, 20) | x | x | x |
| D.12 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3). | x | x | x |
| D.13 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.12) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. | x | x | x |
| D.14 | Parallel beams | List of beams which have been declared equivalent (D.13) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. | x | x | x |
| D.15 | Number of carriers at maximum TRP | The number of carriers per operating band the BS is capable of generating at maximum TRP declared for every beam (D.3). | n/a | x | x |
| D.16 | Operating bands with multi-band dependencies | List of operating bands which are generated using transceiver units supporting operation in multiple operating bands through common active RF components. Declared for each operating band for which multi-band transceiver is used. | c | x | n/a |
| D.17 | Maximum radiated Base Station RF Bandwidth | Maximum Base Station RF Bandwidth in the operating band, declared for each supported operating band (D.4). (Note 15) | c | x | x |
| D.18 | Maximum Radio Bandwidth of the operating band with multi-band dependencies | Largest Radio Bandwidth that can be supported by the operating bands with multi-band dependencies. Declared for each supported operating band which has multi-band dependencies (D.16). | c | x | n/a |
| D.19 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (D.52). | c | x | x |
| D.20 | CA-only operation | Declared of CA-only (with equal power spectral density among carriers) but not multiple carriers operation, declared per operating band (D.4) and per beam (D.3). | c | x | x |
| D.21 | Maximum number of supported carriers per operating band in multi-band operations | Maximum number of supported carriers per supported operating band declared to have multi-band dependencies (D.16). | c | x | n/a |
| D.22 | Contiguous or non-contiguous spectrum operation support | Ability of BS to support contiguous or non-contiguous (or both) frequency distribution of carriers when operating multi-carrier in an operating band. | c | x | x |
| D.23 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a |
| D.24 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.23). (Note 5) | x | x | n/a |
| D.25 | OTA sensitivity supported BS channel bandwidth and SCS | The BS supported SCS and channel bandwidth per supported SCS by each OSDD. | x | x | n/a |
| D.26 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD. | x | x | n/a |
| D.27 | Minimum EIS for FR1 (EISminSENS) | The minimum EISminSENS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared per NR supported channel BW for the OSDD (D.30). The lowest EIS value for all the declared OSDD's is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. (Note 6) | x | x | n/a |
| D.28 | EIS REFSENS for FR2 (EISREFSENS_50M) | The EISREFSENS_50M level applicable in the OTA REFSENS RoAoA, (used as a basis for the derivation of the FR2 EISREFSENS for other channel bandwidths supported by BS). (Note 7) | n/a | n/a | x |
| D.29 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D.31) for each OSDD. | x | x | n/a |
| D.30 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. (Note 8) | x | x | n/a |
| D.31 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D.30). (Note 9) | x | x | n/a |
| D.32 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D.33). | x | x | n/a |
| D.33 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. | x | x | n/a |
| D.34 | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. (Note 10) | x | x | x |
| D.35 | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (D.34). (Note 11) | x | x | x |
| D.36 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. | x | x | x |
| D.37 | The rated carrier OTA BS power, Prated,c,TRP | Prated,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band. (Note 12, 14, 18, 20) | n/a | x | x |
| D.38 | Rated transmitter TRP, Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 12,14, 18, 20) | n/a | x | x |
| D.39 | CLTA placement for co-location test | The manufacturer shall declare the side of EUT where radiating elements are placed closest to the edge of EUT when applicable. The CLTA shall be placed at the EUT side where radiating elements are placed closest. | n/a | x | n/a |
| D.40 | Spurious emission category | Declare the BS spurious emission category as either category A or B with respect to the limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [5]. | c | x | x |
| D.41 | Additional operating band unwanted emissions | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where the additional operating band unwanted emission limits defined in clause 6.7.4 apply. (Note 16, Note 19) | c | x | x |
| D.42 | Co-existence with other systems | The manufacturer shall declare whether the BS under test is intended to operate in geographic areas where one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA, PHS and/or NR operating in another operating band are deployed. | c | x | x |
| D.43 | Co-location with other base stations | The manufacturer shall declare whether the BS under test is intended to operate co-located with Base Stations of one or more of the systems GSM850, GSM900, DCS1800, PCS1900, UTRA FDD, UTRA TDD, E-UTRA and/or NR operating in another operating band. | c | x | n/a |
| D.44 | Single-band RIB or multi-band RIB | List of single-band RIB and/or multi-band RIB for the supported operating bands (D.4). | c | x | n/a |
| D.45 | Single or multiple carrier | BS capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RIB. (Note 17) | c | x | x |
| D.46 | Maximum number of supported carriers per operating band | Maximum number of supported carriers. Declared per supported operating band, per RIB. (Note 15) | c | x | x |
| D.47 | Total maximum number of supported carriers | Maximum number of supported carriers for all supported operating bands. Declared per RIB. | c | x | x |
| D.48 | Other band combination multi-band restrictions | Declare any other limitation under simultaneous operation in the declared band combinations (D.16), which have any impact on the test configuration generation. | c | x | n/a |
| D.49 | Ncells | Number corresponding to the minimum number of cells that can be transmitted by a BS in a particular operating band. Declared per operating band (D.4). | c | n/a | n/a |
| D.50 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band. Declared per operating band (D.4). | c | x | x |
| D.51 | Maximum supported power difference between carriers is different operating bands | Maximum supported power difference between any two carriers in any two different supported operating bands. Declared per operating bands combination (D.52). | c | x | n/a |
| D.52 | Operating band combination support | List of operating bands combinations supported by single-band RIB(s) and/or multi-band RIB(s) of the BS. | c | x | n/a |
| D.53 | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. | n/a | x | x |
| D.54 | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (D.53). | n/a | x | x |
| D.55 | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. | n/a | x | x |
| D.56 | Supported frequency range of the NR operating band | List of supported frequency ranges representing fractional bandwidths (FBW) of operating bands with FBW larger than 6%. | x | x | x |
| D.57 | Rated beam EIRP at lower end of the fractional bandwidth (Prated,c,FBWlow) | The rated EIRP level per carrier at lower frequency range of the fractional bandwidth (Prated,c,FBWlow), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges (D.56). (Note 12, 13, 14, 15, 18, 20) | x | x | x |
| D.58 | Rated beam EIRP at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh) | The rated EIRP level per carrier at higher frequency range of the fractional bandwidth (Prated,c,FBWhigh), at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared per beam for all supported frequency ranges in (D.56). (Note 12, 13, 14 ,15, 18, 20) | x | x | x |
| D.59 | Relation between supported maximum RF bandwidth, number of carriers and Rated maximum TRP | If the rated transmitter TRP and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated transmitter TRP; - The reduced total output power at the maximum number of supported carriers. | n/a | x | x |
| D.60 | Inter-band CA | Declaration of operating band(s) combinations supporting inter-band CA. Declared per operating band combination (D.52). | c | x | x |
| D.61 | Intra-band contiguous CA | Declaration of operating band(s) supporting intra-band contiguous CA. Declared per operating band with CA support. | c | x | x |
| D.62 | Intra-band non-contiguous CA | Declaration of operating band(s) supporting intra-band non-contiguous CA. Declared per operating band with CA support. | c | x | x |
| D.63 | Total maximum number of supported carriers in multi-band operation | Maximum number of supported carriers for all supported operating bands declared to have multi-band dependencies (D.16). | c | x | n/a |
| D.64 | Mechanical down-tilt range | For BS required to meet EEIRP requirement in subclause 6.9 the mechanical down-tilt range as maximum (θmech,max) and minimum (θmech,min) supported mechanical down-tilt angle should be declared. | x | x | n/a |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type for FR1 as specified in TS 38.211 [20], i.e., type A, type B or both. | c | x | n/a |
| D.101 | PUSCH additional DM-RS positions | Declaration of the supported additional DM-RS position(s) for FR2, i.e., pos0, pos1, or both. | n/a | n/a | x |
| D.102 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [20], i.e., format 0, format 1, format 2, format 3, format 4. | c | x | x |
| <unchanged ommited> |  |  |  |  |  |
| D.127 | PUSCH with enhanced DM-RS type | Declaration of support enhanced DM-RS type as specified in TS 38.211 [17]. | x | x | n/a |
| D.128 | PRACH format for HAPS scenario | Declaration of supported PRACH format(s) for HAPS scenario, i.e., format 1. | x | x | n/a |
| NOTE 1: Manufacturer declarations applicable per BS requirement set were marked as "x". Manufacturer declarations not applicable per BS requirement set were marked as "n/a". NOTE 2: For BS type 1-H, the only radiated declarations are related to EIRP and EIS requirements. For BS type 1-H declarations required for the conducted requirements testing, refer to TS 38.141-1 [3]. For declarations marked as 'c', related conducted declarations in TS 38.141-1 [3] apply. When separately declared, they shall still use the same declaration identifier. NOTE 3: Depending on the capability of the system some of these beams may be the same. For those same beams, testing is not repeated. NOTE 4: These operating bands are related to their respective single-band RIBs. NOTE 5: As each identified OSDD has a declared minimum EIS value (D.27), multiple operating band can only be declared if they have the same minimum EIS declaration. NOTE 6: If the BS type 1-H or BS type 1-O is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. NOTE 7: Although EISREFSENS_50M level is based on a reference measurement channel with BWChannel = 50 MHz, it does not imply that BS has to support 50 MHz channel bandwidth. NOTE 8: Not applicable for BS type 2-O. NOTE 9: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. NOTE 10: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error, TAE or EVM. NOTE 11: The OTA coverage reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 12: If a BS type 2-O is capable of 64QAM DL operation but not capable of 256QAM DL operation, then up to two rated output power declarations may be made. One declaration is applicable when configured for 64QAM transmissions and the other declaration is applicable when not configured for 64QAM transmissions. NOTE 13: If D.57 and D.58 are declared for certain frequency range (D.56), there shall be no "Rated beam EIRP" declaration (D.11) for the operating band containing that particular frequency range. NOTE 14: If a BS is capable of 1024QAM DL operation then up to three rated output power declarations may be made. One declaration is applicable when configured for 1024QAM transmissions, a different declaration is applicable when configured 256QAM transmissions and the other declaration is applicable when configured neither for 256QAM nor 1024QAM transmissions. NOTE 15: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. NOTE 16: If BS is declared to support Band n20 (D.4), the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 17: In case of BS type 1-H, this declaration applies per TAB connector. NOTE 18: If a BS type 2-O is capable of 256QAM DL operation, then up to three rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, a different declaration is applicable when configured for 64QAM transmissions and the other declaration is applicable when not configured neither for 256QAM nor 64QAM transmissions. NOTE 19: If BS is declared to support Band n24 (D.4), the manufacturer shall declare if the BS may operate in geographical areas where FCC regulations apply. Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 20: If a BS is capable of 256QAM DL operation but not capable of 1024QAM DL operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, and the other declaration is applicable when not configured for 256QAM transmissions |  |  |  |  |  |

### <Next of the change>

## 6.9	OTA spatial emission

### 6.9.1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

### 6.9.2	Minimum requirement

The minimum requirement for BS type 1-H and BS type 1-O operation is defined in TS 38.104 [2], clause 9.9.2.

### 6.9.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to radiated power in specified elevation angle ranges with respect to the horizon.

### 6.9.4	Method of test

#### 6.9.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1. SC carrier, the one characterised by all of requirements below:

- Highest supported Power Spectral Density (PSD) level with the corresponding BW.

In the test range, the EUT is mounted at the positioner with mechanical down-tilt set to zero.

#### 6.9.4.2	Procedure

1. Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3

2. Place the test object at the positioner, such that blocking effect in the vertical domain is minimized.

3. Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

4. The measurement device characteristics shall be:

-	Detection mode: True RMS.

5. Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

6. Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1.. N.

7. Enable test beam  k for k=1..K as described in Annex O for all declared steering ranges (D.10).

8. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

9. Repeat step 6 and 7 for all K test beams.

10. Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

11. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in TR 38.141-2, Annex N.

NOTE: step 9 and step 10 could be exchangeable.

### 6.9.5	Test requirements

#### 6.9.5.1	Requirement for BS type 1-O

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 to 7075 MHz, the measured EEIRP  shall not exceed the limits specified in table 6.9.5.1-1 for mechanical down-tilt angle equal to zero. If non-zero mechanical down-tilt is considered, the measured EEIRP profile needs to be shifted as:

θH=-θ+θMT

, where

θH – angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

 is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°.

MT is the down-tilt angle representing the mechanical tilt defined between -90° and +90°.

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Bin number i | Elevation angular range θHLi ≤ θ < θHHi (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- | --- |
| 1 | 0≤<5 | 27 |
| 2 | 5≤<10 | 23 |
| 3 | 10≤<15 | 19 |
| 4 | 15≤<20 | 18 |
| 5 | 20≤<30 | 16 |
| 6 | 30≤<60 | 15 |
| 7 | 60≤<90 | 15 |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |  |

Figure 6.9.5.1-1: Definitions of θHLi and θHHi angles.

### <Next of the change>

# Annex N (normative):

# EEIRP calculation

# N.1	General

The EEIRP calculation is based on two averaging processes:

- Calculation of average EIRP pattern based on all test beam directions.

- Averaging over horizontal and vertical angles within specified vertical angles bins.

These two averaging processes are inter-related and should not be viewed as the order in which the calculation is done.

The required sampling resolution for the EIRP pattern measured per test beam is described in Annex N.3

# N.2	Averaging processes

In this section the details of the averaging calculation to extract the EEIRP profile from measured EIRP samples is described.

    - N.2.1 Averaging over beamforming directions

The first averaging process involves averaging over different beamforming directions. For a given vertical angle θ and horizontal angle φ with a sampling of K beamforming directions, the equation can be written as:

EIRPθnθ,φmφ=1Kk=1KwkEIRPk(θnθ,φmφ)                              (1)

The power measured is the sum of the EIRP in two orthogonal polarisations.

The discrete EIRP pattern measured for each individual test beam is denoted by EIRPk(θnθ,φmφ). For each EIRP radiation pattern sampling points can be described as θn,φm, where n=1..N and m=1..M. The number of samples required along j axis and q axis (N and M) is determined such that the criteria in Annex N.3 is fulfilled with samples over the half-sphere described by -180≤φ≤180 degrees and -90≤θ≤0 degrees (Upper half-sphere vizible from sky). In the case meachanical down-tilt is considered, additional samples is required below the antenna aperture normal is required.

    - N.2.2 Averaging over horizontal and vertical angles

The second averaging process is over a spherical strip. This average EIRP is referred to as EEIRP for a spherical strip and can be expressed as:

EIRPEEIRPθHLi , θHHi=(θHHi-θHLi)×π180°MNsinθHHi-sinθHLin=nLnHm=1MEIRPθn,φm cosθn    ,                       (2)

where

θn	is the nth midpoint between the elevation intervals (midpoint is the middle point between the intervals, e.g. for the interval 1°-2°, the midpoint is 1.5°).

φm	        is the mth midpoint between the azimuth intervals

nL        is the lowest elevation point within the θHLi , θHHi range

nH	       is the highest elevation point within the θHLi , θHHi range

M	      is the number of azimuth points within the (-180°, 180°) range

N	     is the number of elevation points within the θHLi , θHHi range

Note: All angles are in degrees.

# N.3	EIRP sampling resolution

The maximum EIRP sampling resolution in degrees required for the EIRP pattern measurement is given by:

∆θ=1∆φ=180πλD

, where l is the wavelength and D is the physical length of the array antenna diagonal. The physical length of the array antenna diagonal can be determined by following expression:

D=MMsubdel2+Ndaz2

, where M is the number of sub-array rows, Msub is the number of elements in the sub-array, del is the vertical element separation, N is the number of sub-array columns and daz is the azimuth element separation.

# Annex O (normative):

# EIRP test beam constellation

# O.1	General

For a declared steering range part of D.10, the test beam constellation consisting of K=AB test beams is determined as:

θa,b=aAθmax-θmin-θmax-θmin2+θminφa,b=bBφmax-φmin-φmax-φmin2+φmin

, where a=1..A, b=1..B, A=TBD rows and B=TBD columns. Parameters jmin, jmax, qmin and qmax is provided from manufacturer declaration of steering range.