3GPP TSG-RAN WG4 Meeting #114		 			                        R5-2501700
Athens, Greece, 17th – 21st February, 2025
 
Agenda Item: 7.11.2

Source:	SKY Perfect JSAT

Title: Discussion on VSAT UE parameters for Ku-band co-existence study

Document for:	Discussion and Decision

- Introduction

This document proposes parameters related to VSAT UE power for use in the coexistence study between TN and NTN in the Ku-band. Additionally, it provides considerations for applying the coexistence study results to the Core Requirement.

- Analysis and Discussion

  - Analysis of VSAT UE’s power-related parameters

At RAN4#113, the way forward (WF) is to conduct a first study with a parabolic antenna of 100 cm aperture (41.3 dBi gain), 88.2 dBm EIRP, and a VSAT max Tx power of 50W (for GSO). The following excerpt from R4-2419909[1] outlines the details of this way forward.

- Sub-topic 2-4: VSAT Max Tx antenna gain Agree first study is worst case (GSO) of 100 cm aperture (41.3dBi), 88.2 dBm EIRP, other options not precluded e.g. different size, EIRP and antenna type. The antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization. - Sub-topic 2-17: VSAT max Tx power Based on sub-topic 2-4, the agreement is 50 W for GSO and 10 W for NGSO.

Observation 1: At RAN4#113, the way forward (WF) is to conduct a first study using the NTN VSAT UE parameters for coexistence studies, specifically with a parabolic antenna of 100 cm aperture (41.3 dBi gain), 88.2 dBm EIRP, and a VSAT max Tx power of 50W (for GSO).

On the other hand, regarding the Array Antenna, it has been agreed at RAN#113 to conduct a coexistence study based on the following parameters. The following WF content is excerpted from R4-2419909[1].

- Sub-topic 2-1: VSAT antenna pattern Agree for VSAT antenna model, to progress the co-ex study with the VSAT equipped with parabolic antenna first. And to further discuss the phased array (flat-panel) antenna modeling in parallel. Agree to use 38.921 unless there is a need to use sub array then 38.803 The following parameters are agreed: Table 2-1 VSAT phased array antenna characteristics Ku band VSAT phased array antenna. 1; Ku band VSAT phased array antenna Characteristics. 1.1; Antenna pattern; Table 2-2 and 2-3 (TR 38.921). 1.2; Element gain (dBi) (Note 2); 0 or 3.5 ?. 1.3; Y axis / Z axis element 3 dB beam width of single element (degree); 90° for Y axis 90° for Z axis. 1.4; Y axis / Z axis element front-to-back ratio (dB); 30 dB. 1.5; Antenna polarization; Circular（RHCP or LHCP）. 1.6; Antenna array configuration (Y axis × Z axis)(Note 4); 45x45 elements. 1.7; Number of supported polarizations, P; 1. 1.8; Y axis / Z axis radiating element spacing; 0.5 lambda. 1.9; Array Ohmic loss (dB) (Note 2); 2. 1.10; Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); [4.44 dBm] TBA. 1.11; maximum coverage angle in the horizontal plane (degrees); 0~360 degrees. 1.12; vertical coverage range (degrees) between beam direction and normal direction; 0~60 degrees. 1.13; normal direction; Toward X+ axis. Note 1: Void Note 2: The element gain in row 1.2 includes the loss given in row 1.9. Note 3: The conducted power per element assumes Y axis × Z axis ×Number of supported polarizations elements (i.e. power per Y axis / Z axis polarized element). Note 4: All of the Y axis × Z axis elements are assumed as the horizontal radiating elements in the horizontal plane. Note 5: Similar to subtopic 2-4, other options not precluded e.g. antenna array configuration (Y axis × Z axis). Agree 3.5 dB element gain

From this model, the EIRP of the Array Antenna can be calculated as follows:

EIRPdBm=4.44dBm+10×log1045 ×45+3.5dB+ 10×log1045 ×45≅ 74.1(dBm)

Observation 2: At RAN4#113, the array antenna model has been agreed upon as the parameter for the NTN VSAT UE coexistence study, with an EIRP of approximately 74.1 dBm.

At RAN#106, the standardization of Ku-band covered NTN VSAT Type 4 (Mobile VSAT communicating with GSO using a mechanically steered antenna) and NTN VSAT Type 5 (Mobile VSAT communicating with GSO and NGSO using an electronically steered antenna). The WID RP-243314[2] for Ku Band for NR NTN is as follows:

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 4 and 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability Carrier aggregation is not currently in scope for this spectrum only WI but will be necessary to enable supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Note: Since only VSAT Type 4 and 5 are in scope, any implementation of type 2 (Fixed VSAT communicating with GSO and LEO with electronic steering antenna) can be based on the same RF requirements as VSAT type 5.

Observation 3: The standardization scope for Ku-band includes NTN VSAT Type 4 (Mobile VSAT communicating with GSO using a mechanically steered antenna) and NTN VSAT Type 5(Mobile VSAT communicating with GSO and NGSO using an electronically steered antenna).

SKY Perfect JSAT calculate link budget and estimate the link quality and the achievable throughput in case 5G NR waveform carrier is transmitted from Ku-band VSAT UE to GSO Software Defined Satellite and legacy GSO satellite. The details are provided in the contribution R4-2419558[3]and R4-2501691[8]. This contribution states that, as a result of the link budget study, an EIRP of 88.2 dBm is required.

Observation 4: For Ku-band GSO SDS satellites and legacy satellites, the link budget results indicate that a UE EIRP of 88.2 dBm is required.

At RAN#106, it was agreed to include NTN VSAT Type 4 and Type 5 in the standardization scope for Ku-band. And based on the above link budget results, an EIRP of 88.2 dBm is required for both NTN VSAT Type 4 and Type 5 in the case of GSO.

For NTN VSAT Type 4, as mentioned above, at RAN4#113, it was agreed in the WF to conduct the first study on coexistence using the NTN VSAT UE parameters of a parabolic antenna with a 100 cm aperture (41.3 dBi), an EIRP of 88.2 dBm, and a VSAT max Tx power of 50W (for GSO). Therefore, it would be appropriate to continue the coexistence study using these parameters. Additionally, the antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

Proposal 1:　 For the coexistence study assuming NTN VSAT Type 4, the first study should be conducted using the NTN VSAT UE parameters of a parabolic antenna with a 100 cm aperture (41.3 dBi), an EIRP of 88.2 dBm, and a VSAT max Tx power of 50W (for GSO). Additionally, the antenna model for NTN VSAT uses circular polarization, but since many GSO satellites use linear polarization, evaluations are conducted by converting to linear polarization.

For NTN VSAT Type 5, as mentioned above, it has been agreed to conduct the coexistence study with a VSAT UE EIRP of approximately 74.1 dBm. On the other hand, JSAT's link budget results indicate that an EIRP of 88.2 dBm is also required for VSAT UE Type 5 in the case of GSO.

As an example of an existing array antenna for GSO VSAT, ALL.SPACE's Ku-band array antennas [9]can support EIRP levels ranging from 79 dBm to 99 dBm. This suggests that modeling an array antenna with an EIRP of 88.2 dBm is also feasible. Therefore, in the Ku-band coexistence study, the evaluation for NTN VSAT Type 5 should also be conducted based on an array antenna model corresponding to an EIRP of 88.2 dBm.  Regarding polarization, in the case of NTN VSAT Type 5, the same approach as NTN VSAT Type 4 is applied, with evaluations conducted by converting to linear polarization when simulating GSO satellite.

Proposal 2:　 For NTN VSAT Type 5, it is appropriate to define an array antenna model with an EIRP of approximately 88.2 dBm and use it for the coexistence study. Additionally, the antenna model for NTN VSAT uses circular polarization, but since many GSO satellites use linear polarization, evaluations are conducted by converting to linear polarization when simulating GSO satellite.

Handling of Coexistence Study Results

At RAN4#112, it was decided that the frequencies for TN and NTN in the coexistence study would use the same frequencies, at 11 GHz and 14 GHz. The details of the WF from R4-2413519[4] are provided below.

- Issue 2-4: Coexistence frequencies - Use 11GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band. - Issue 2-10: TN Parameters As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from: - The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz - The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

Additionally, at RAN4#112, the scenarios for the coexistence study were agreed upon as follows in the WF.

- Issue 2-2: Coexistence scenarios Adopt the interference scenarios in [Intelsat] R4-2411507 for the coexistence studies

The scenarios in R4-2411507[5] are as follows:

Table2.2-1 R4-2411507  Table 4 Interference Scenarios

| No. | Combination | Aggressor | Victim | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |

Observation 5: It has been agreed in the WF that the coexistence study will be conducted using 11 GHz and 14 GHz for both TN and NTN.

At RAN4#112bis, the initial simulation results were presented in R4-2416140[7], showing that for Scenario No.5, a high ACS value is required for VSAT.

In response to this result, at RAN4#112bis, it was agreed in the WF that relaxation might be necessary when deriving core requirements based on the VSAT ACS simulation results from the coexistence study [6].

- Sub-topic 2-22: TN parameters NTN to NTN won’t be considered. Continue with the TN - NTN coexistence study noting that the applicability of the VSAT ACS simulation result may have to be relaxed for the core requirement derivation. Use the IMT parameters from TR38.922 version 0.3.0 for the IMT study on 15 GHz as a starting point.

Observation 6: It has been agreed in the WF that relaxation might be necessary when deriving core requirements based on the VSAT ACS simulation results from the coexistence study.

Due to changes in the transmission power parameters for VSAT UE based on the initial simulation results from RAN4#112bis, the following scenarios may be affected by this parameter change. Additionally, when determining the Core Requirement based on the coexistence study results for these scenarios, the ACLR of VSAT UE will have an impact.

Table2.2-2　 Scenarios Impacted by VSAT UE Transmission Power

| No. | Combination | Aggressor | Victim | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |

Observation 7: When the transmission power of the VSAT UE changes, Scenarios 1 and 3 are impacted. When determining the Core Requirement based on the coexistence study results for these scenarios, the ACLR value of the VSAT UE will have an impact.

Verify the actual frequency allocation of TN, which is subject to coexistence studies with the VSAT UE uplink. The frequency allocation status is summarized in R4-2411507[5], and an excerpt is provided below.

Interferers Figure 4 Priority 1 Configuration and Interfering Carriers And Figure 5 Priority 2 Configuration and Interfering Carriers

For the VSAT UE uplink in the 13.75-14.5 GHz range, the only TN frequency allocation under consideration is 14.8-15.35 GHz, as discussed at WRC-27, meaning that TN and NTN frequencies are separated by more than 300 MHz.

For the VSAT UE uplink in the 12.75-13.25 GHz range, there is currently no TN frequency allocation adjacent to NTN.

Observation 8: The only TN frequency allocation subject to coexistence studies with the VSAT UE uplink in the 13.75-14.5 GHz range is 14.8-15.35 GHz, meaning that TN and NTN frequencies are separated by more than 300 MHz. For the VSAT UE uplink in the 12.75-13.25 GHz range, there is currently no TN frequency allocation adjacent to NTN.

The impact of the VSAT UE uplink on TN is expected to be significantly smaller, as the frequencies are separated by more than 300 MHz. This differs greatly from the results of a coexistence study conducted under the assumption of using the same frequency or the frequencies adjacent to each other (i.e.: no frequency separation).

Observation 9: The impact of the VSAT UE uplink on TN is expected to be significantly smaller, as the frequencies are separated by more than 300 MHz. This differs greatly from the results of a coexistence study conducted under the assumption of using the same frequency or the frequencies adjacent to each other (i.e.: no frequency separation).

Proposal 3: RAN4 should take into account that the actual frequency allocation of TN and NTN is separated by more than 300 MHz when considering the ACLR of VSAT UE based on the coexistence study results. Given this separation, it should be noted that relaxation may be necessary when deriving core requirements from the ACLR simulation results.

- Conclusions

Observation 1: At RAN4#113, the way forward (WF) is to conduct a first study using the NTN VSAT UE parameters for coexistence studies, specifically with a parabolic antenna of 100 cm aperture (41.3 dBi gain), 88.2 dBm EIRP, and a VSAT max Tx power of 50W (for GSO).

Observation 2: At RAN4#113, the array antenna model has been agreed upon as the parameter for the NTN VSAT UE coexistence study, with an EIRP of approximately 74.1 dBm.

Observation 3: The standardization scope for Ku-band includes NTN VSAT Type 4 (Mobile VSAT communicating with GSO using a mechanically steered antenna) and NTN VSAT Type 5(Mobile VSAT communicating with GSO and NGSO using an electronically steered antenna).

Observation 4: For Ku-band GSO SDS satellites and legacy satellites, the link budget results indicate that a UE EIRP of 88.2 dBm is required.

Proposal 1:  For the coexistence study assuming NTN VSAT Type 4, the first study should be conducted using the NTN VSAT UE parameters of a parabolic antenna with a 100 cm aperture (41.3 dBi), an EIRP of 88.2 dBm, and a VSAT max Tx power of 50W (for GSO). Additionally, the antenna model for NTN VSAT uses circular polarization, but since many GEO satellites use linear polarization, evaluations are conducted by converting to linear polarization

Proposal 2:　 For NTN VSAT Type 5, it is appropriate to define an array antenna model with an EIRP of approximately 88.2 dBm and use it for the coexistence study. Additionally, the antenna model for NTN VSAT uses circular polarization, but since many GSO satellites use linear polarization, evaluations are conducted by converting to linear polarization when simulating GSO satellite.

Observation 5: It has been agreed in the WF that the coexistence study will be conducted using 11 GHz and 14 GHz for both TN and NTN.

Observation 6: It has been agreed in the WF that relaxation might be necessary when deriving core requirements based on the VSAT ACS simulation results from the coexistence study.

Observation 7: When the transmission power of the VSAT UE changes, Scenarios 1 and 3 are impacted. When determining the Core Requirement based on the coexistence study results for these scenarios, the ACLR value of the VSAT UE will have an impact.

Observation 8: The only TN frequency allocation subject to coexistence studies with the VSAT UE uplink in the 13.75-14.5 GHz range is 14.8-15.35 GHz, meaning that TN and NTN frequencies are separated by more than 300 MHz. For the VSAT UE uplink in the 12.75-13.25 GHz range, there is currently no TN frequency allocation adjacent to NTN.

Observation 9: The impact of the VSAT UE uplink on TN is expected to be significantly smaller, as the frequencies are separated by more than 300 MHz. This differs greatly from the results of a coexistence study conducted under the assumption of using the same frequency or the frequencies adjacent to each other (i.e.: no frequency separation).

Proposal 3: RAN4 should take into account that the actual frequency allocation of TN and NTN is separated by more than 300 MHz when considering the ACLR of VSAT UE based on the coexistence study results. Given this separation, it should be noted that relaxation may be necessary when deriving core requirements from the ACLR simulation results.

- References

- R4-2419909, Way Forward for [113][313] NR_NTN_Ku_Band_General; Moderator(Eutelsat Group), RAN4#113

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN; Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#106

- R4-2419558, Ku band Maximum TRP and EIRP Analysis; SKY Perfect JSAT, RAN#113

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General; Moderator (Eutelsat Group), RAN4#112

- R4-2411507, Ku Band Co-existence: Intelsat, RAN#112

- R4-2416953, Way Forward for [112bis][313] NR_NTN_Ku_Band_General; Moderator(Eutelsat Group), RAN4#112bis

- R4-2416140, NTN Ku-band – coexistence study; Ericsson

- R4-2501691, Discussion on UE RF requirements for NR NTN Ku band; SKY Perfect JSAT, RAN4#114

- ALL.SPACE Web site, “The S2000 Series,” https://www.all.space/products-services/s2000-terminal-range