# 3GPP TSG-RAN WG4 Meeting #114	R4-2501732

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.12.1.1

Source: 		Qualcomm Incorporated

Title: 	Discussion on Rel-19 ATG UE RF requirements

Document for:	Approval

# Introduction

In RAN4#113, UE RF requirements for Rel-19 ATG enhancement were discussed, and the agreements are captured in WF [1]. In this paper, we provide our further views on inter-band CA and UL MIMO.

# 2. Discussion

## 2.1 Inter-band CA

The issue of whether to allow n39 UL for CA_n3A-n39A was discussed, the following agreements were captured in [1].

Issue 2-1: whether to allow n39 UL for CA_n3A-n39A Agreement: - The conclusions about allowing n39 UL for CA_n3A-n39A if any should only apply to ATG and will not have any impact on existing NR specification for legacy NR UE. - Further check if the performance of these parameters in the following table can be achieved in ATG UE with 5MHz frequency gap in n39 filter. - Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; 50dB. n39 Tx filter rejection at 1805~1880MHz; 50dB. - Other RF architectures or parameters are not precluded.

The implementation solutions have been extensively investigated for NR handheld UE and the conclusion is that n39 UL is not allowed in CA_n3-n39 [2]. In [1], an exemplary RF architecture with separate antenna were proposed assuming 5MHz frequency gap is reserved between DL band n3 and band n39.

To analyze the feasibility and MSD values, the key parameters include antenna isolation, the rejection for band n3 Rx filter at 1885~1915MHz and n39 Tx filter rejection at 1805~1880MHz. In [1], the following parameters were assumed in Table 1.

Table 1: Parameters for ATG UE with 5MHz frequency gap in n39 filter in [1]

| Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 50dB |
| n39 Tx filter rejection at 1805~1880MHz | 50dB |

For antenna isolation, 10dB is usually assumed for handheld UE analysis. While ATG UE can have more flexibility to provide better antenna isolation thanks to the big form factor. Therefore, 20dB antenna isolation can be assumed for the ATG UE.

For n39 Tx rejection at 1805~1880MHz, since the spurious emissions of n39 are specified as -15.5dBm/5MHz at 1885-1880MHz and -33dBm/5MHz at 1805-1885MHz, respectively, we can estimate that the leakage and PA noise from n39 to n3 is about-27dBm/5MHz. Therefore, 40dB Tx filter rejection from n39 to n3 can be assumed.

For the rejection of band 3 Rx at 1885~1915MHz, n3 DL filter is almost not attenuation anything at n39 range starting from1885MHz when assuming a typical filter. Even we consider a specific high capability filter for ATG, 20dB rejection is a reasonable assumption. 50dB rejection listed in [1] is a too optimistic assumption.

With the above assumptions, the MSD value can be calculated by below table assuming 23dBm transmit power and 30MHz BW.

Observation 1: Based on the assumptions of 20dB antenna isolation, 20dB rejection for band n3 at Rx filter, and 40dB n39Tx filter rejection, the MSD for band n3 is 14.4dB with 30MHz UL BW and 23dBm transmit power.

Proposal 1: RAN4 to take the above 14.4dB MSD as the starting point when enabling n39 UL for CA_n3A-n39A.

Another issue discussed in RAN4#114 is whether preclude Case 4 and no conclusion was reached.

Issue 1-1-2: UE antenna type for inter-band CA Agreement: - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed. Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.. WF: - Further check whether preclude case 4.

The UE antenna type is highly depending on the band combination (e.g., whether two bands are close to each other) and the implementation (e.g., different vendors might have different antenna design even for the same band or band combination). Therefore, it’s not possible to conclude a unified antenna design for the UE antenna type for the specific band combination from RF perspective. Based on our understanding, RRM session are discussing the MRTD requirements with different cases on the UE antenna type. Since it is hard to conclude UE antenna type for inter-band CA from RF perspective, we suggest RRM session to define the MRTD requirements based on the worst-case assumption for UE antenna type.

Proposal 2: From RF perspective, we suggest RRM session to define the MRTD requirements based on the worst-case assumption for UE antenna type.

## 2.2 UL MIMO

The issues of whether ULFPTx mode for UL-MIMO is supported for ATG UE were discussed in previous meetings. The following agreement was made in last meeting [1].

Issue 4-2: ULFPTx mode for UL-MIMO WF: - FFS - Companies are courage to analyze the additional specification impact and benefit if ULFPTx mode for ATG is introduced.

The intention of ULFPTx mode was specified to increase UL power with 2Tx for NR UE. While for ATG, there is no UE power class, and the maximum output power of ATG UE is indicated by signalling. This means that the maximum transmit power of ATG UE can be reported based on actual capabilities with the sum of transmit power from all the connectors regardless of with or without ULFPTx. In addition, defining the ULFPTx mode might limit the implementation flexibility for ATG UE. For example, for handheld UE, 26dBm with mode 1 mean PA1 is with 23dBm capability and PA2 is with 23dBm capability while for ATG UE if UE indicate the maximum transmit power as 36dBm with mode 1, it is not necessary to use two 33dBm PAs.

Observation 2: Defining ULFPTx mode might limit the flexibility of ATG UE implementation.

From specification impact perspective, if ULFPTx mode is introduced for ATG UE, the legacy RF requirements such as maximum output power, and signalling design for legacy ULFPTx mode need to be changed. In addition, it is still not clear what’s the benefit from network PoV if this feature is introduced considering there already has not been limit on the maximum output power.

Proposal 3: RAN4 not to introduce ULFPTx mode for ATG UE before it is clear for RAN4 that what’s the benefit from network PoV.

# 3.	Conclusion

In this paper, we provide our views on UE RF requirements in ATG enhancements. The following observations and proposals are made:

Observation 1: Based on the assumptions of 20dB antenna isolation, 20dB rejection for band n3 at Rx filter, and 40dB n39Tx filter rejection, the MSD for band n3 is 14.4dB with 30MHz UL BW and 23dBm transmit power.

Proposal 1: RAN4 to take the above 14.4dB MSD as the starting point when enabling n39 UL for CA_n3A-n39A.

Proposal 2: From RF perspective, we suggest RRM session to define the MRTD requirements based on the worst-case assumption for UE antenna type.

Observation 2: Defining ULFPTx mode might limit the flexibility of ATG UE implementation.

Proposal 3: RAN4 not to introduce ULFPTx mode for ATG UE before it is clear for RAN4 that what’s the benefit from network PoV.

# 4.	References

- R4-2420338, WF on UE RF for ATG enhancement, CMCC

- TS 38.101-1 v18.6.0