3GPP TSG-RAN WG4 Meeting # 114							R4-2501796

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.5.2.3

Source: 	OPPO

Title: 	on non-collocated other aspects

Document for:	Approval

- Introduction

During the RAN4#113 meeting, the UE architecture and UE type definition has been fully discussed and the WF agreement has been captured at [1]. Still there is some ambiguity and we would like to further discuss this issue.

- Discussion

The WF agreement has been captured as below.

<Issue 2-3-1:  UE Capability Type definition for EN-DC and NR-CA>

Way forward:

- Continue further discussion in Issue 1-1.

<Issue 2-3-2:  Reference UE architecture>

Agreement:

- No need to discuss the table in the future RAN4 meetings.

  - The table in WID is not reference point for RAN4.

It has been already agreed that not to further discuss the table in the future RAN4 meeting. However, still we might need some clarification of the situation that the UE types we have been using since from Rel-17 till Rel-19 and perhaps to be further used in the future release.

In our previous meeting discussion paper, the UE architecture has been fully discussed and captured again below:

Observation 1: For LTE intra-band NC CA, one shared LNA has been used for the two RX chains for the two non-contiguous carriers.

Observation 2: For NR intra-band NC CA, similar architecture of LTE has been agreed as baseline architecture.

Figure 1 LTE intra-band NC CA baseline RF architecture

The LTE baseline RF architecture for intra-band NC CA only considers 2 layer for each CC. While for NR intra-band NC CA, 4 layer for each CC is mandatory below figure 2 shows the baseline RF architecture.

Figure 2 NR intra-band NC CA baseline RF architecture

In this case, the LNA is shared while the rest of the RX chain starting from mixer to digital baseband are all separate. This is quite similar to the type 3b UE architecture as listed in the table 1 below for UE architecutre.

Observation 3: based on the LTE UE intra-band NC CA architecture baseline, the baseline for NR intra-band NC CA is shown as figure 2 and marked as type 3b.

Here we can see the difference introduced for the topic is that the UE type 1 which has been considered as baseline architecture seems not the truth as to consider the LTE intra-band non-contiguous CA. Also, as captured above from our previous discussion paper, the NR intra-band NC CA baseline RF architecture will be shared LNA architecture as shown in figure 2. As the LNA will treat the CC1+CC2 together and the power difference of CC1 and CC2 is quite limited. This has been agreed as 6dB PSD difference in LTE intra-band NC CA.

Observation 1: The NR intra-band NC CA with shared LNA also follows LTE intra-band NC CA that 6dB power imbalance of the two CCs is required.

Furthermore, as we have defined the type 2 and type 4 UE who has totally separate RX chain for the 4, 6 and 8 layers, the 25dB power imbalance can be reached which has also considered the deployment scenario for non-collocated BS.

Observation 2: The NR intra-band NC CA with non-collocated deployment needs UE receiver to handle 25dB power imbalance of the two CCs and the totally separate RX chain can be one of the implementations.

So, the above observation 1 and observation 2 are already studied and agreed. The definition of type 2 and type 4 is quite clear and the requirement is also clear. Currently we are working on the accomplishment for the BS signalling and this issue is discussed in the other agenda item.

A further step to this WID might be whether there exists some implementation that can handle more than 6dB but less than 25dB power imbalance. This needs further study. However, current WID scope only covers the type 2 and type 4 UE requirement and signalling, so this up scope needs RAN discussion.

Observation 3: The potential forward of the WID is to investigate whether there exists some implementation that can handle more than 6dB but less than 25dB power imbalance and define such requirement and signalling.

Proposal 1: Current work on type 4 has no ambiguity when considering the UE baseline architecture.

Proposal 2: Whether to further study the potential forward as to investigate whether there exists some implementation that can handle more than 6dB but less than 25dB power imbalance will leave to RAN discussion.

3	Conclusions

In this paper, we give discussion on the UE architecture and potential future work on up scoping the WID, the observations and proposals are as below:

Observation 1: The NR intra-band NC CA with shared LNA also follows LTE intra-band NC CA that 6dB power imbalance of the two CCs is required.

Observation 2: The NR intra-band NC CA with non-collocated deployment needs UE receiver to handle 25dB power imbalance of the two CCs and the totally separate RX chain can be one of the implementations.

Observation 3: The potential forward of the WID is to investigate whether there exists some implementation that can handle more than 6dB but less than 25dB power imbalance and define such requirement and signalling.

Proposal 1: Current work on type 4 has no ambiguity when considering the UE baseline architecture.

Proposal 2: Whether to further study the potential forward as to investigate whether there exists some implementation that can handle more than 6dB but less than 25dB power imbalance will leave to RAN discussion.

4 References

[1] R4-2420399	WF on non-collocated intra-band EN-DC and NR-CA, KDDI