3GPP TSG-RAN WG4 Meeting # 114							R4-2501799

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.3.2

Source: 	OPPO

Title: 	UE RF requirements for intra-band non-contiguous CA

Document for:	Approval

- Introduction

During the last RAN4#113 meeting, the A-MPR simulation assumption for intra-band non-contiguous CA has been discussed and a WF has been agreed. Based on the WF, we provide the A-MPR simulation and the first round result has been collocated and provided in this paper for further check.

- Discussion

During the Rel-15 discussion, when the SUL_n78_n80 was introduced, different implementation has been discussed as shown below which is captured from [1]:

ssue 1-1-1: A-MPR simulation cases for intra-band NCCA with NS_33

Agreement:

- Evaluate 10+10MHz channel bandwidth combination

WF agreement:

- Use 5855MHz to 5925MHz with 7 channels for A-MPR channel configuration.

Issue 1-1-2: A-SEM requirements for intra-band NCCA with NS_33

Agreement:

  - Apply the NR NCCA SEM principle

Issue 1-1-3: A-SE requirements for intra-band NCCA with NS_33

WF agreement:

- The A-SEM requirement applies only within 5855 MHz to 5925 MHz according to ETSI EN 302 571.

- The A-SE requirement applies as shown in Table below and according to ETSI EN 302 571.

| Protected band | Frequency range (MHz) |  |  | Maximum Level (EIRP) | MBW (MHz) |
| --- | --- | --- | --- | --- | --- |
| Frequency range | 5835 | - | 5855 | -30 | 1 |
| Frequency range | 5925 | - | 5945 | -30 | 1 |

Based on the simulation assumption, below configuration has been used to develop the A-MPR requirement.

Table 1 A-MPR simulation configuration

| case | LCRB1 | RBSTART1 | LCRB2 | RBSTART2 | SCS | CBW1 | CBW2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 0 | 10 | 42 | 15 | 10 | 10 |
| 2 | 10 | 18 | 10 | 22 | 15 | 10 | 10 |
| 3 | 10 | 19 | 10 | 22 | 15 | 10 | 10 |
| 4 | 10 | 27 | 10 | 14 | 15 | 10 | 10 |
| 5 | 10 | 33 | 10 | 7 | 15 | 10 | 10 |
| 6 | 10 | 34 | 10 | 7 | 15 | 10 | 10 |
| 7 | 10 | 41 | 10 | 0 | 15 | 10 | 10 |
| 8 | 25 | 0 | 25 | 27 | 15 | 10 | 10 |
| 9 | 25 | 19 | 25 | 8 | 15 | 10 | 10 |
| 10 | 25 | 20 | 25 | 8 | 15 | 10 | 10 |
| 11 | 25 | 26 | 25 | 1 | 15 | 10 | 10 |
| 12 | 30 | 0 | 30 | 22 | 15 | 10 | 10 |
| 13 | 30 | 19 | 30 | 3 | 15 | 10 | 10 |
| 14 | 30 | 19 | 30 | 2 | 15 | 10 | 10 |
| 15 | 30 | 21 | 30 | 1 | 15 | 10 | 10 |
| 16 | 50 | 1 | 50 | 0 | 15 | 10 | 10 |
| 17 | 10 | 1 | 10 | 8 | 15 | 10 | 10 |
| 18 | 12 | 33 | 12 | 7 | 15 | 10 | 10 |
| 19 | 15 | 31 | 15 | 5 | 15 | 10 | 10 |
| 20 | 20 | 29 | 20 | 3 | 15 | 10 | 10 |

As the major requirement for the A-MPR is the ASE requirement which needs -30dBm/MHz at the band edge. This will be quite stringent to the UE. Hence, we have simulated the first CC at 5865 channel and 5875 channel to see the difference. For the gap, we have applied the 10MHz. For the upper edge of the band, it should be quite the same as the band lower edge and hence the simulation is omitted.

The results are shown as below:

Table 2 Simulation result for A-MPR

| case | A-MPR 5865MHz | A-MPR 5875MHz | MPR |
| --- | --- | --- | --- |
| 1 | 9.3 | 7.5 | 5.6 |
| 2 | 7.5 | 7.5 | 7.5 |
| 3 | 8.2 | 8.2 | 8.2 |
| 4 | 8.4 | 8.4 | 8.4 |
| 8 | 7.7 | 5.5 | 5.5 |
| 9 | 10.1 | 10.1 | 10.1 |
| 10 | 11.4 | 11.4 | 11.4 |
| 12 | 7.6 | 6.2 | 5.7 |
| 17 | 8.4 | 8.4 | 8.4 |

We see in table 2 only some of the results have been listed. This is because as analysed in previous meeting for MPR simulation, the image and DC suppression needs some further refinement to have an appropriate MPR results and hence the A-MPR for such cases cannot be simulated well.

At least from current results, we see for 5875MHz channel slightly better A-MPR results can be observed. This can be explained that as being further away from the band edge, the ASE requirement is less stringent in such case. However, still some A-MPR results are the same as MPR results and this can be the case that ACLR is the dominate factor.

Observation 1: For channel further away from the band edge, better A-MPR can be observed.

Observation 2: Some of the configuration, the ACLR is dominant factor and A-MPR is the same as MPR.

3	Conclusions

In this paper, we give discussion on the 0us switching time for SUL combination and non-zero switching time for DC combination with SUL combination, the observations and proposals are as below:

Observation 1: For channel further away from the band edge, better A-MPR can be observed.

Observation 2: Some of the configuration, the ACLR is dominant factor and A-MPR is the same as MPR.

4 References

[1] R4-2420333	WF on NR sidelink intra-band NC CA, OPPO, LGE