3GPP TSG-RAN WG4 Meeting #114	R4-2501824

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	Discussion on system parameter and UE RF aspects of 7 MHz channel bandwidth

Agenda item:	7.9.2

Document for:	Approval

Introduction

RAN#106 has agreed the WI to introduce a new channel bandwidth of 7 MHz to NR bands n26 and n5 [1]. The objectives of the core part of this WI are quoted in the following.

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4] - No impact to 38.307 is expected. - Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective. - Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective. NOTE: Once this WI is completed, the 7 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI.

In this contribution, we discuss the system parameters and UE RF aspects of introducing a channel bandwidth of 7 MHz.

System parameters

Spectrum utilization

The new channel bandwidth of 7 MHz only includes 15 kHz SCS. For the existing 5 MHz and 10 MHz channel bandwidth, the maximum transmission bandwidth configuration is 25 and 52 RBs, respectively, as defined in Table 5.3.2-1 of TS 38.101-1. They correspond to 90% and 93.6% spectrum utilization, respectively.

For 7 MHz, a similar utilization should be implementation feasible to meet transmitter and receiver characteristics (such as emission masks to be discussed in Chapter 3) as the existing channel bandwidths. For NRB=35, 36 and 37, the spectrum utilizations are 90%, 92.6% and 95.1%, respectively. Figure 1 shows the spectrum utilization for channel bandwidths from 3 to 50 MHz with 15 kHz SCS. NRB=37 for 7 MHz looks too stringent if it is compared with the spectrum utilization of 5 MHz and NRB=35 looks too relaxed. NRB=36 looks most reasonable to maximize the spectrum utilization for 7 MHz channel bandwidth and was the highest number of RBs considered in TR 38.844 [2] for 7 MHz.

Figure 1 spectrum utilization vs channel bandwidth

It is proposed to assume 36 RBs for the maximum transmission bandwidth configuration for 7 MHz channel bandwidth.

Minimum guardband

The minimum guardband is defined in Table 5.3.3-1 of TS 38.101-1. It is based on the formula (CBW-NRB*SCS*12-SCS)/2 for all the existing channel bandwidths, where CBW is channel bandwidth, NRB is the maximum transmission bandwidth configuration and SCS is the subcarrier spacing.

With this formula applied to NRB=36 for 7 MHz, the minimum guardband is calculated to 252.5 kHz.

The minimum guardband for 7 MHz is 252.5 kHz.

Channel arrangement

Channel spacing, channel raster (including the enhanced channel raster) and sync raster are specified in 5.4.1, 5.4.2 and 5.4.3 of TS 38.101-1. The synchronization raster for above 3 MHz channel bandwidth will support any channel raster locations for 7 MHz channel bandwidth.

Channel spacing, channel raster and sync raster are not affected by the introduction of 7 MHz channel bandwidth.

UE RF

Minimum output power

It is proposed to assume the same minimum output power for 7 MHz as 5 MHz and 10 MHz.

Table 6.3.1-1: Minimum output power

| Channel bandwidth | (MHz) | 3,5,7,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -40 | -40+10log10 (BWChannel /20) | -40+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

The minimum output power for 7 MHz shall be -40 dBm.

Transmit OFF power

It is proposed to assume the same transmit off power for 7 MHz as 5 MHz and 10 MHz.

Table 6.3.2-1: Transmit OFF power

| Channel bandwidth | (MHz) | 3,5,7,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| Transmit OFF power | (dBm) | -50 |  |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

Transmit OFF power for 7 MHz shall be -50 dBm.

Occupied bandwidth

It is proposed to assume the same occupied channel bandwidth framework for 7 MHz as 5 MHz and 10 MHz.

Table 6.5.1-1: Occupied channel bandwidth

|  | NR channel bandwidth (MHz) |
| --- | --- |
|  | 3, 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Occupied channel bandwidth (MHz) | Same as NR channel bandwidth |

Occupied bandwidth for 7 MHz channel bandwidth is the same as the channel bandwidth.

Spectrum emission mask

It is proposed to assume the same emission mask framework for 7 MHz as 10 MHz.

Table 6.5.2.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 7, 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |

Spectrum emission mask for 7 MHz shall be based on the same formula as for 10 MHz.

ACLR

NR ACLR and UTRA ACLR are specified in 6.5.2.4.1 and 6.5.2.4.2 of TS 38.101-1. For 7 MHz channel bandwidth, the same ACLR levels (in Table 6.5.2.4.1-2 and 6.5.2.4.2-1 of TS 38.101-1) shall be assumed to support the adjacent NR and UTRA channel coexistence scenario in the same way as other existing channel bandwidths. The relaxation of ACLR shall not be considered to relax the channel filter requirement for 7 MHz.

The same NR and UTRA ACLR level for existing channel bandwidth shall be assumed.

Spurious emissions

General spurious emissions are specified in 6.5.3.1 of TS 38.101-1. The spurous emissions limits shall follow Table 6.5.3.1-2. The formula for OOB boundary for 7 MHz channel bandwidth shall be the same as the existing channel bandwidth 5 MHz and wider channel bandwidth, i.e., FooB=BW_channel + 5 (MHz).

Spurious emissions for 7 MHz channel bandwidth shall follow the formula for 5 MHz and wider channel bandwidths.

A-MPR

NS_12, NS_13, NS_14 and NS_15 are defined for n26. A-MPR for 7 MHz channel bandwidth needs to be evaluated.

A-MPR for NS_12, NS_13, NS_14 and NS_15 for 7 MHz channel bandwidth shall be further evaluated for NR band n26.

Maximum input level

It is proposed to assume the same maximum input level for 7 MHz as 5 MHz and 10 MHz.

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 7, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -252 | -25 + 10log10(BWChannel /20)Note 2 | -202 |
|  |  | -273,5 | -27 + 10log10(BWChannel /20)Note 3,5 | -223,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. |  |  |  |  |

Maximum input level for 7 MHz channel bandwidth is the same as 5 MHz and 10 MHz.

ACS

NR ACS are specified in 7.5 of TS 38.101-1. For 7 MHz channel bandwidth, the same ACS levels (in Table 7.5-1, 7.5-3 and 7.5-4 of TS 38.101-1) shall be assumed to support the adjacent channel coexistence scenario in the same way as other existing channel bandwidths 5 MHz and 10 MHz for n26 and n5. The relaxation of ACS shall not be considered to relax the channel filter requirement for 7 MHz.

Table 7.5-1: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 7,10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 | 30 | 27 – 10log10(BWChannel /20) |
| NOTE1: ACS value is rounded to the next higher 0.5dB value |  |  |  |  |

Table 7.5-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm |  | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5-4: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration4 | dBm | -56.5 |  | -53.5 | -50.5 + 10log10(BWChannel /20) |
| Pinterferer | dBm | -25 |  |  |  |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

For 7 MHz channel bandwidth, the same ACS level for 5 MHz and 10 MHz shall be assumed.

In-band blocking

It is proposed to assume the same in-band blocking level for 7 MHz as 5 MHz and 10 MHz.

Table 7.6.2-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB |  | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | MHz | 3 | 5 |  |  |
| FIoffset, case 1 | MHz | 4.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 7.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 3: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

It is proposed to assume the same in-band blocking level for 7 MHz as 5 MHz and 10 MHz.

Out-of-band blocking

It is proposed to assume the same out-of-band blocking level for 7 MHz as 5 MHz and 10 MHz.

Table 7.6.3-1: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Channel bandwidth | Power in transmission bandwidth configuration [dBm] |
| --- | --- |
| 3 MHz | REFSENS + 6.0 dB |
| 5 MHz | REFSENS + 6.0 dB |
| 7 MHz | REFSENS + 6.0 dB |
| 10 MHz | REFSENS + 6.0 dB |
| 15 MHz | REFSENS + 7.0 dB |
| 20 MHz | REFSENS + 9.0 dB |
| 25 MHz | REFSENS + 10.0 dB |
| 30 MHz | REFSENS + 11.0 dB |
| 35 MHz | REFSENS + 11.5 dB |
| 40 MHz | REFSENS + 12.0 dB |
| 45 MHz | REFSENS + 12.5 dB |
| 50 MHz | REFSENS + 13.0 dB |
| 60 MHz | REFSENS + 14.0 dB |
| 70 MHz | REFSENS + 14.5 dB |
| 80 MHz | REFSENS + 15.0 dB |
| 90 MHz | REFSENS + 15.5 dB |
| 100 MHz | REFSENS + 16.0 dB |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |

It is proposed to assume the same out-of-band blocking level for 7 MHz as 5 MHz and 10 MHz.

Narrow band blocking

It is proposed to scale the narrow band blocking level for 7 MHz from 5 MHz and 10 MHz. Pw=14.5 dB for 7 MHz should be assumed.

.

It is proposed to scale the narrow band blocking level for 7 MHz from 5 MHz and 10 MHz. Pw=14.5 dB for 7 MHz should be assumed.

Spurious response

It is proposed to assume the spurious response level for 7 MHz as 5 MHz and 10 MHz.

Table 7.7-1: Spurious response parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |

It is proposed to assume the same spurious response level for 7 MHz as 5 MHz and 10 MHz.

Wide band intermodulation

It is proposed to assume the same wide band intermodulation level for 7 MHz as 5 MHz and 10 MHz.

Table 7.8.2-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Pw in Transmission Bandwidth Configuration, per CC5 | dBm | REFSENS + 8 dB | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |
| BWInterferer 2 | MHz | 3 | 5 |  |  |
| FInterferer 1 (Offset) | MHz | -BW/2 – 4.5 /+BW/2 + 4.5 | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. NOTE 5: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

It is proposed to assume the same wide band intermodulation level for 7 MHz as 5 MHz and 10 MHz.

Conclusion

The following Observations and Proposals were made:

Proposal 1: It is proposed to assume 36 RBs for the maximum transmission bandwidth configuration for 7 MHz channel bandwidth.

Proposal 2: The minimum guardband for 7 MHz is 252.5 kHz.

Proposal 3: Channel spacing, channel raster and sync raster are not affected by the introduction of 7 MHz channel bandwidth.

Proposal 4: The minimum output power for 7 MHz shall be -40 dBm.

Proposal 5: Transmit OFF power for 7 MHz shall be -50 dBm.

Proposal 6: Occupied bandwidth for 7 MHz channel bandwidth is the same as the channel bandwidth.

Proposal 7: Spectrum emission mask for 7 MHz shall be based on the same formula as for 10 MHz.

Proposal 8: The same NR and UTRA ACLR level for existing channel bandwidth shall be assumed.

Proposal 9: Spurious emissions for 7 MHz channel bandwidth shall follow the formula for 5 MHz and wider channel bandwidths.

Proposal 10: A-MPR for NS_12, NS_13, NS_14 and NS_15 for 7 MHz channel bandwidth shall be further evaluated for NR band n26.

Proposal 11: Maximum input level for 7 MHz channel bandwidth is the same as 5 MHz and 10 MHz.

Proposal 12: For 7 MHz channel bandwidth, the same ACS level for 5 MHz and 10 MHz shall be assumed.

Proposal 13: It is proposed to assume the same in-band blocking level for 7 MHz as 5 MHz and 10 MHz.

Proposal 14: It is proposed to assume the same out-of-band blocking level for 7 MHz as 5 MHz and 10 MHz.

Proposal 15: It is proposed to scale the narrow band blocking level for 7 MHz from 5 MHz and 10 MHz. Pw=14.5 dB for 7 MHz should be assumed.

Proposal 16: It is proposed to assume the same spurious response level for 7 MHz as 5 MHz and 10 MHz.

Proposal 17: It is proposed to assume the same wide band intermodulation level for 7 MHz as 5 MHz and 10 MHz.

- References

- RP-243323 New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA, RAN4#106

- TR 38.844 V18.0.0 Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths (sections 6.2.2.2 and 6.7.6.1 as well as table 6.3.3-1)