3GPP TSG-RAN WG4 Meeting #114                                         R4-2501839

Athens, Greece, February 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On RF requirements for Rel-19 NR-MIMO

Agenda Item:	7.22.2

Document for:	Approval

Introduction

In previous meeting, RAN4 further discussed on this WI. The progress on FR1 3Tx and the proposed scenario for mTRP operation in FR2 can be found in [1].

<Agreement>: PC for 3Tx requirement - 3Tx PC1.5 cannot be supported for full power mode 0 in Rel-19. <Way forward>: FFS on PC1.5 for power scaling mode (3-layer) with its applicable UE type. <Agreement>: PC3 for 3Tx - Use the same MPR as MPR for PC3 single carrier. <Agreement>: PC2 for 3Tx - Use the same MPR as MPR for PC2 single carrier. - FFS on whether PC2 MPR with 1Tx or 2Tx will be applied <Way forward>: FFS on PC1.5 MPR to use the same MPR as PC1.5 MPR with 2Tx. Subject to Tx Diversity support. <Agreement>: Supported layer for mode 0 - For the performance verification of 3Tx, use 1-layer only for mode 0. <Way forward>: FFS on study of UE RF requirement for different DL/UL TCI states in the WI or TEI - R4-2417632 and R1-2410870 are for information.

In this contribution, we would like to provide our views on this topic.

Discussion

On FR1 3Tx operation

Per RAN4 discussion progress so far, most of the issues regarding introducing FR1 3Tx for single carrier operation have been tackled. One remaining issue is about PC1.5 w/o full power transmission mode with its applicable UE type. Considering that the Rel-19 WI scope on 3Tx is quite limited e.g. only non-coherent precoder and full power transmission mode 0 can be allowed, the actual output power may achieve PC1.5 only in very limited cases, which would make the support of PC1.5 quite useless at least in this release. So our preference is not to further consider PC1.5 for 3Tx in Rel-19.

Proposal 1: For Rel-19 RAN4 3Tx discussion under Rel-19 NR_MIMO_Ph5 WI, suggest to preclude PC1.5.

As for the PC2 MPR requirement for 3Tx, we think a simple solution would be directly reuse the PC2 MPR requirement for 2Tx regardless UE support of TxD.

Proposal 2: Reuse the PC2 2Tx MPR requirement for the MPR requirement for PC2 with 3Tx.

On the asymmetric DL sTRP/UL mTRP

The reply LS in [2] answers RAN4 question as below:

Reply for Question 1: From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

We think this answer is aligned with our previous understanding that the so called “UL TRP” is about whether SSB and/or other DL transmission e.g. CSI-RS will be configured by TRP but not to indicate a new type of TRP which is incapable of doing so. Thus RAN4 also needs to take this into consideration. However, from UE perspective we wonder if such kind of deployment is already covered by the spec, then seemingly no dedicated requirement for such scenario is needed.

Observation: Seemingly from UE perspective the asymmetric DL sTRP/UL mTRP is already covered by the existing RF requirements.

Proposal 3: Clarify that “UL TRP” is about whether SSB and/or other DL transmission like CSI-RS can be configured to UE rather than a term for new type of TRP which is incapable of any DL transmissions.

- If UE RF spec impact to incorporate asymmetric DL sTRP/UL mTRP is deemed necessary, no need to introduce new UE capability but one can start with reflecting the specific network DL transmission configuration.

Conclusion

In this contribution we discussed on the RF requirements for Rel-19 NR MIMO. We have the following observation and proposals:

Observation: Seemingly from UE perspective the asymmetric DL sTRP/UL mTRP is already covered by the existing RF requirements.

Proposal 1: For Rel-19 RAN4 3Tx discussion under Rel-19 NR_MIMO_Ph5 WI, suggest to preclude PC1.5.

Proposal 2: Reuse the PC2 2Tx MPR requirement for the MPR requirement for PC2 with 3Tx.

Proposal 3: Clarify that “UL TRP” is about whether SSB and/or other DL transmission like CSI-RS can be configured to UE rather than a term for new type of TRP which is incapable of any DL transmissions.

- If UE RF spec impact to incorporate asymmetric DL sTRP/UL mTRP is deemed necessary, no need to introduce new UE capability but one can start with reflecting the specific network DL transmission configuration.

- References

- R4-2420400, “WF on UE RF requirements for MIMO enhancement”, Samsung, 3GPP TSG RAN4 Meeting #113, Orlando, USA, November 18-22, 2024.

- R4-2400005, “Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement”, RAN1, 3GPP TSG RAN4 Meeting #113, Orlando, USA, November 18-22, 2024.