3GPP TSG-RAN WG4 Meeting #114	R4-2501842

Athens, Greece, February 17 – 21, 2025

Source:	Nokia

Title:	NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC

Agenda item:	7.18.3

Document for:	Discussion

Introduction

In RAN4#112, RAN4#112bis and RAN4#113 meetings, the discussions on Rel-19 NR demodulation performance Phase 5: BS Demodulation MMSE-IRC have led to several agreements, which are documented in the way forwards (WFs), i.e., [1], [2] and [3], respectively. In particular, the latest RAN4#113 meeting has the following important agreement [3]:

Interference profile - On how to account for HPUE in existing LTE profiles: - For interference profile not to consider HPUE scenario for BS MMSE-IRC requirements for this Rel-19 WI

Such an agreement brings a consequence that there would be no system-level simulation conducted by companies to obtain the proper interference profile for 5G NR with HPUEs. Hence, the interference profile from LTE in TR 36.884 [7] will be directly reused in the link-level simulations for Rel-19 NR BS Demodulation Performance MMSE-IRC. Furthermore, as documented in [2], the Dominant Interference Proportion (DIP) values in LTE in [7] were translated into their corresponding Interference-to-Noise-Ratio (INR) values to be used in the link-level simulation run by companies.

In addition to the agreements, [3] also captures the remaining open issues to be further discussed and agreed in RAN4#114. Some of those open issues are concerning simulation parameters, where few options have been proposed as possible candidates to be selected.

In this contribution, we provide our views on the remaining open issues in [3] to be addressed in RAN4#114. Some of those views are based on our simulation results presented in RAN4#114, which can be found in [4].

Discussion

The WF [3] has classified the remaining open issues into 3 subtopics, namely, Interference modelling, Test parameters and simulation, and Manufacturer declarations and applicability rules. Our views on the remaining open issues are provided in each of the respective subsections.

Interference Modelling

The remaining open issue in this subtopic is concerning the number of interferers for 2Rx:

Number of interferers for 2Rx - Option 1: Define BS MMSE-IRC requirements for 2Rx - Option 1A: 1 interferer - Option 1B: 2 interferers - Option 1C: Consider 2 interferers for HomNet and 1 interfere for HetNet - Option 2: Do not need to specify requirements for 2Rx - Companies are encouraged to provide simulation results for 1 or 2 interferers for 2Rx, to check the performance gain.

As stated in TR 36.884 [7], there is no significant gain of MMSE-IRC to the baseline MMSE receiver in the case of 2Rx when the explicit interferer number is increased from 1 to 2. Hence, in [7] and [9], 2 explicit interferers are modeled for 4Rx and 8Rx cases, while 2Rx has only 1 interferer.

Referring to TR 36.884 [7], the least SINR gain observed is 3,1 dB, which is the gain of 2Rx in Homogeneous Network (HomNet), as highlighted below:

## 7.3 Summary of link level evaluation According to the above simulation campaign, the link performance gain of MMSE-IRC over MMSE can be summarized as follows: - With respect to homogeneous network, for 1x2 cases the gain is about 3,1 dB; for 1x4 cases the gain is 4,4 dB ~4,7 dB; for 1x8 cases the gain is 4,9 dB ~5,0 dB. - With respect to heterogeneous network, for 1x2 cases the gain is 5,5 dB~5,9 dB; for 1x4 cases the gain is 7,5 dB~8,3 dB; for 1x8 cases the gain is 8,4 dB~8,9 dB. Therefore, it is concluded that the MMSE-IRC receiver can bring in the significant gain over MMSE receiver.

In RAN4#112bis and RAN4#113, some companies have provided their simulation results which were summarized in [8] and [5], respectively. In the case of 2Rx, while there is SNR gain of MMSE-IRC over MMSE, nonetheless, for HomNet, the gain is less than 3 dB, regardless of the number of interferers.

In this meeting, we provide our simulation results in [4], which also show that (similar to TR 36.884 [7])for 2Rx, there is no significant difference in the SNR gain when the interferer number is increased from 1 to 2. Furthermore, the SNR gain of MMSE-IRC over MMSE of 2Rx are less than 3dB in the HomNet cases. Only in certain Heterogeneous Network (HetNet) cases, SNR gains of over 3 dB can be observed.

In RAN4, when defining demodulation performance requirements, it is common to add an impairment margin on top of the ideal simulation. The additional impairment margin can be around 3 dB. Hence, the amount of impairment margin itself can already offset the SNR gain of MMSE-IRC over MMSE for the 2Rx. Consequently, the final requirements for the 2Rx could not separate/differentiate the MMSE-IRC and MMSE.

The simulation results show that the SNR gains of MMSE-IRC compared to MMSE in the 2Rx case are less than 3 dB, especially in HomNet, regardless of the interferer number.

In RAN4, it is common to add an impairment margin, which can be around 3 dB, to the ideal simulation results. Such an amount of impairment margin can already offset the SNR gain of MMSE-IRC over MMSE.

In the WID, the objective is to define the requirements for both HomNet and HetNet. Hence, the SNR gain of MMSE-IRC should be sufficiently high enough for both HomNet and HetNet.

RAN4 do not need to specify requirements for 2Rx since the SNR gains of MMSE-IRC over MMSE are less than 3 dB, especially in HomNet.

For the 4Rx and 8Rx cases, RAN4 shall also ensure that the SNR gains of MMSE-IRC over MMSE are more than 3 dB for both HomNet and HetNet.

Test parameters and simulation assumptions

The open issues to be addressed in this subtopic are as below:

Channel bandwidth - Proposals on the CHBW for requirement definition for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: - For 15kHz SCS, cover 5MHz, 10MHz - For 30kHz SCS, cover 10MHz, 40MHz - Option 3: minimum CBW - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS - Proposals for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz MCS - Consider the following options for performance evaluation and as candidates for further down-selection: o For 2Rx: ü Option 1: MCS 2 ü Option 2: MCS 16 o For 4Rx: ü Option 1: MCS 16 o For 8Rx: ü Option 1: MCS 20 ü Option 2: MCS 16 - Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 for MCS 2, MCS 16 and MCS 20 respectively. - Check whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration. Propagation condition – For serving cell, consider the following options for performance evaluation and as candidates for further down-selection: o Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet o Option 2: Consider different channel for different MCS ü Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10 – For the interference cell, use TDLC300-100. Waveform – whether to cover DFT-s-OFDM - Candidate options: - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only DMRS configuration - Cover DMRS configuration type 1. FFS whether to include DMRS configuration type 2. - Use DMRS port #0 for both serving and interfering PUSCH - On DMRS sequence generation, use different DMRS sequence for each gNB: - For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively - FFS DMRS sequence generation configuration for DFT-s-DFDM Test requirement definition - Candidate options: - Option 1: FFS whether SINR based definition is needed - Option 2: Use the SNR based requirement definition

As there are several issues that are related to the Waveform, i.e., to include DFT-s-OFDM or not, we will first address the issue on Waveform followed by the other issues.

Waveform – whether to cover DFT-s-OFDM

While DFT-s-OFDM is also an important waveform in 5G, nonetheless, most of the use cases in 5G would use CP-OFDM. In LTE, cf., [7][9], the requirements of BS demodulation performance for MMSE-IRC were made for DFT-OFDM. As MMSE-IRC is a receiver algorithm, which is already well defined mathematically, and its requirements for DFT-OFDM has been already defined in LTE, for Rel-19 NR BS demodulation performance MMSE-IRC, RAN4 can focus on CP-OFDM.

RAN4 to consider only CP-OFDM.

Channel bandwidth

In RAN4#112bis, we have addressed the issue on channel bandwidth in [6]. The lowest and highest bandwidth sizes shall be considered for the requirements for each SCS. If RAN4 decides to have only 1 bandwidth size for each SCS, the lowest bandwidth size for each SCS shall be considered.

Re-using FRCs currently available in 38.104 as much as possible would make the specification to be compact and easier to maintain. The bandwidth sizes considered in 38.104 are 5/10/20MHz for 15KHz subcarrier spacing and 10/20/40/100MHz for 30KHz.

RAN4 to prioritize 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS to cover both extremes and to have all DUTs testable regardless of BW support declaration. If RAN4 would reuse FRCs from 38.104, 5/20MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz shall be prioritized.

RAN4 to consider only the minimum BW if only one BW is chosen per SCS, namely, 5MHz BW with 15KHz SCS and 10MHz BW with 30KHz SCS.

MCS

It has been agreed in [2] and [3] that RAN4 will reuse the DIP from LTE, which used 16QAM as the transmitted signal from the interferers. From simulation results in [4], for 8Rx, the SNR gain of MCS16 is higher than for MCS20. Hence, RAN4 shall consider the same modulation scheme for the intended user and the interferer (i.e., 16QAM), and MCS16 shall be selected for the requirements.

RAN4 to consider the same MCS for both intended signal and interference signal, which is MCS16.

Propagation condition

In LTE case, as shown in [7] in Table 7.1-2: Cases for link level evaluations, the studies on the gain of MMSE-IRC over MMSE were performed using the same propagation condition of intended cell and interfering cell, namely, (EPA5, EPA5) and (EVA70, EVA70). While in the definition of requirements in [7] and [9], a different propagation of intended cell and interfering cell was used, namely, (EPA5, ETU5) and (EVA70, ETU70). Furthermore, as shown in [7] in Table 8.1-3: Cases for link level evaluations, the propagation condition is chosen based on the type of the network (i.e., HetNet vs HomNet).

If we refer to TS 38.104 [10] (e.g., Table 8.2.1.2-1), however, the propagation condition is chosen based on the MCS, namely, MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10. This is exactly as in option 2

In LTE, to study the MMSE-IRC gain in TR 36.884, the propagation conditions were chosen according to the network type.

In NR, in TS 38.104, the propagation condition is chosen based on the MCS.

In RAN4#113 it was agreed to reuse the FRCs in TS 38.104.

Reusing the FRCs and chosing the propagation condition based on MCS would make TS 38.104 to be compact, easier to maintain and consistent.

To have a consistency in TS 38.104, RAN4 to set to the propagation condition based on the MCS, namely, MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10.

DMRS configuration

As discussed in Section 2.2.1, only CP-OFDM needs to be considered. Furthermore, as already agreed, RAN4 shall cover DMRS configuration type 1.

RAN4 to keep previous agreements on DMRS configuration for CP-OFDM and cover DMRS configuration type 1.

Test requirement definition

As INR is used instead of DIP, similar to the NR UE specification TS 38.101-4 [11], the requirements shall also use SNR instead of SINR.

RAN4 to use the SNR based requirement definition.

Manufacturer declarations and applicability rules

The remaining open issues in this subtopic are below:

Applicability rule for different test scenarios - Proposals on applicability for Homnet and Hetnet scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS - Proposal 2: HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS - Proposals on additional applicability rule: - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Proposal 2: Introduce the applicability rules that only one of HomNet and HetNet scenarios is tested Manufacturer declarations - Proposals: - Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements Declaration identifier; Declaration; Description; Applicability. BS type 1-C; BS type 1-H. D.xxx; MMSE-IRC receiver; Declaration of BS supporting MMSE-IRC receiver.; x; x.

Applicability rule for different test scenarios

Referring to LTE in [9], the “Note*: Not applicable for Local Area BS and Home BS.” inside the tables 8.2.6.1-1 to 8.2.6.1-6 implies that the HomNet requirements are not applicable for Local Area BS and Home BS.

In TS 36.104 for LTE, the note in the requirement tables implies that HomNet requirements are not applicable for Local Area BS and Home BS, which is partly included in “Proposal 2: HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS

RAN4 to keep the same note on the applicability as in LTE specification TS 36.104 and add: “HetNet requirements are only applied for Home BS and Local BS”. Otherwise, RAN4 to have one generic applicability rule: “only one of HomNet and HetNet scenario is tested”

Manufacturer declarations

As Rel-19 NR BS demodulation performance for MMSE-IRC is directly reusing the DIP from LTE, it should follow what was done in LTE concerning any related manufacturer declaration. Furthermore, as MMSE-IRC is only one form of linear interference-rejection (or interference-cancellation) algorithms, there is no need to have a manufacturer declaration on MMSE-IRC.

RAN4 do not need to introduce a new Manufacturer Declaration on MMSE-IRC.

Conclusion

In this contribution, we provide our views on Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC. The following Observations and Proposals were made:

Observation 1: The simulation results show that the SNR gains of MMSE-IRC compared to MMSE in the 2Rx case are less than 3 dB, especially in HomNet, regardless of the interferer number.

Observation 2: In RAN4, it is common to add an impairment margin, which can be around 3 dB, to the ideal simulation results. Such an amount of impairment margin can already offset the SNR gain of MMSE-IRC over MMSE.

Observation 3: In the WID, the objective is to define the requirements for both HomNet and HetNet. Hence, the SNR gain of MMSE-IRC should be sufficiently high enough for both HomNet and HetNet.

Proposal 1: RAN4 do not need to specify requirements for 2Rx since the SNR gains of MMSE-IRC over MMSE are less than 3 dB, especially in HomNet.

Proposal 2: For the 4Rx and 8Rx cases, RAN4 shall also ensure that the SNR gains of MMSE-IRC over MMSE are more than 3 dB for both HomNet and HetNet.

Proposal 3: RAN4 to consider only CP-OFDM.

Observation 4: Re-using FRCs currently available in 38.104 as much as possible would make the specification to be compact and easier to maintain. The bandwidth sizes considered in 38.104 are 5/10/20MHz for 15KHz subcarrier spacing and 10/20/40/100MHz for 30KHz.

Proposal 4: RAN4 to prioritize 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS to cover both extremes and to have all DUTs testable regardless of BW support declaration. If RAN4 would reuse FRCs from 38.104, 5/20MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz shall be prioritized.

Proposal 5: RAN4 to consider only the minimum BW if only one BW is chosen per SCS, namely, 5MHz BW with 15KHz SCS and 10MHz BW with 30KHz SCS.

Proposal 6: RAN4 to consider the same MCS for both intended signal and interference signal, which is MCS16.

Observation 5: In LTE, to study the MMSE-IRC gain in TR 36.884, the propagation conditions were chosen according to the network type.

Observation 6: In NR, in TS 38.104, the propagation condition is chosen based on the MCS.

Observation 7: In RAN4#113 it was agreed to reuse the FRCs in TS 38.104.

Observation 8: Reusing the FRCs and chosing the propagation condition based on MCS would make TS 38.104 to be compact, easier to maintain and consistent.

Proposal 7: To have a consistency in TS 38.104, RAN4 to set to the propagation condition based on the MCS, namely, MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10.

Proposal 8: RAN4 to keep previous agreements on DMRS configuration for CP-OFDM and cover DMRS configuration type 1.

Proposal 9: RAN4 to use the SNR based requirement definition.

Observation 9: In TS 36.104 for LTE, the note in the requirement tables implies that HomNet requirements are not applicable for Local Area BS and Home BS, which is partly included in “Proposal 2: HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS

Proposal 10: RAN4 to keep the same note on the applicability as in LTE specification TS 36.104 and add: “HetNet requirements are only applied for Home BS and Local BS”. Otherwise, RAN4 to have one generic applicability rule: “only one of HomNet and HetNet scenario is tested”

Proposal 11: RAN4 do not need to introduce a new Manufacturer Declaration on MMSE-IRC.

- References

- R4-2413570, WF on NR demodulation performance: Phase 5, 3GPP RAN4#112, China Telcom, Maastricht, Netherlands, August 19-23, 2024

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#112bis, China Telcom, Hefei, China, October 14-18, 2024

- R4-2419780, Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#113, China Telcom, MediaTek, Orlando, USA, November 18-22, 2024

- R4-2501843, Simulation Results – NR BS Demodulation Performance Requirements for MMSE-IRC, 3GPP RAN4#114, Nokia, Athens, Greece, February 17-21, 2025

- R4-2418934, Simulation results summary for BS requirement with MMSE-IRC receiver, 3GPP RAN4#113, Samsung, Orlando, USA, November 18-22, 2024

- R4-2416442, NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC, 3GPP RAN4#112bis, Nokia, Hefei, China, October 14-18, 2024

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, v13.1.0, 09-2016

- R4-2415994, Simulation results summary for BS requirement with MMSE-IRC receiver, 3GPP RAN4#112bis, Samsung, Hefei, China, October 14-18, 2024

- 3GPP TS 36.104, Evolved Universal Terrestrial Radio Access (E-UTRA) - Base Station (BS) radio transmission and reception, v18.5.0, 03-2024

- 3GPP TS 38.104, NR - Base Station (BS) radio transmission and reception, V18.6.0, 06-2024

- 3GPP TS 38.101-4, NR - User Equipment (UE) radio transmission and reception, Part 4: Performance requirements, V18.6.0, 12-2024