3GPP TSG-RAN WG4 Meeting #114 	R4-2501852

Athens, Greece, Feb 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	TP to TR 38.908 – section 6.5

Agenda Item:	7.13.4.1

Document for:	Approval

1 Introduction

In RAN4#113 the TP to TR 38.908 on section 6.5 [1] was agreed and updated in the latest version of the TR [2].

As the test procedure is still not finalised there are some modifications and improvements needed to the text.

This TP discussing outstanding issues updates the TR text with the latest understanding.

2 Discussion

2.1 Initial conditions

The initial conditions section is used in the conformance specification to list the conditions under which the procedure is carried out. This generally includes environmental conditions, frequencies etc. In directional requirements it has included the test beam directions.

Currently the text is missing the explanation of the beams to be tested (or the test beam direction set) as well as the steering rages and mechanical tilt conditions. The reasoning was that it was to be in other clauses there is a test beam clause (6.3) which is currently empty however it is our understanding that this will contain information on the test beam direction set selection rather than the mechanical tilt and steering range limitations for test. If it also contains this information then the text provided here can be merged into that section.

The test beam direction set is a big issue which is currently unresolved, however this section should at least explain how the test beam direction set are referenced in the test definition.

The mechanical tilt and the steering range definitions are also important to the test definition and should be documented.

The technical background to these decisions may be documented elsewhere in the TR however this section should indicate how the conformance text deals with them.

3 TP to TR 38.908

<start of change>

6.5	Method of test

6.5.1	General

The EEIRP requirement contains 2 averaging processes, an average over the measurement points on the measurement grid (see sub-clause 6.2) and an average of beam steering directions. Mathematically these averages can be carried out in any order, as such the method of test may also carry out the averages in any order.

As it is likely that moving the positioner so a new measurement grid point can be measured will be a slower process than changing the beam steering direction, as such the method is written assuming beam steering averaging is done first.

6.5.2	Initial conditions

The initial conditions section contains the test set up initial conditions including:

- the test environment,

- the channels to be tested

- the beams to be tested or the test beam direction set in this case

- Mechanical tilt and steering range conditions for test

- etc

Test environment

EEIRP is to be measured under normal conditions

Currently only a few parameters are measured under extreme conditions which is extremely challenging as the OTA chamber cannot be environmentally controlled. Controlling the temperature of the DUT locally is done for limited tests but this could not be applied to EEIRP measurements.

RF channels

EEIRP measurement is a very long measurement, the number of channels tested will directly multiple the test time.

The frequency variation across the EEIRP band is approximately 10%, over this range, the gain of the main beam will change. However EEIRP is dominated by the sidelobe behaviour of the antenna pattern rather than the main beam, looking at the EEIRP performance of an antenna over the frequency range there is no clear worst case, the performance is no worse at Bottom channel or Top channel than it is at the middle channel.

As no channel is worse than any other it could be argued any channel could be used, as such the middle channel is the obvious choice.

Test signal and test model

EEIRP requirement is based on PSD, testing should be carried out with the highest PSD the BS is capable of.

The performance is dependent on the radiation beam shape, non-linearities or distortions potentially generated by multi-carrier signals will not significantly alter the beam shape of the wanted signal.

As such as single carrier test signal is used with test model NR-FR1-TM1.1.

As the requirement is PSD if the BS output power is constant then this will be achieved at the smallest channel band width. Potentially in order to meet EEIRP small channel bandwidth may have a limited output power capability. As specific channel BW is hence not specified only that we test with the highest PSD and corresponding channel BW.

Beams to be tested.

The test beam direction set is discussed in sub-clause 6.3. the set of beams used for the test should be described or referenced in the initial conditions.

Mechanical tilt and steering range conditions for test

The test beam direction set is applied to the BS steering range and applicable mechanical tilt conditions.

The BS should be compliant to the EEIRP requirement over all operational conditions however it can be seen from figure 6.5.2-1 that the worst case conditions with respect to steering range and mechanical tilt are quite clear.

Figure 6.5.2-1. left: Margin vs mechanical down tilt, right margin vs electrical steering range

In figure 6.5.2-1 on the left hand graph the mechanical tilt is varied for various electrical steering ranges. In all cases it can be seen that as mechanical tilt increases the margin gets larger.

On the right hand graph the electrical steering range is varied (for a fixed mechanical tilt). It is clear that as the steering range increases the margin reduces

The worst-case condition is when mechanical tilt is minimum (closest to horizon) and the electrical steering range is maximum. As such this condition should be declared and tested.

In real products we should allow for that fact that these 2 conditions may not be declared simultaneously as such the following caveat should be added.

If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt

The mechanical tilts and associated electrical steering ranges are declared.

[The minimum mechanical down tilt and the maximum electrical steering range shall be declared.

If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt]

The mechanical tilts and associated electrical steering ranges are declared.

6.5.3	Test procedure

The test procedure is based on a CATR/IAC where the BS is placed on a positioner and rotated so the required measurement direction is in line with the measurement antenna. The procedure has similarities to existing EIRP and TRP measurement procedures.

The test procedure is as follows:

1. Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3

2. Place the test object at the positioner, such that blocking effect in the vertical domain is minimized.

3. Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

4. The measurement device characteristics shall be:

-	Detection mode: True RMS.

-	[The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1 for FR1.]

5. Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

6. Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1.. N.

7. Enable test beam  k for k=1..K.

8. Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

9. Repeat step 6 and 7 for all K test beams.

10. Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

11. Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in TR 38.141-2, Annex Y.

NOTE: step 9 and step 10 could be exchangeable.

<end of change>

References

[1]	R4-2419821	TP to TR 38.908 – section 6.5– Huawei

[2] R4-2419207 TR 38.908 v1.2.0 Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz - ZTE