3GPP TSG-RAN WG4 Meeting #114 	R4-2501853

Athens, Greece, Feb 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	TP to TR 38.908 – section 6.6

Agenda Item:	7.13.4.1

Document for:	Approval

1 Introduction

In RAN4#113 the TP to TR 38.908 on section 6.6 [1] was agreed and updated in the latest version of the TR [2].

There are a number of open issues regarding the measurement uncertainty calculation which need further discussion and capturing in the TR.

This paper discusses those issues and suggests text to be captured in the TR

2 Discussion

The 2 open issues in the MU budget are currently the dynamic range uncertainty and the EEIRP sampling error.

Dynamic range uncertainty

Dynamic range uncertainty is defied as an error relating to the chamber spatial isolation.

Isolation inside the chamber depends on a number of things not least the effectiveness of the absorber material lining the chamber – looking at a range of chambers it seems the quoted isolation can be between 50 to 80dB.

In the existing MU calculations in TR 37.941 the MU value for dynamic range uncertainty is 0.51dB with a Gaussian distribution.

The smallest EEIRP requirement level is 15dBm/MHz, assuming a maximum antenna gain of 34dBi and a BS output power of 43dBm/20MHz ( a EIRP of 43 – 10*log(20) +34 = 64dBm/MHz) to achieve this an isolation of 58dB would be required.

Pint=10*log101064-5810+101510=15.5dBm/MHz i.e. 0.5dB error

As the example uses a high power BS with a high gain antenna the existing MU assumption seems appropriate considering chamber technology.

EEIRP sampling Error

EEIRP sampling error is defined as the difference between averaging over an infinite number of random beam directions and averaging over a defined subset during the test.

A single error can be identified by simulating a fixed BS configuration and comparing the results for a reference case (with a large number of beams) and the test beam direction set. This process has been used to identify suitable test beam direction sets in sub-clause 6.3.

In order for this to be valid for any BS config and declared steering range the error can be calculated for many different BS configs. This results in a distribution of errors which can be used to estimate the measurement uncertainty value. This is a measurement uncertainty as the BS is the uncertain part of the measurement, the test must be suitable for a wide range of BS configurations and steering range definitions.

Looking at 3 different antenna configurations and 8 different steering range definitions we have the following distribution of errors:

Figure 2-1 Histogram of EEIRP error for 16 beam 4x4 beam direction set.

This distribution is normal and has a standard deviation of approx. 0.75dB (Hence a 95% confidence interval of 1.5dB).

This figure can be used as the MU value for the EIRP sampling error.

TP to TR 38.908

6.6	Measurement uncertainty

The EEIRP measurement is very similar to the in-band TRP measurements, as such the in-band emission MU budgets from TR 37.941 can be used as a starting point for the EEIRP budgets.

Additional sources of uncertainty are:

Dynamic range uncertainty

[One major difference with EEIRP however is that it is necessary to measure and add power on the same frequency as the wanted signal when the wanted signal is present and steered in another direction. Existing measurements either are measuring on adjacent channels (where frequency isolation can be used) or where the wanted signal is also being measured and is dominant. As such we are relying on the chamber spatial isolation to remove the effect of the wanted signal. Reflections and refractions of the main beam will limit the minimum power level that can be accurately measured.]

Dynamic range uncertainty is defied as an error relating to the chamber spatial isolation.

Isolation inside the chamber depends on a number of things not least the effectiveness of the absorber material lining the chamber – looking at a range of chambers it seems the quoted isolation can be between 50 to 80dB.

In the existing MU calculations in TR 37.941 the MU value for dynamic range uncertainty is 0.51dB with a Gaussian distribution.

The smallest EEIRP requirement level is 15dBm/MHz, assuming a maximum antenna gain of 34dBi and a BS output power of 43dBm/20MHz ( a EIRP of 43 – 10*log(20) +34 = 64dBm/MHz) to achieve this an isolation of 58dB would be required.

Pint=10*log101064-5810+101510=15.5dBm/MHz i.e. 0.5dB error

As the example uses a high power BS with a high gain antenna the existing MU assumption seems appropriate considering chamber technology.

EEIRP sampling Error

The difference between the test beam directions set and the

Differences exist between the test beam directions set and infinite set of random beam directions and also due to errors caused by the sampling grid.

The can be referred to as EEIRP sampling error a definition is provided below:

EEIRP sample error – Error due to the sampling of the 2 averaging processes used to define EEIRP. The sampling error covers errors due to difference in power distribution due to the test beam direction set and an infinite set of random beam directions as well as errors due to the sampling grid.

EEIRP sampling error is defined as the difference between averaging over an infinite number of random beam directions and averaging over a defined subset during the test.

A single error can be identified by simulating a fixed BS configuration and comparing the results for a reference case (with a large number of beams) and the test beam direction set. This process has been used to identify suitable test beam direction sets in sub-clause 6.3.

In order for this to be valid for any BS config and declared steering range the error can be calculated for many different BS configs. This results in a distribution of errors which can be used to estimate the measurement uncertainty value. This is a measurement uncertainty as the BS is the uncertain part of the measurement, the test must be suitable for a wide range of BS configurations and steering range definitions.

Looking at 3 different antenna configurations and 8 different steering range definitions we have the following distribution of errors:

Figure 6.6-1 Histogram of EEIRP error for 16 beam 4x4 beam direction set.

This distribution is normal and has a standard deviation of approx. 0.75dB (Hence a 95% confidence interval of 1.5dB).

This figure can be used as the MU value for the EIRP sampling error.

These 2 errors have been added to the MU budget as follows:

| Table 6.6-1: IAC MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-1 | Positioning misalignment between the BS and the reference antenna |  |  | 0.03 | Rectangular | 1.73 | 1 |  |  |  |  | 0.02 |  |
| A1-2 | Pointing misalignment between the BS and the receiving antenna |  |  | 0.3 | Rectangular | 1.73 | 1 |  |  |  |  | 0.17 |  |
| A1-3 | Quality of quiet zone |  |  | 0.1 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4a | Polarization mismatch between the BS and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5a | Mutual coupling between the BS and the receiving antenna |  |  | 0 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | [0.26] | Gaussian | 1.00 | 1 |  |  |  |  | [0.26] |  |
| A1-7 | Impedance mismatch in the receiving chain |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A1-8 | Random uncertainty |  |  | 0.1 | Rectangular | 1.73 | 1 |  |  |  |  | 0.06 |  |
| A2-21 | Measurement system dynamic range uncertainty |  |  | [0.51] | Gaussian | 1 | 1 |  |  |  |  | [0.51] |  |
| *new | [EEIRP sampling error] |  |  | [0.75] | [Gaussian] | 1 | 1 |  |  |  |  | [0.75] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1-9 | Impedance mismatch between the receiving antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-11 | Impedance mismatch between the reference antenna and the network analyzer |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.04 |  |
| A1-3 | Quality of quiet zone (normal test conditions) |  |  | 0.10 | Gaussian | 1.00 | 1 |  |  |  |  | 0.10 |  |
| A1-4b | Polarization mismatch between the reference antenna and the receiving antenna |  |  | 0.01 | Rectangular | 1.73 | 1 |  |  |  |  | 0.01 |  |
| A1-5b | Mutual coupling between the reference antenna and the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| A1-6 | Phase curvature |  |  | 0.05 | Gaussian | 1.00 | 1 |  |  |  |  | 0.05 |  |
| C1-3 | Uncertainty of the network analyzer |  |  | [0.20] | Gaussian | 1.00 | 1 |  |  |  |  | [0.20] |  |
| A1-12 | Influence of the reference antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| A1-13 | Reference antenna feed cable loss measurement uncertainty |  |  | 0.06 | Gaussian | 1.00 | 1 |  |  |  |  | 0.06 |  |
| A1-14 | Influence of the receiving antenna feed cable |  |  | 0.05 | Rectangular | 1.73 | 1 |  |  |  |  | 0.03 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A1-15 | Uncertainty of the absolute gain of the receiving antenna |  |  | 0.00 | Rectangular | 1.73 | 1 |  |  |  |  | 0.00 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [1.06] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [2.07] |
| Table 6.6-1: CATR MU value derivation for EEIRP measurement, FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |  |  |  |
|  |  |  |  | 4.2<f≤7.125 GHz |  |  |  |  |  |  |  | 4.2<f≤7.125 GHz |  |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) |  |  | 0.30 | Rectangular | 1.73 | 1 |  |  |  |  | 0.173 |  |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) |  |  | 0.26 | Gaussian | 1.00 | 1 |  |  |  |  | 0.260 |  |
| A2-2a | Standing wave between BS and test range antenna |  |  | 0.21 | U-shaped | 1.41 | 1 |  |  |  |  | 0.148 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.001 |  |
| A2-4a | QZ ripple experienced by BS |  |  | 0.09 | Gaussian | 1.00 | 1 |  |  |  |  | 0.093 |  |
| A2-12 | Frequency flatness of test system |  |  | 0.25 | Gaussian | 1.00 | 1 |  |  |  |  | 0.250 |  |
| A2-17 | Measurement system dynamic range uncertainty |  |  | [0.51] | Gaussian | 1 | 1 |  |  |  |  | [0.51] |  |
| *new | [EEIRP sampling error] |  |  | [0.75] | [Gaussian] | 1 | 1 |  |  |  |  | [0.75] |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer |  |  | 0.20 | Gaussian | 1.00 | 1 |  |  |  |  | 0.20 |  |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver |  |  | 0.33 | U-shaped | 1.41 | 1 |  |  |  |  | 0.23 |  |
| A2-6 | Insertion loss of receiver chain |  |  | 0.18 | Rectangular | 1.73 | 1 |  |  |  |  | 0.10 |  |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) |  |  | 0.00 | Gaussian | 1.00 | 1 |  |  |  |  | 0.00 |  |
| A2-7 | Influence of the calibration antenna feed cable |  |  | 0.02 | U-shaped | 1.41 | 1 |  |  |  |  | 0.02 |  |
| C1-4 | Uncertainty of the absolute gain of the reference antenna |  |  | 0.43 | Rectangular | 1.73 | 1 |  |  |  |  | 0.25 |  |
| A2-8 | Misalignment positioning system |  |  | 0.00 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.00 |  |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) |  |  | 0.50 | Exp. normal | 2.00 | 1 |  |  |  |  | 0.25 |  |
| A2-9 | Rotary joints |  |  | 0.05 | U-shaped | 1.41 | 1 |  |  |  |  | 0.03 |  |
| A2-2b | Standing wave between calibration antenna and test range antenna |  |  | 0.09 | U-shaped | 1.41 | 1 |  |  |  |  | 0.06 |  |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) |  |  | 0.01 | Gaussian | 1.00 | 1 |  |  |  |  | 0.01 |  |
| A2-11 | Switching uncertainty |  |  | 0.26 | Rectangular | 1.73 | 1 |  |  |  |  | 0.15 |  |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [1.13] |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  |  |  |  |  |  | [2.21] |

As with the TRP cases the CATR is slightly higher error so we can concentrate on this when considering the final MU value.

References

[1]	R4-2419822	TP to TR 38.908 – section 6.5– Huawei

[2] R4-2419207 TR 38.908 v1.2.0 Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz - ZTE