3GPP TSG-RAN WG4 Meeting #114 	R4-2501856

Athens, GR, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	On modification of existing RX RF requirements for SBFD

Agenda Item:	7.23.2.3

Document for:	Approval

Introduction

During RAN4#113, BS RF requirements for SBFD have been discussed [6]. In this contribution, we would like to share our views regarding the open issues.

Discussion

2.1 General consideration on the grid shift assumption

In our view eventually the same grid shift assumption should be adopted for both TX and RX requirements derivation since both requirements are defined for SBFD BS and the same performance shall be guaranteed under the same deployment.

As those requirements are actually the guarantee of SBFD network performance, we feel the need to collect some measurements based on real deployment in order to cross-check the reliability of RAN4 co-ex simulation. To be specific, we extract the site location from a region where typical co-existence among operators with active NR traffic can be identified. Note that the concerning deployment is dense urban macro in C band, co-location case is also included.

According to the CDF of site distance, following characteristics for such real deployment can be observed:

- Site distance is smaller than 10m @11%-tile of the CDF

- Site distance is smaller than 50m @45%-tile of the CDF

While in RAN4 co-ex simulation, 10% grid shift (site distance sets to 500m) means that the adjacent BS is only 28.8m away, the distance would increase to 57.6m for 20% grid shift and to 144m for 50% grid shift.

Based on base station locations in one example region, the CDF of in-band blocking is simulated and the results are shown below.

And our RAN4 simulation results are also put here for comparasion, details can be seen in clause 2.4. So it is obvious that using grid shift less than 20% as RAN4 co-ex assumption will lead to an over stringent requirement.

Table 2.1-1: blocking level @ 99%CDF

| Blocking level(dBm) |  |
| --- | --- |
| Grid-shift | 99% CDF |
| 10% | -5.1 |
| 20% | -12.4 |
| 50% | -22.4 |
| 100% | -25.8 |

Observation 1: using grid shift less than 20% as simulation assumption will result over stringent requirement.

Proposal 1: it is proposed to adopt 20% grid shift to be used for both TX and RX requirements definition for adjacent channel co-existence cases.

2.1 Reference sensitivity and OTA sensitivity

Per WF in [6], the following WF was reached.

Agreement:

0.7 dB degradation on top of existing minimum requirements for OTA sensitivity.

The degradation is to add 0.7 dB addition to reference sensitivity due to self-interference. For example, the OTA sensitivity for SBFD slot can be defined as below,

Table 10.3.2-1: Wide Area BS reference sensitivity levels for SBFD slot

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -94.9 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 10.3.2-2: Medium Range BS reference sensitivity levels for SBFD slot

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -89.9 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

Table 10.3.2-3: Local Area BS reference sensitivity levels for SBFD slot

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -86.9 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |

2.2 Dynamic range requirement

Per WF in [6],

- FFS on the followings:

  - The dynamic range requirements of FR1 can reuse the existing IoT level.

  - The dynamic range requirements of FR2-1 can reuse existing IoT level.

During Rel-18 SI, it is agreed that dynamic range is still applicable for SBFD capable BS. Receiver dynamic range requirement is to ensure that the base station can receive high throughput also in the presence of increased interference levels. For FR1 20 dB interference over thermal noise is used for NR and LTE. From a scenario of view, the case of co-channel interference from uplink signals is the same as normal NR and LTE. One question is whether BS2BS CLI signal shall be included. 20 dB IOT level is high enough to reflect an acceptable coverage. Hence there is no need to consider higher dynamic range.

Proposal 2: The dynamic range requirements of FR1 can reuse the existing IoT level.

For FR2, dynamic range requirement for range2 NR BS is not defined. Per TR 38.817-02, the absolute levels for demodulation requirements shall be well above (e.g. at least ~5dB) the AWGN noise floor, thus the small dynamic range shall be handled via demodulation requirements. From the simulation results in R4-2419222 and R4-2417853, dynamic range requirements are not needed for FR2 SBFD.

Proposal 3: dynamic range requirements are not needed for FR2 SBFD.

2.3 ACS requirement

Per WF in [6],

WF:

- FFS on the followings:

  - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

For ACS and in-band blocking, both requirements are defined out of the BS channel bandwidth instead of uplink subband bandwidth. ACS and in-band blocking are defined with different interferer frequency offsets at adjacent channel(s). For channel bandwidth wider than 20 MHz, ACS interferer frequency offset is ~9.5 MHz and in-band blocking interferer frequency offset is 30 MHz, using 20 MHz NR interfering signal. And value of the interferer level is 9 dB offset. Hence, we propose to consider the two requirements together.

Proposal 4: it is proposed to consider the ACS and in-band blocking requirements together. The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level.

2.4 In-band blocking

Per WF in [6],

Agreement: - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements. - For FR1 WA BS, FFS following options. - Option 1: a unified blocking requirement to consider both scenarios - Option 2: separate requirements are defined - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - Option 3: In-band blocking requirement is defined by manufacture declared level of interference in the specified range, which comes from the system-level evaluation (from 100% grid-shifting to 10% grid-shifting). - For FR1 WA BS co-existence study, FFS what grid-shift value(s) should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - Option 5: 5% - Option 6: 20% ~ 50% - For FR2-1 WA SBFD BS, FFS following two options. - Option 1: The current FR2-1 WA BS in-band blocking requirements can be re-used - Option 2: Define in-band blocking interferer signal power to -23 dBm

There are some consideration on the consideration on the coexistence scenarios in [7] and some observations are made as below.

Observation 2: without improving the legacy TDD unwanted emission, only defining high blocking receiver requirement cannot guarantee the adjacent channel co-existence between a SBFD BS and a legacy TDD BS.

Observation 3: to guarantee the adjacent channel co-existence between SBFD BSs, both blocking level at receiver side and OBUE leakage at transmitter side should be improved.

As discussed in [R4-2413284], some simulation result is shown in table 2.4-2 with following assumption.

  - The scenarios to be studied are Scenario 1, defined in TR 38.858

  - The case to be studied is Case 3

  - absolute power level of interference from the DL adjacent channel interference

  - the reference point: before array gain

  - Grid-shift: Option-1: 10%, 20%, 50%, 100%

  - Percentage values: 99%

Table 2.4-4: blocking level @ 99%CDF

| Blocking level(dBm) |  |
| --- | --- |
| Grid-shift | 99% CDF |
| 10% | -5.1 |
| 20% | -12.4 |
| 50% | -22.4 |
| 100% | -25.8 |

From the simulation result of RAN4 simulations and the simulation based on the BS locations of a real deployment at C band, as discussed in clause 2.1, it is proposed to define in-band blocking interferer signal power to -15 dBm for FR1 WA BS.

Proposal 5: it is proposed to define in-band blocking interferer signal power to -15 dBm for FR1 WA BS.

2.5 Receiver intermodulation

Per WF in [4],

Necessity of New RX intermodulation requirement with 1 interfering signal WF: - FFS on the following options: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal. Necessity of New RX intermodulation requirement with 2 interfering signals WF: - FFS on the following: - Option 1: - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: new IMD requirement with 2 interfering signal scenario is not needed. - Option 3: No need to define new RX intermodulation requirement with 2 interfering signals if new requirement with self-interference and 1 interfering signal is defined. - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

The motivation of the two new RX intermodulation requirements are to consider BS2BS cross link interference. For general in-band blocking case as discussed in clause 2.4, the stronger intermodulation product from closer in-band blocking blocker and self sub-band DL TX is already covered. Hence there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s).

Proposal 6: there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s). The two new requirement is implicitly captured by the SBFD RX blocking requirement.

Conclusion

In this contribution we discussed on the BS RX requirement for SBFD. We have the following observations and proposals:

Observation 1: using grid shift less than 20% as simulation assumption will result over stringent requirement.

Proposal 1: it is proposed to adopt 20% grid shift to be used for both TX and RX requirements definition for adjacent channel co-existence cases.

Proposal 2: The dynamic range requirements of FR1 can reuse the existing IoT level.

Proposal 3: dynamic range requirements are not needed for FR2 SBFD.

Proposal 4: it is proposed to consider the ACS and in-band blocking requirements together. The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level.

Observation 2: without improving the legacy TDD unwanted emission, only defining high blocking receiver requirement cannot guarantee the adjacent channel co-existence between a SBFD BS and a legacy TDD BS.

Observation 3: to guarantee the adjacent channel co-existence between SBFD BSs, both blocking level at receiver side and OBUE leakage at transmitter side should be improved.

Proposal 5: it is proposed to define in-band blocking interferer signal power to -15 dBm for FR1 WA BS.

Proposal 6: there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s). The two new requirement is implicitly captured by the SBFD RX blocking requirement.

- References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958, Way Forward for [111][313] NR_duplex_evo,	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416528, Topic summary for [112bis][307] NR_duplex_evo_BSRF, Moderator (Huawei)

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF, Huawei

- R4-2419245, On modification of existing RX RF requirements for SBFD, Huawei