3GPP TSG-RAN4 Meeting #114	R4-2501861

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

Source: 	Huawei, HiSilicon

Title: 	Remaining issues on BS RF requirements for 15 GHz

Agenda Item:	7.2.3

Document for:	Approval

Introduction

In this contribution, we provide considerations and proposals on the remaining issues for TX UEM and RX blocking requirements.

Discussion

2.1 AAS BS Spectral mask

In the LS to WP5D, spectral mask for WA BS is agreed as below.

TABLE 1A

AAS BS Spectral mask (Operating band unwanted emissions limits) for 14800 - 15350 MHz operation (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz  f < 50MHz | 2dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz  f < min(100 MHz, fmax) | -14 dBm | 100 kHz |
| 100 MHz  f  fmax | -13 dBm | 1 MHz |
| NOTE: fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

It is noted that for 50 MHz  fbasic limits are adopted while for f < 50MHz the OTA limits are applied. It should be changed to OTA limits so the following update is proposed.

Proposal 1: the spectral mask is updated as below and sent the update to WP5D

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz  f < 50MHz | 2dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz  f < min(100 MHz, fmax) | -5 dBm | 100 kHz |
| 100 MHz  f  fmax | -4 dBm | 1 MHz |
| NOTE: fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

2.2 In-band blocking

Issue 4-2-X: In-band blocking

Agreement

For LS: Mention “To be specified"
For TR: FFS for next meeting

In the LS to WP5D [3], the BS ACS =30 dBc is agreed and in-band blocking level is still “To be specified”. As a comparation, the BS ACS is 46 dBc for sub-5 GHz bands. Hence the in-band blocking can also be largely reduced from existing -43 dBm blocking. The actual requirements can be further studied in the WI phase.

|  |  | IMT |  |
| --- | --- | --- | --- |
| No. | Parameter | Base station (AAS) | Mobile station |
| 4.3 | ACLR | 30 dB | 24 dB (Note 4) |
| 5.3 | Blocking response | In-band blocking level: To be specified Out-of-band blocking level: See Table 3A for radiated requirements, (ΔfOOB = 100 MHz) (Note 5) | 200 MHz BW requirements the same as [2], § 7.6A Tables 7.6A.2.1-1 and 7.6A.3-1 Bandwidth Class C for blocking levels and § 7.7 Table 7.7A-1 Bandwidth Class C for spurious response |
| 5.4 | ACS | 30 dB | 24 dB |

Proposal 2: for frequency range 14800 - 15350 MHz, the in-band blocking can be largely reduced from existing -43 dBm and the actual requirements can be further studied in the WI phase

Conclusion

In the contribution we made two proposals on the remaining issues for TX UEM and RX blocking requirements.

Proposal 1: the spectral mask is updated as below and sent the update to WP5D

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz  f < 50MHz | 2dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz  f < min(100 MHz, fmax) | -5 dBm | 100 kHz |
| 100 MHz  f  fmax | -4 dBm | 1 MHz |
| NOTE: fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

Proposal 2: for frequency range 14800 - 15350 MHz, the in-band blocking can be largely reduced from existing -43 dBm and the actual requirements can be further studied in the WI phase

- References

- R4-2417200, 3GPP TR 38.922 V0.3.0

- R4-2420330, “WF on 15GHz BS parameters”, ZTE Corporation

- R4-2420385, “LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, TSG RAN WG4