3GPP TSG-RAN WG4 Meeting #114	R4-2501901

Athens, Greece, 17th February – 21st February 2025

Agenda item:	7.18.3

Source: 		NTT DOCOMO, INC.

Title: 	Simulation results for NR MMSE-IRC BS demodulation.

Document for:	Discussion

- Introduction

We show the simulation results based on WF in RAN4#113[1].

- Simulations

PUSCH requirements with 2 interferer cells, Mapping type A, CP-OFDM, SCS:15kHz and CBW:5MHz

| Tx/Rx | MCS table | Propagation conditions | DIP set | MMSE-MRC | MMSE-IRC | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 1T2R | MCS16 | TDLA30-10 for HetNet | Set1: (-1.11, -10.91) | 15.77 | 12.59 | 3.18 |
|  |  |  | Set2: (-0.43, -13.78) | 19.96 | 13.89 | 6.07 |
|  |  | TDLB100-400 for HomNet | Set1: (-1.11, -10.91) | 17.00 | 15.55 | 1.45 |
|  |  |  | Set2: (-0.43, -13.78) | 20.71 | 18.90 | 1.81 |
| 1T4R | MCS16 | TDLA30-10 for HetNet | Set1: (-1.11, -10.91) | 12.32 | 7.03 | 5.29 |
|  |  |  | Set2: (-0.43, -13.78) | 17.02 | 7.38 | 9.64 |
|  |  | TDLB100-400 for HomNet | Set1: (-1.11, -10.91) | 13.50 | 9.84 | 3.66 |
|  |  |  | Set2: (-0.43, -13.78) | 17.90 | 11.23 | 6.67 |
| 1T8R | MCS20 | TDLA30-10 for HetNet | Set1: (-1.11, -10.91) | 11.92 | 8.23 | 3.69 |
|  |  |  | Set2: (-0.43, -13.78) | 16.59 | 8.58 | 8.01 |
|  |  | TDLB100-400 for HomNet | Set1: (-1.11, -10.91) | 13.42 | 10.04 | 3.37 |
|  |  |  | Set2: (-0.43, -13.78) | 18.12 | 10.90 | 7.22 |

PUSCH requirements with 2 interferer cells, Mapping type A, CP-OFDM, SCS:30kHz and CBW:10MHz

| Tx/Rx | MCS table | Propagation conditions | DIP set | MMSE-MRC | MMSE-IRC | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 1T2R | MCS16 | TDLA30-10 for HetNet | Set1: (-1.11, -10.91) | 15.39 | 12.43 | 2.96 |
|  |  |  | Set2: (-0.43, -13.78) | 19.85 | 13.89 | 5.96 |
|  |  | TDLB100-400 for HomNet | Set1: (-1.11, -10.91) | 16.69 | 14.18 | 2.51 |
|  |  |  | Set2: (-0.43, -13.78) | 21.23 | 16.75 | 4.49 |
| 1T4R | MCS16 | TDLA30-10 for HetNet | Set1: (-1.11, -10.91) | 12.10 | 7.01 | 5.08 |
|  |  |  | Set2: (-0.43, -13.78) | 16.67 | 7.46 | 9.21 |
|  |  | TDLB100-400 for HomNet | Set1: (-1.11, -10.91) | 13.15 | 8.78 | 4.37 |
|  |  |  | Set2: (-0.43, -13.78) | 17.79 | 10.16 | 7.63 |
| 1T8R | MCS20 | TDLA30-10 for HetNet | Set1: (-1.11, -10.91) | 12.18 | 8.14 | 4.04 |
|  |  |  | Set2: (-0.43, -13.78) | 16.81 | 8.54 | 8.27 |
|  |  | TDLB100-400 for HomNet | Set1: (-1.11, -10.91) | 13.18 | 9.32 | 3.86 |
|  |  |  | Set2: (-0.43, -13.78) | 17.88 | 10.27 | 7.61 |

- Conclusion

This contribution shows the simulation results of NR MMSE-IRC PUSCH demodulation.

References

- R4-2419780, Way Forward for [113][321] NR_demod_Ph5_Part1_General_BS, China Telecom, MediaTek, RAN4#113