3GPP TSG-RAN WG4 Meeting #114									                        R4- 2501935

Athens, Greece, Feb. 17 – 21, 2025

Agenda item:			7.1.1.3.2

Source:	Ericsson

Title:	Discussion on the MIMO layers for 6Rx UE

Document for:	Approval

# Introduction

With advancements in hardware and design, modern smartphones increasingly integrate multiple Rx antennas to enhance DL performance. For instance, certain foldable mobile phones have adopted 6Rx antennas. Incorporating 6Rx facilitates a higher peak data rate by supporting more DL MIMO layers compared to 4Rx configurations. On the other hand, due to the higher diversity, it can also help increase the downlink coverage by improving the receiver sensitivity.

In 3GPP Release 18, RAN4 finalized the standardization efforts for enabling 8Rx for CPE/FWA/vehicle/industrial type of UEs. Although 6Rx was proposed and deliberated upon, it was given lower priority during the Rel-18 discussion. To enhance DL coverage and throughput for handheld UEs with larger sizes, RAN4 has approved a new WI to specify requirements for both handheld and FWA UEs [1].

6Rx for handheld and FWA UE

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

The following sections will discuss the objectives specified in the WI, focusing on the 6Rx MIMO layers.

# Support of MIMO layers

The progress of MIMO 6 layers has been very slow during the last meeting, the WF on the MIMO layers for 6Rx are captured below [2]:

Sub-topic 2-1:	General considerations for MIMO layer evaluation for 6Rx UE

Issue 2-1-1: Tx EVM assumptions for 6-layer feasibility evaluation

Way Forward: RAN4 to further discuss the following options.

-	Option 1: Use Tx EVM = 1%

-	Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation

-	Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM

Issue 2-1-2: 6-Layer Feasibility Evaluation Assumptions

Way Forward: RAN4 to further discuss and align on simulation assumptions provided in Annex A and discuss possible options for refinement and down selection for key parameters as identified in company proposals as summarised below. The bullet list included is not intended to be a complete set of options and it does not specifically down select or preclude other options for refinement. It is only intended to summarize the options presented.

-	Antenna Configuration:

-	Limit the transmit/receive antenna configuration to 8Tx/6Rx and exclude 16Tx/6Rx and 32Tx/6Rx antenna configurations.

-	Consider both 8Tx/6Rx and 32Tx/6Rx antenna configurations

-	DMRS Configuration:

-	Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) from Annex A table.

-	Consider DMRS Configuration Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table.

-	Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) and Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table.

-	Consider additional DMRS Configuration Option 1a: Rel-18 1+1(6L), Rel-18 1+1(4L)

-	Rank

-	Include only Rank 4 and Rank 6 configurations

-	Include Rank 5, in addition to Rank 4 and Rank 6

-	Modulation and Coding Schemes for Adaptive MCS

-	Consider Table 2 only, 64QAM + 256QAM

-	Consider Table 1 only, 64QAM

-	Consider Table 1 and Table 2

-	Additional margins

-	Apply at least 3dB correction to the SNR values reported by companies in baseband simulation results

-	A 2~3 dB additional margin or 2% Rx EVM shall be considered

-	BS Antenna Correlation

-	RAN4 to use BS correlation matrix with alpha = 0.1

-	RAN4 shall use no correlation on BS Tx Ports

-	RAN4 shall not use BS correlation matrix other than 3GPP specified ones

Issue 2-1-3: Performance requirements for 6Rx

Agreement: RAN4 shall discuss the performance requirements for 6Rx once the performance part commences:

Issue 2-1-4: Way forward for performance requirements for 6-layers for handheld UE and FWA

Way Forward: RAN4 to further discuss the following options.

-	Option 1: Once the gain and feasibility for handheld UE with 6 MIMO layers are verified, then 6 MIMO layers related requirements defined for handheld UE could be applicable for FWA devices. Otherwise, no 6 MIMO layers related requirements should be defined for neither handheld UE nor FWA devices.

-	Option 2: RAN4 to introduce 6-layer performance requirements for handheld and FWA devices.

-	Option 3: RAN4 shall conclude that whether to introduce 6 layers for both FWA and handheld UE depends on the outcome of 6 MIMO layers evaluation on handheld UE.

-	Option 4: From WF in R4-2417209, companies to further discuss the following aspect of 6-layer support.

-	If the handheld feasibility is confirmed, then the handheld and FWA devices will share the same 6-layer requirements.

-	If the handheld feasibility is not confirmed, 6-layer requirements will be specified for FWA device only.

-	Option 5: Do not specify 6-layer requirements for handheld and FWA devices in Rel-19.

Issue 2-1-5: Baseband complexity

Way Forward: RAN4 to determine if RAN4 needs to analyse the increased baseband complexity and power consumption for 6Layers compared to 4Layers including LDPC decoder, channel estimation, DMRS based interference estimation and MIMO detector.

Issue 2-1-6: Issues for UE Device Types

Way Forward: RAN4 to further discuss the following proposals and determine any necessary actions.

-	Proposal 1: For the foldable UE assumed for the 6 layers feasibility study, the following aspects should be considered for evaluation with 1~2 dB margin:

-	When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

-	If the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

-	The hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

-	Proposal 2: RAN4 should clarify the nature of the proposed ECC-based correlation models and specify the types of devices that they represent

Sub-topic 2-2:	6-layer Support

Issue 2-2-1: 6-layer Support Feasibility

Way Forward: RAN4 to further discuss the following options.

-	Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

-	Option 2: 6-layer support is feasible for FWA devices

-	Option 3: Companies to further discuss align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility for handhelds using the WF in R4-2417209 and any updates related to Issue 2-1-2 above.

-	Option 4: 6-layer support is not feasible for handheld UEs

Issue 2-2-2: 6-layer Support as optional feature

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following.

-	Introduce 6 MIMO layers support as an optional feature, if 6-layer performance requirements are introduced for 6Rx.

As we have pointed out in the introduction, modern smartphones increasingly integrate multiple Rx antennas to enhance DL performance. Numerous simulation results show that supporting a higher number of DL MIMO layers can improve the DL peak throughput.

The radio channel and several other factors impact the number of layers that can be supported, and these should be thoroughly studied and analyzed.

One factor is the receive antenna correlation. When antenna elements are closely spaced, the correlation between them increases.  In the previous 8Rx discussions, the objective is to define the requirement for FWA and CPE UEs. Given their larger form factor compared to the handheld UE, the demodulation performance study assumed a low antenna correlation [3]. In the 6Rx WI, the objective is to specify FWA and handheld UE requirements. Although the example bands specified in WI are mid-bands and the antenna size can be relatively compact, we understand the difficulty of integrating 6 antenna elements into the handheld deceive without any correlation.

- Since MIMO 8 layers are already supported for FWA, it is proposed that MIMO 6 layers should also be supported, at least for FWA devices.

We have accomplished some simulations based on the parameter assumptions listed in the WF. The simulation assumption is shown in Table 1, PDSCH performance with fixed rank is shown in Figure 1, and PDSCH performance with adaptive rank is shown in Figure 2.

Table 1 Simulation parameter assumption

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz |
| SCS |  | 30 |
| Antenna configuration |  | 8Tx, 6Rx 32Tx, 6Rx |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank [4,6] |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L) |
|  | Precoding | Random (PMI Rel-15 Type I codebook) for 8Tx |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Other options omitted. |

Figure 1 PDSCH performance with fixed rank of 4 and 6 (UE correlation option 1)

From Figure 1, we can observe that with adaptive MCS but fixed rank configuration, the performance of PDSCH with rank 6 has gain over the performance with rank 4 when SNR is within higher range.

- Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 1 when SNR is within a higher range

Figure 2 PDSCH performance with fixed rank (UE correlation option 2)

Figure 2 also showed that with UE correlation option 2 and adaptive MCS but fixed rank configuration, the performance of PDSCH with rank 6 has gain over the performance with rank 4 when SNR is within higher range.

- Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 2 when SNR is within a higher range

Figure 3 PDSCH performance with fixed rank (UE correlation option 2 + 32Tx)

Under the same assumptions of figure 2 except antenna configuration, figure 3 showed that the performance of PDSCH with 32Tx and rank 6 has gain over the performance with rank 4 when SNR is within higher range.

- Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 2 plus 32Tx when SNR is within a higher range.

The maximum EVM per radio for the DL is specified in 38.104. This is a minimum requirement at the TAB connector, and so basestations may achieve lower EVM to some degree, depending on load level. Also, and importantly for AAS basestations with large arrays, the signal experienced by the UE is an OTA combination of the ensemble of the outputs of the individual transmitters. Since the wanted signal is combined coherently and EVM components that are not coherent between different transmitters will not combine, the EVM experienced at the UE may exceed the per-transmitter EVM. Taking this into account, results are presented using 1.25% OTA EVM at the UE as an example, and the actual OTA EVM will depend on the AAS type, size, frequency, etc.

We have also evaluated the PDSCH throughput performance with fixed MCS/rank conditions and the correlation captured in the 3GPP spec., as shown in 2, where we reuse the simulation assumption discussed in PDSCH demodulation requirements as much as possible. Figure 3 (a) and Figure 4 (a) showed the absolute PDSCH throughput with low and medium antenna correlation conditions, respectively. We have also shown the relative throughputs in Figure  (b) and Figure  (b) to evaluate the achieved peak throughput. We have also selected TDLA30-10, usually configured for PDSCH demodulation requirements with higher layers and/or MCS.

Table 2	Simulation parameters for evaluation of MIMO layers in 6Rx UE.

| Parameters | Values |
| --- | --- |
| Propagation channel | TDLA30-10 (TDL-A, delay spread 30ns, Doppler frequency 10Hz) |
| Antenna configuration | 8Tx, 6Rx |
| Antenna correlation | low, medium (See TS 38.101-4 Table B.2.3.2.2-1) |
| MCS | QPSK 1/3 (MCS4) 16QAM 1/2 (MCS13) 64QAM 1/2 (MCS17) |
| MIMO layers | 4 and 6 |
| Precoding | Follow PMI with Rel-15 Type-I codebook |
| PDSCH DMRS configuration | Type 1, 1+1 Single symbol for 4 layers Double symbol for 6 layers |
| Receiver type | MMSE-IRC |

| (a) Absolute throughput | (b) Relative throughput |
| --- | --- |

Figure 3	PDSCH fixed MCS/rank link-simulation results with low correlation.

| (a) Absolute throughput | (b) Relative throughput |
| --- | --- |

Figure 4	PDSCH fixed MCS/rank link-simulation results with medium correlation.

With the low antenna correlation condition, as illustrated in Figure , the performance with 6 layers achieves higher throughput compared to 4 layers for QPSK, 16QAM, and 64QAM MCS. This demonstrates a performance gain observed in the given scenario.

- With the TDLA30-10 low antenna correlation condition, the performance gain with 6 layers over 4 layers for QPSK, 16QAM and 64QAM is observed.

Under medium antenna correlation conditions, the performance with 6 layers using QPSK can achieve higher throughput, demonstrating a performance gain. However, no obvious performance gain has been observed for MCS13 and MCS17.

- With TDLA30-10 medium correlation, the performance gain with 6 layers over 4 layers for low and medium MCS is observed.

The link simulation results demonstrate a performance gain with 6 layers for QPSK, 16QAM, and 64QAM under lower antenna correlation conditions, making it applicable for FWA/CPE type devices.

In the meantime, a performance improvement with 6 layers over 4 layers was also observed under medium antenna correlation for low and medium MCS levels, which could benefit handheld devices.

Another factor is the likelihood of getting a rank 6 channel in a practical deployment scenario. Rank distributions depend on SINR distribution and propagation conditions. For example, the likelihood of getting a higher rank is low in a deployment scenario with a larger ISD and rural or suburban areas. With a relatively small ISD and dense urban area, the probability of getting a higher rank is probably high.

This WI considers both the handheld and FWA/CPE devices, and considering their differences in the form factor, the correlation concerning the real antenna arrangement in the device varies. From the analysis provided above, we could observe that, with low antenna correlation conditions, MIMO 6 layers could yield performance gain compared to 4 layers. Furthermore, support of 6 layers also depends on the likelihood of getting a rank 6 channel in the practical deployment scenarios with different ISDs. Therefore, we made the following proposal.

- The MIMO 6-layer is feasible for handheld UE and should be supported.

- The support of MIMO 6 layers could be considered an optional feature.

According to the current RRC signalling, the number of MIMO layers can be configured as 1, 2, 3, 4, 5, 6, 7, 8. However, the existing UE capability signalling only allows a UE to report to 2, 4, and 8-layer DL MIMO [4]. If agreed to be introduced, one potential solution would be to introduce/add a new candidate value, i.e. 6-layer.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 1 when SNR is within a higher range

Observation 2	Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 2 when SNR is within a higher range

Observation 3	Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 2 plus 32Tx when SNR is within a higher range.

Observation 4	With the TDLA30-10 low antenna correlation condition, the performance gain with 6 layers over 4 layers for QPSK, 16QAM and 64QAM is observed.

Observation 5	With TDLA30-10 medium correlation, the performance gain with 6 layers over 4 layers for low and medium MCS is observed.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Since MIMO 8 layers are already supported for FWA, it is proposed that MIMO 6 layers should also be supported, at least for FWA devices.

Proposal 2	The MIMO 6-layer is feasible for handheld UE and should be supported.

Proposal 3	The support of MIMO 6 layers could be considered an optional feature.

# References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- R4-2420376, WF on 6Rx requirements, AT&T

- R4-2405154, Draft BigCR on 38.101-4 for introduction of 8Rx performance requirements

- TS 38.331, V18.1.0