3GPP TSG-RAN WG4 Meeting #114									                        R4- 2501937

Athens, Greece, Feb. 17 – 21, 2025

Agenda item:			7.6.3

Source:	Ericsson

Title:	Discussion on the impact on UE RF requirements of fragmented carriers

Document for:	Approval

# Introduction

Fragmented carriers in the same band requiring multiple Rx chains may limit the UE CA capabilities if part of complex band combinations with many DL CCs combined with DL MIMO in fragmented operator spectrum holdings. A new RAN4 study item was approved in the RAN #104 meeting to investigate solutions to address this.

The objectives of the SI are as follows [1]:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

The following sections will discuss the impact on UE RF requirements of fragmented carriers.

# Discussions

## Tx leakage

The REFSENS requirements were defined based on the sub-block gap. The impact of the transmitter leakage and receiver IM2 were considered by defining the UL configuration for NCCA. The relaxation of REFSENS, ΔRIBNC for SCC was specified in TS 38.101-1. The requirements were defined based on the separate Rx chain architecture, the Tx leakage remains the same regardless of the type of Rx architecture. The factor that may affect the receiving REFSENS is the wider LPF with a worse suppression level. However, as pointed out in Error! Reference source not found., for some of the contiguous CA bands, e.g. n3 and n7, the maximum aggregated bandwidth is as wide as the max DL band. Therefore, the wider LPF can be feasible in the fragmented carrier operation as well. There may be a need to relax the PCC REFSENS. However, any relaxation must carefully account for the potential of DL coverage degradation.

- Any potential relaxation must carefully account for the degradation of DL coverage.

## PCell DL performance degradation

Consider n25 in Toronto as an example of an asymmetric fragmented spectrum. It is understandable that when using the single Rx chain architecture, the LPF needs to have a wider bandwidth to cover both CCs. The attenuation could become worse than that of a narrow bandwidth case. The Tx leakage on the DL carrier closer to the UL operating band could degrade the SNR, degrading the performance. This is nothing new, as the performance will be degraded using a single Rx chain.

Figure 1. An example band combination of n25 in Toronto.

Assume the network configures the intra-band combination CA_n25(2A) with two NC DL carriers and one single UL carrier next to the duplex gap. If the UE only supports this configuration with a single Rx chain, then the large relaxation of reference sensitivity would have a huge impact on the DL coverage. In the current spec, a relaxation of [24.6] dB REFSENS on SCC was introduced, as shown in Table 1.

Table 1 Power class 3 intra-band non-contiguous CA reference sensitivity with one uplink.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |

The analysis in [2] indicated that the UE receiver gain setting (implementation-specific) must be changed due to the sensitivity being degraded by 13.1 dB on PCC and 13.5 dB on SCC respectively, which would affect both the DL PCell and DL SCell. If these are the actual reduction of the UE OTA/conducted sensitivity, the DL PCell coverage would be significantly reduced when this band combination is configured. The PCell coverage may be lost even if the actual DL PCell coverage reduction is not as bad as a degradation of the RAN4 minimum requirement. The network must ensure a sufficient DL margin before configuring this band combination to maintain PCell coverage.

- NCCA band combinations with significant PCell REFSENS degradation are not feasible, as the PCell coverage may be lost upon configuration. The impact depends on the UE’s actual performance (i.e. the actual degradation is up to the specified MSD for the PCell).

- A band combination implemented by a single Rx chain with significant degradation on PCell REFSENS is not considered feasible.

The PCell REFSENS should also be maintained for NCCA regardless of the UE implementation. In case the fragmented carriers are in a TDD band or an FDD band with the PCell (Tx) in another band of a composite BC, then the impact on the ACS is less: the dynamic range requirements are alleviated and the impact on the wanted signal of cross modulation of the adjacent channel interferer is less (the Tx level more attenuated).

## In-gap requirements

#### Issue 3-4: Discussion on in-gap requirements [3]

RAN4 evaluate performance impact on the cases:

- image of in-gap interference overlaps with wanted carrier

- For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC

- image of in-gap blocking interference does not overlap with wanted carrier

The image originates from the frequencies that are symmetric to the desired signal frequencies around the LO center frequency. When the bandwidth of two CCs differs, they become asymmetric around the LO center frequency, as shown in Figure 2. In this scenario, part of the desired CC may overlap with the image of a strong interfering signal in the gap, which is a blocker in this case, as illustrated in Figure 2.

In practice, the image rejection ratio shall be better than the minimum SNR required to demodulate 256QAM in the DL as seen by the BB on each of the two carriers. The SNR at the receiver input must then be significantly higher than the BB SNR requirement to compensate for the impairments introduced by the receiver chain (filtering, mixing).

In the conformance test for NCCA, only the SCC is measured, but in 38.101-1, the performance of each carrier must be met.

Figure 2. The bandwidth of the wanted CC differs.

When the bandwidth of two CCs is equal and they are symmetric around the LO center frequency as shown in Figure 3, in this case, the receiver can readily applicable to this scenario without having to enhance the image rejection ratio since the image always comes from the other CCs whose power level within the 6 dB PSD difference.

Figure 3. The bandwidth of the wanted CC is equal.

- The image rejection shall be larger than the minimum SNR required to demodulate 256QAM on each DL CC, e.g. larger than (25 dB + max PSD imbalance between 2 CCs).

ACS and IBB are RF minimum performance requirements devised to ensure that the UE can operate in most scenarios intended for the CA feature without undue degradation. UEs also exceed the minimum requirements, but these do not ensure an interference-free operation under all circumstances. No specific action by the NW would be taken if there is a slight degradation of the ACS. However, in some cases, e.g. when the PCell actual coverage would be degraded due to the Rx gain setting as in the example in section 2.2, then the specific UE implementation of the NCCA band combination with a single Rx is not feasible.

- ACS and IBB are RF minimum performance requirements devised to ensure that the UE can operate in most scenarios in the field that are intended for the CA feature without undue degradation.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Any potential relaxation must carefully account for the degradation of DL coverage.

Observation 2	NCCA band combinations with significant PCell REFSENS degradation are not feasible, as the PCell coverage may be lost upon configuration. The impact depends on the UE’s actual performance (i.e., the actual degradation is up to the specified MSD for the PCell).

Observation 3	The image rejection shall be larger than the minimum SNR required to demodulate 256QAM on each DL CC, e.g. larger than (25 dB + max PSD imbalance between 2 CCs).

Observation 4	ACS and IBB are RF minimum performance requirements devised to ensure that the UE can operate in most scenarios in the field that are intended for the CA feature without undue degradation.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	A band combination implemented by a single Rx chain with significant degradation on PCell REFSENS is not considered feasible.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers

- R4-2418862, Discussion of impacts on UE RF requirements and DL performance impacts, MediaTek Inc.

- R4-2420337, WF on NR FR1 DL fragmented carriers study, MediaTek Inc.