# 3GPP TSG-RAN WG4 Meeting # 114                                                          R4-2501955

# Athens, Greece, 17 – 21 February 2025

# Title:	1UL MSD simplifications rules for Rel-19

# Agenda Item:	7.1.1.1.1

# Source:	Skyworks Solutions Inc., Nokia.

# Document for:	Approval

# Introduction

The discussion paper presents our views on the 1UL MSD simplification options captured in the agreed Way Forward (WF) [1].

# Discussion

## 1UL MSD simplification WF agreements

At RAN4 meeting #113, the following agreements were reached on simplifying the 1UL MSD requirements [1].

Agreement: For new Rel-19 inter-band CA combinations within FR1, and for MSD due to harmonic, receiver harmonic mixing and cross-band isolation interference, the 1UL HPUE MSD requirements are specified using a delta MSD relative to the default power class 3 MSD as:

PCx MSD = PC31Tx MSD + PCx, where x = 2 for PC2 or x=1.5 for PC1.5,

  - Companies are invited to evaluate two options,

    - Option 1: RAN4 keeps on specifying two sets PC2 requirements [3,4]:

      - one LUT/equation for PC2 1Tx: PC21Tx MSD = PC31Tx MSD + PC21Tx, e.g. plateau at ~3dB,

      - one LUT/equation for PC2 2Tx: PC22Tx MSD = PC31Tx MSD + PC22Tx, e.g. plateau at ~8dB,

    - Option 2: RAN4 specifies a single set PC2 requirement [2]:

      - one PC2 LUT/equation PC2 MSD = PC31Tx MSD + PC2, e.g. plateau at ~6dB.

- To ensure that 1UL PC1.5 MSD is greater than 1UL PC2 2Tx MSD, companies are invited to derive the HPUE delta MSD LUT/Equations values assuming:

    - a [9dB] 1Tx Main/Div interference imbalance. Imbalance assumptions less than 9dB are not precluded,

    - [0.5]dB RevIM (Rtx) and 3dB interferer increase for harmonic/harmonic mixing and for crossband isolation.

- Handling of band combination without MSD for PC3 but with small or moderate PC2/PC1.5 MSD:

  - Option 1: FFS LUT rule and FFS on test point configuration

  - Option 2: Legacy HPUE MSD tables may still be used to capture an exception MSD test point

  - Option 3: no HPUE MSD requirement for those cases.

- For proponents of LUT simplification approach, FFS the LUT granularity (e.g. 1dB steps)

## 1UL MSD simplification proposal

Our views are captured for each item as follows:

- On the number of PC2 MSD requirements:

  - Option 1: We believe that option 2 would be unfair to 1Tx PC2 implementations that inherently offer better performance than 2Tx PC2 implementations.

- On ensuring that 1UL PC1.5 MSD is greater than the 1UL PC2 2Tx MSD [4]:

  - Even if 9dB 1Tx Main/Div interference imbalance represents a realistic worst-case and most often used assumption by RAN4 proponents, it was shown that adopting this assumption leads to a high PC1.5 plateau of ~ 10.8dB (without Rev-IMD) [4,5]. Due to concern that such a high MSD de-rating may penalize PC1.5, we prefer to compromise by assuming a 6dB 1Tx Main/Div interference imbalance,

  - Regarding other assumptions:

    - For rev-IMD, we assume a 0.5dB contribution based on our previous 2Tx power amplifier measurements,

    - For PC2 interference level increase: we assume that PC2 1Tx interference is 3dB higher than the PC3 1Tx for interference due to uplink harmonics, receiver harmonic mixing and cross-band isolation based on our previous measurements [6],

- On handling band combinations which are not affected at PC3 but may be affected by small or moderate PC2/PC1.5 MSD:

  - Our preference is for Option 2 justified by the fact that:

    - these band combinations remain exceptions, and that

    - without a PC3 MSD test point, the concept of PCx look-up table (LUT) cannot be applied.

- On the LUT format, we propose a format similar to that of [7]. Note that other formats may be considered, for example RAN4 may also consider the slightly more compact “equation-based” LUT format of [4], or single linear expression clamped with a plateau value as illustrated in the Annex 1 and Annex 2.

Based on these assumptions, Figure 1 plots as a function of the agreed PC3 MSD level,

  - PC2 1Tx in green, with a plateau reaching ~3dB;

  - PC2 2Tx in blue, with a plateau reaching ~6dB;

  - PC 1.5 in orange, with a plateau reaching ~9dB.

Figure 1:  Proposed PC2 1Tx (green), PC2 2Tx (blue) and PC 1.5 (orange) with 0.5dB rev-IMD and 6dB 1Tx Main/Div interference imbalance.

Based on the results of Figure 1, we make the following 1UL MSD simplification proposals.

Proposal 1: Handling of the 1UL MSD requirements in Rel-19 TS 38.101-1

- The legacy Rel-18 MSD test points are ported to Rel-19, in their current MSD table format, and are not modified for all power classes, and all types of 1UL and 2UL MSDs.

- In Rel-19, and until the 1UL MSD simplification LUT is finalized: RAN4 keeps on specifying the 1UL HPUE MSD requirements according to the legacy agreements from Rel-17 and Rel-18, i.e.:

  - For 1UL PC2 TDD band: RAN4 follows the legacy of specifying only one PC2 MSD requirement. This corresponds to PC2 achieved with 1Tx,

  - For 1UL PC2 FDD band: RAN4 follows the legacy of specifying two PC2 MSD requirements: one for PC2 achieved with 1Tx and one for PC2 achieved with 2Tx.

  - For 1UL PC2 SUL band: there is no legacy agreement since these MSD requirements were never discussed and never specified in Releases prior to Rel-19 [8]. RAN4 to treat SUL PC2 in a generic manner, i.e. RAN4 to specify two PC2 MSD requirements: one for PC2 1Tx and one for PC2 2Tx.

  - For 1UL PC1.5: One set of MSD requirements which, by definition is specified for 2Tx.

- Once the 1UL HPUE simplification LUT is agreed, and for new Rel-19 inter-band CA combinations:

  - The default power class 1UL MSD is specified, using the legacy MSD tables.

  - The 1UL HPUE MSD requirements are specified. However, the MSD requirements are no longer captured in the legacy HPUE MSD tables. The 1UL HPUE MSDs are specified using PC3 to HPUE LUTs.

  - The 1UL HPUE LUTs are captured in TS 38.101-1, Clause 7.3A.2.3, Reference sensitivity power level for Inter-band CA. The following eligibility criteria should be captured in this clause’s core requirements:

The 1UL HPUE MSD LUT simplification applies to Rel-19 CA band combinations, for which:

  - A valid 1UL HPUE configuration is specified in the two-band inter-band CA configuration tables (Table 5.5A.3.1-1a ~ 5.5A.3.1-1n), and,

  - A default power class 3 (1Tx) 1UL MSD test point is specified, and,

  - A 1UL HPUE MSD test point is not already specified in either Table 7.3A.4-2a, Table 7.3A.4-4a-1, Table 7.3A.4-4b, Table 7.3A.6-1a-1, or Table 7.3A.6-1b.

Proposal 2: Handling of the Rel-19 TPs for TR

- The procedure is unchanged for the default power class: the 1UL MSD must be studied, and, where applicable, the default PC MSD is captured in the TP and specified in the TS,

- The 1UL HPUE MSD requirements are no longer studied. The 1UL HPUE MSD requirements may be captured by simply referring to the TS 38.101-1 LUTs.

Proposal 3: For new Rel-19 inter-band CA combinations within FR1, the 1UL HPUE MSD requirements due to uplink harmonic, harmonic mixing, and cross-band isolation interference, are specified as:

- PC21Tx MSD = PC31Tx MSD + PC21Tx,

- PC22Tx MSD = PC31Tx MSD + PC22Tx,

- PC1.5 MSD = PC31Tx MSD + PC1.5,

and where PC21Tx, PC22Tx and PC1.5 are specified with the LUT of Table 1, and where the PC3 MSD entry shall be rounded to the closest integer.

Example:

- If PC3 MSD <=2.4dB, then then LUT PC3 entry = 2.0dB

- If PC3 MSD >=2.5dB, then then LUT PC3 entry = 3.0dB.

Table 1: 1UL PC21Tx , PC22Tx , PC1.5 LUT.

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| <1 | 0.6 | 0.9 | 2.0 |
| 1 | 0.8 | 1.2 | 2.7 |
| 2 | 1.2 | 2.1 | 4.1 |
| 3 | 1.6 | 2.7 | 5.1 |
| 4 | 1.8 | 3.3 | 5.9 |
| 5≤ <7 | 2.0 | 4.0 | 7.0 |
| 7≤ ≤9 | 2.5 | 5.0 | 8.0 |
| ≥10 | 3.0 | 6.0 | 9.0 |

Other formats are not precluded, such as “equation-based” LUT of [4] or in the form of a linear expression + plateau, as illustrated in Annex 1 and Annex 2.

Proposal 4: Handling of the Rel-19 NRCA band combinations which have no PC3 MSD but may have small PC2 or PC1.5 MSD:

- TPs for TR or draft CRs for PC3 which do not propose an MSD for PC3 shall be flagged IF a company/delegate believes a HPUE MSD is needed, otherwise HPUE MSD is not specified.

- For the configurations where PC3 MSD is not specified, the 1UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b.

Finally, it may be possible to reduce the conformance test burden for HPUE band combinations by verifying only the UE performance of the highest supported power class. Yet, since the interference coupling paths are different for 1Tx and 2Tx implementations, it is necessary to verify the performance of the highest supported power class of each architecture: 1Tx and 2Tx. By doing so, it is possible to save the conformance test time of the default PC3 MSD test points.

Proposal 5: To reduce the conformance test burden of HPUE band combinations, send an LS to RAN5 to recommend verifying only the MSD requirements of the highest supported 1Tx power class and the highest supported 2Tx power class of a given band combination.

# Conclusion

This paper captures our views on simplifying the 1UL MSD requirements for Rel-19 HPUE NRCA inter-band combinations. The main benefits to our proposals are:

- For new Rel-19 CA combinations, RAN4 no longer needs to study the 1UL HPUE MSD requirements. Only the MSD of the default power class 3 needs to be evaluated. This will help reduce the RAN4 workload.

- Errors and inconsistencies observed in [5] are “automatically” removed:

  - For all MSD types: Harmonic interference, Cross-band isolation, or Rx harmonic mixing.

  - For all 1UL HPUE power classes: PC2 and PC1.5.

  - For all front-end architectures: 1Tx and 2Tx.

Proposal 1: Handling of the 1UL MSD requirements in Rel-19 TS 38.101-1

- The legacy Rel-18 MSD test points are ported to Rel-19, in their current MSD table format, and are not modified for all power classes, and all types of 1UL and 2UL MSDs.

- In Rel-19, and until the 1UL MSD simplification LUT is finalized: RAN4 keeps on specifying the 1UL HPUE MSD requirements according to the legacy agreements from Rel-17 and Rel-18, i.e.:

  - For 1UL PC2 TDD band: RAN4 follows the legacy of specifying only one PC2 MSD requirement. This corresponds to PC2 achieved with 1Tx,

  - For 1UL PC2 FDD band: RAN4 follows the legacy of specifying two PC2 MSD requirements: one for PC2 achieved with 1Tx and one for PC2 achieved with 2Tx.

  - For 1UL PC2 SUL band: there is no legacy agreement since these MSD requirements were never discussed and never specified in Releases prior to Rel-19 [8]. RAN4 to treat SUL PC2 in a generic manner, i.e. RAN4 to specify two PC2 MSD requirements: one for PC2 1Tx and one for PC2 2Tx.

  - For 1UL PC1.5: One set of MSD requirements which by definition is specified for 2Tx.

- Once the 1UL HPUE simplification LUT is agreed, and for new Rel-19 inter-band CA combinations:

  - The default power class 1UL MSD is specified, using the legacy MSD tables.

  - The 1UL HPUE MSD requirements are specified. However, the MSD requirements are no longer captured in the legacy HPUE MSD tables. The 1UL HPUE MSDs are specified using PC3 to HPUE LUTs.

  - The 1UL HPUE LUTs are captured in TS 38.101-1, Clause 7.3A.2.3, Reference sensitivity power level for Inter-band CA. The following eligibility criteria should be captured in this clause’s core requirements:

The 1UL HPUE MSD LUT simplification applies to Rel-19 CA band combinations, for which:

  - A valid 1UL HPUE configuration is specified in the two-band inter-band CA configuration tables (Table 5.5A.3.1-1a ~ 5.5A.3.1-1n), and,

  - A default power class 3 (1Tx) 1UL MSD test point is specified, and,

  - A 1UL HPUE MSD test point is not already specified in either Table 7.3A.4-2a, Table 7.3A.4-4a-1, Table 7.3A.4-4b, Table 7.3A.6-1a-1, or Table 7.3A.6-1b.

Proposal 2: Handling of the Rel-19 TPs for TR

- The procedure is unchanged for the default power class: the 1UL MSD must be studied, and, where applicable, the default PC MSD is captured in the TP and specified in the TS,

- The 1UL HPUE MSD requirements are no longer studied. The 1UL HPUE MSD requirements may be captured by simply referring to the TS 38.101-1 LUTs.

Proposal 3: For new Rel-19 inter-band CA combinations within FR1, the 1UL HPUE MSD requirements due to uplink harmonic, harmonic mixing, and cross-band isolation interference, are specified as:

  - PC21Tx MSD = PC31Tx MSD + PC21Tx,

  - PC22Tx MSD = PC31Tx MSD + PC22Tx,

  - PC1.5 MSD = PC31Tx MSD + PC1.5,

and where PC21Tx, PC22Tx and PC1.5 are specified with the LUT format of Table 1, and where the PC3 MSD entry shall be rounded to the closest integer.

Example:

- If PC3 MSD <=2.4dB, then then LUT PC3 entry = 2.0dB

- If PC3 MSD >=2.5dB, then then LUT PC3 entry = 3.0dB.

Table 1: 1UL PC21Tx , PC22Tx , PC1.5 LUT.

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| <1 | 0.6 | 0.9 | 2.0 |
| 1 | 0.8 | 1.2 | 2.7 |
| 2 | 1.2 | 2.1 | 4.1 |
| 3 | 1.6 | 2.7 | 5.1 |
| 4 | 1.8 | 3.3 | 5.9 |
| 5≤ <7 | 2.0 | 4.0 | 7.0 |
| 7≤ ≤9 | 2.5 | 5.0 | 8.0 |
| ≥10 | 3.0 | 6.0 | 9.0 |

Other formats are not precluded, such as “equation-based” LUT of [4] or in the form of a linear expression + plateau, as illustrated in Annex 1 and Annex 2.

Proposal 4: Handling of the Rel-19 NRCA band combinations which have no PC3 MSD but may have small PC2 or PC1.5 MSD:

- TPs for TR or draft CRs for PC3 which do not propose an MSD for PC3 shall be flagged IF a company/delegate believes a HPUE MSD is needed, otherwise HPUE MSD is not specified.

- For the configurations where PC3 MSD is not specified, the 1UL HPUE MSD requirements are specified in the legacy MSD tables 7.3A.4-2a, 7.3A.4-4a-1, 7.3A.4-4b, 7.3A.6-1a-1, 7.3A.6-1b.

Finally, it may be possible to reduce the conformance test burden for HPUE band combinations by verifying only the UE performance of the highest supported power class.

Proposal 5: To reduce the conformance test burden of HPUE band combinations, send an LS to RAN5 to recommend verifying only the MSD requirements of the highest supported 1Tx power class and the highest supported 2Tx power class of a given band combination.

# References

- R4-2420380 WF on MSD rule, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc, Murata Corporation Ltd., Nokia, Qualcomm France.

- R4-2419082 MSD for HPUE, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Qualcomm France.

- R4-2417784 Further discussion on HPUE MSD rules, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Murata Corporation.

- R4-2419699 1UL HPUE NRCA MSD Simplifications for Rel-19, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

- R4-2419741 1UL HPUE NRCA MSD Observations, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

- R4-2413068 Update on Harmonic MSD simplification, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, The Netherlands, Skyworks Solutions, Inc.

- R4-2407156 Guidelines for FDD PC2 Harmonic MSD Analysis, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, Skyworks Solutions, Inc.

- R4-2419721 Impact of SUL HPUE on MSD, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

# Annex 1 – “Equation-based” LUT for 1UL MSD simplification

5.1 Alternative LUT for PC21Tx:  Maximum deviation from calculated = -0.2 / +0.3dB

| Agreed PC3 MSD (dB) | PC21Tx (dB) |
| --- | --- |
| PC3MSD < 1.0 | 0.6 |
| PC3MSD ≥ 1.0 | min(3, ceil(0.21*PC3MSD + 0.7,0.1)) |

Figure 1: PC21Tx “Equation-based” LUT (dots), calculated PC21Tx (plain line), PC21Tx accuracy (dotted line)

5.2 Alternative LUT for PC22Tx: Maximum deviation from calculated = -0.3 / +0.3dB

| Agreed PC3 MSD (dB) | PC22Tx (dB) |
| --- | --- |
| PC3MSD < 1.0 | 0.9 |
| 1.0 ≤ PC3MSD ≤ 4.0 | min(6, ceil(0.85*PC3MSD + 0.3,0.1)) |
| PC3MSD > 4.0 | min(6, ceil(0.22*PC3MSD + 2.7,0.1)) |

Figure 2: PC22Tx “Equation-based” LUT (dots), calculated PC22Tx (plain line), PC22Tx accuracy (dotted line)

5.3 Alternative LUT for PC1.5 : Maximum deviation from calculated = -0.3 / +0.4dB

| Agreed PC3 MSD (dB) | PC1.5 (dB) |
| --- | --- |
| PC3MSD < 1.0 | 2.0 |
| 1.0 ≤ PC3MSD ≤ 4.0 | min(9, ceil(1.3*PC3MSD + 1.3,0.1)) |
| PC3MSD > 4.0 | min(9, ceil(0.32*PC3MSD + 4.7,0.1)) |

Figure 3: PC1.5 “Equation-based” LUT (dots), calculated PC1.5 (plain line), PC1.5 accuracy (dotted line)

# Annex 2 – Single expression for 1UL PCx , where x=2 and 1.5.

- PC21Tx:

  - for PC3MSD <1.0, PC21Tx =0.6dB,

  - for PC3MSD ≥1.0, PC21Tx = min(3, 0.22*PC3MSD + 0.77)

  - Maximum deviation from calculated: -0.1/+0.4dB

Figure 5: PC21Tx (black line), calculated PC2 1Tx (green line), PC21Tx accuracy (dotted line)

- PC22Tx:

  - for PC3MSD <1.0, PC21Tx =0.9dB,

  - for PC3MSD ≥1.0, PC21Tx = min(6, 0.6*PC3MSD + 0.6)

  - Maximum deviation from calculated: -0.3/+1.0dB

Figure 6: PC22Tx (black line), calculated PC22Tx (blue line), PC22Tx accuracy (dotted line)

- PC1.5:

  - for PC3MSD <1.0, PC21Tx =0.9dB,

  - for PC3MSD ≥1.0, PC21Tx = min(9, 0.78*PC3MSD + 1.91)

  - Maximum deviation from calculated: -0.9/+1.0dB

Figure 7: PC1.5x (black line), calculated PC1.5 (blue line), PC1.5 accuracy (dotted line)