3GPP TSG-RAN WG4 Meeting #114	R4-2501959

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	Testing framework and procedure for one-sided models

Agenda item:	7.20.2

Document for:	Discussion

Introduction

In RAN4#106, a new SI ‎[1] “New SID for NR_AIML_air_Phase2” was agreed to continue the study on the remaining issues of AI/ML air interface phase1. In this new SI, following scope was agreed for RAN4:

- Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least:

  - Relation to legacy requirements

  - Performance monitoring and LCM aspects considering use-case specifics

  - Generalization aspects

  - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.)

  - UE processing capability and limitations

Post-deployment validation due to model change/drift

In this paper, we discuss testability and interoperability aspects for all one-sided models uses cases:

- AI/ML Beam Management

- AI/ML based CSI Prediction

- AI/ML based Positioning enhancements

Discussion

AI/ML Beam Management

The discussions on Release 19 AI/ML enabled beam management (BM) use case were continued in RAN4#113 meeting and the agreements achieved during this meeting are summarized in the WF ‎[2]. However, no agreement was made on the testing setup for beam prediction testing topic, as the companies were of different opinion regarding the suitability of single AoA and multiple AoAs based test setups. Hence, companies were invited to bring further analysis on the test setups with single AoA and multiple AoAs.

Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs.

In this paper, we provide our further views on test setups with single and multiple AoAs in the context of AI/ML BM testing, as listed below:

- Testing setup with single and multiple AoAs,

- Generalization and test coverage related aspects.

Testing setup with single and multiple AoAs

      - Test setup

- It is well known that the ground truth is important for the conformance testing of AI/ML BM feature and depends on the UE architecture and implementation. RAN4 in 112-bis meeting has agreed that both the Set A and Set B beams should be generated in the chamber for ground-truth extraction. Set B beams need to be emulated inside the OTA test chamber during the conformance testing to provide the L1-RSRP to the DUT to perform the test inference during the inference phase. Set A beams need to be emulated inside the OTA test chamber during/before/after the conformance testing only to extract the ground truth.

- In RAN4#113 meeting, companies have provided their views on the suitability of single AoA based test setup and multiple AoAs based test setup for the conformance testing of AI/ML based BM feature. Here, we provide our views on this topic and highlight the trade-off between these two types of test setup.

- As the signal(s) from the test probe(s) inside the OTA test chamber are received by the UE antenna (panel) directly without any change in AoA, the number of available test probes correspond to the number of different AoAs from which the UE can receive the signals during the conformance testing. Or it is possible for a few probes (probe array) can create one or a few different AoAs, the number of AoA is not the same as the number of test probes.

Ideally, all the beams in Set A and/or Set B should be generated without UE rotation as the similar scenario will be observed in the real deployments. However, the number of Set A and/or Set B beams to be emulated during the test is typically larger than the number of beams that can be generated by the TE. Based on the TE vendors’ input during the RAN4#112-bis meeting, all Set A and/or Set B beams can’t be generated during the conformance testing without using UE rotation as only a limited number of Tx-beam RX-PAS profiles can be emulated with a given number of test probes.

During the conformance test, the TE should split the Set A/B beams by certain amount of channel instances/snapshots. Each of the channel instance/snapshot should contain a certain number of Set A/B beams. As this feature targets FR2 channels which are sparse in nature and does not have as many as delay bin/taps as those in FR1 channels. Furthermore, the number of delay bin/taps of FR2 channels is also determined by the exact FR2 frequency of communication, Tx/Rx beam width, channel scenarios, etc. Therefore, how many beams should be generated in each test instance/interval/iteration in the test chamber is a more relevant topic to the design of the test procedure and the ability of the TE.

Clarify the questions for the test setup with UE rotations: (1) How many Set A and/or Set B beams can be generated inside the test chamber at the same time? (2) How many test instance/iteration/intervals should be defined based on total number of Set A/B beams?

As the TE vendors clarified that only a limited number of beams with different AoAs can be emulated inside the test chamber, it means that multiple Set A and/or Set B beams will arrive at the DUT from the same/similar direction(s). Hence, a natural question to be asked is that whether it will be possible for a UE to distinguish between different beams which may have different Beam IDs but will arrive at the DUT from the same/similar AoA.

UE vendors to clarify if the Set A and/or Set B beams emulated inside the test chamber can’t arrive at the DUT from different AoAs, will it be possible for the DUT to distinguish between the different beams which may have different Beam IDs but will arrive at the DUT from the same AoA.

Furthermore, since in FR2 OTA testing, it is not feasible to get ideal RSRP measurements to validate predicted RSRP values. Apart from the non-idealities of the RF components, these uncertainties arise due to spherical coverage, UE gain variations and the allowed tolerance margin in the UE accuracy. The uncertainty factors depend on UE implementations, such as UE Rx beam codebook design, the RX beam used and the RX beam gain, etc. In this context, NW/TE do not have any direct means to evaluate how accurate are the predictions especially if they are compared only with the UE reported measurements.

To obtain more accurate ground truth for the predicted RSRP, one potential alternative may be that Network/TE receives the approximated ground truth via direct measurements in UL. These UL measurements can be used as ground truth for downlink measurements if the beam correspondence is achieved. More specifically, the uncertainty introduced by the UE implementation (e.g., for UE measurements) are taken out when the same beams, in DL, are used for DL TX (at TE/BS) and RX (at UE) are also used in UL for TX (at UE) and RX (at TE/BS). Moreover, the accuracy of ground truth evaluation can be increased even further if the same channel used during the DL measurements are reapplied during UL transmission.

Reported RSRP values as approximated ground truth are error-prone and have several uncertainties.

To obtain a more accurate ground truth, RAN4 should consider the case where UL measurements received at TE/NW can be used as ground truth for DL measurements in case when Beam correspondence is ensured between UL and DL beams.

      - Single AoA and multiple AoA

In the RAN4 discussions related to the use of channel model during the conformance testing, the views of different companies were quite diverse where some companies were supporting the use of AWGN/TDL propagation condition based simpler testing solution while the others were in favor of CDL channel model-based conformance testing. The reasons for preferring the AWGN/TDL channel based testing are: (1) less complexity relative to CDL model based test, (2) questions on the feasibility of CDL based test due to the limitations of the test system to emulate the total number of taps in the CDL model, (3) possibility of using AWGN/TDL model with UE rotation to emulate the different AoAs, (4) using the different test probes with different transmit power to emulate the different L1-RSRP values, (5) less attractiveness of scaling MIMO OTA test systems in terms of time, cost and very limited use, etc.

Therefore, we next propose a simple solution which combines the AWGN/TDL based test systems and UE orientation to perform the conformance testing of AI/ML based BM use case to enable the beam sweeping of Set A and/or Set B beams in an iterative manner, by using a combination of the available test probes and different UE orientations/rotations, during the RAN4 conformance testing using the OTA test chamber where single or multiple test probe(s) are available. One benefit of rotating the UE is that it can enable the test system to simulate the AoA angles outside the angular range of test probes. Generally, this enables test system can generate any possible AoA beam angles, for example, the test set up for one AoA can use TDL model, by rotating the UE, all the angles of AoAs can be generated.

The proposed solution assumes: (1) OTA test chamber is equipped with single or multiple test probe(s), (2) AWGN channel model to be used during the test, (3) One beam is generated by using only one of the available test probe(s) during the conformance test, (4) or one beam is generated by using a probe array for any possible AoA angles.

The solution proposes the DUT to perform beam sweeping in multiple iterations and to measure the RSRP of the corresponding beams in Set A and/or Set B during the conformance test for which a new test command is to be defined and to be sent to the DUT, and we assume the 3D coordination origin to define the AoA angles is in the center of the UE positioner inside Chamber. After the DUT receives this test command from the TE and configures itself to perform beam sweeping in multiple iterations, one of the possible/allowed UE orientations is selected and TE selects the power level (typically different) for each of the available test probes. It is worth mentioning that in this solution, only one test probe is active at any given point in time and represents one Tx beam. This implies that with ‘P’ test probes in the OTA testing (anechoic) chamber, using one probe at a time will implicitly mean emulating ‘P’ different Tx beams (one at a time) for a given UE orientation. Thereafter, if more than ‘P’ Tx beams are required, TE can change the UE orientation but uses the same set of available test probes (with the Tx power of each test probe to be held constant as previously selected or to be set differently) to emulate another set of ‘P’ Tx beams. In this fashion, multiple UE orientations can be selected depending on the number of Tx beams required, each AoA angle of them. The above solution is valid for emulating all Set A and/or Set B beams in an iterative manner to perform beam sweeping during the conformance testing.

Figure 2: An example of the proposed AWGN based test system with UE rotation.

Given the Set A and/or Set B beams can be generated iteratively during the conformance test as proposed above, DUT can measure the L1-RSRP of each beam in Set B and provides it as input to its AI/ML model for Set A beam prediction.

A combination of AWGN/TDL based test system and UE rotation can be a potential candidate for the conformance testing of AI/ML based BM use case.

RAN4 to discuss and explore the feasibility of performing beam sweeping in multiple iterations during the conformance testing of AI/ML based BM use case.

Generalization and test coverage related aspects

In the RAN4#110 and RAN4#111 meetings, it was agreed to discuss generalization aspects for each AI feature in case-by-case manner. The overfitting of UE functionalities to the test environment was also raised by some companies. In our view, testing of generalization aspect is one of the main challenges for RAN4 testing. RAN4 may define multiple tests with different conditions.

In each of the test, TE configures the same specified UE configuration for AI/ML BM, the same UE configuration is tested under different scenarios to verify the UE’s model capability to generalize. In the tables below we attempt to capture different parameters that may be needed to define different generalization scenarios for AI/ML enabled BM use case based on set of parameters/configurations to both DUT / TE. For identified scenarios in AI/ML enabled BM-Case1 and BM-Case2, the minimum level of performance may need to be defined. Similarly acceptable tolerance margin for other scenarios may also need to be agreed, potentially based on simulation results.

Table 1: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR (Deployment Scenario) | Good / Bad Radio conditions |
| Propagation Model | TDL/CDL |

For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in Table 1.

In order to address the testability issue of generalization aspects of AI/ML BM use case, we propose a new metric to indicate the generalization capabilities of an AI/ML functionality, which may be referred as “generalization score”. This may be used to verify/check whether the BM functionality generalize well or not.

As an example, if there are 3 scenarios of interest, any one scenario can be the baseline setup and the remaining 2 scenarios will then be used as other set ups to test the generalization aspects. The generalization score can be calculated as a function of individual scenario verdicts, e.g., as simple average or weighted average of the individual generalization verdict of each Scenario ‘j’ (i.e., Scenario ‘1’ to Scenario ‘N), where the verdict of Scenario ‘j’ = 1 if the functionality generalizes to Scenario ‘j’ else it is equal to 0.

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML BM functionality, to verify the generalization performance of the functionality in different scenarios.

AI/ML based CSI Prediction

Tests for AIML Model Switching in Non-static Scenarios

The definition of the static scenarios is given in RAN4#110bis meeting as follows.

- Agreement: - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenarios/configurations as baseline. - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

Also, in RAN4#113, the agreement is outlined to use static conditions as the baseline for evaluation.

- Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

The definition of the static scenarios was clarified already at RAN4#110bis, and RAN4#113 made an agreement that static conditions are used as a baseline. Based on the agreed definition, the static condition is the case where the channel model and SNR settings are fixed and do not change over the test while specific channel realizations may be dynamic. It is also agreed that the non-static condition can be described based on the application in the different use cases, and non-static models may include non-stationary SNR and other conditions.

In the case of CSI prediction, a refined definition of non-static condition will need to describe a highly dynamic environment. For instance, the UE mobility, e.g., speed, can be considered with different deployment scenario, frequencies, UE density, and distributions. The non-static condition needs to enable a testing equipment to generate a random environment where a wireless channel is dynamically fluctuating.

When defining non-static scenarios, we should aim to test the cases where the UE is allowed to internally switch models without explicit signaling to the NW while maintaining the same functionality. For CSI prediction, UE will switch from one AI/ML model to another when radio conditions change (e.g., transitioning from an indoor to an outdoor channel model, variations in UE speed affecting Doppler, and/or changes in delay spread). While such switching might briefly interrupt inference, these transitions should not significantly impact functionality performance. Therefore, RAN4 requirements have to specify that the impact on performance metrics (e.g., relative PDSCH throughput) should not exceed any acceptable tolerance margins. Thus, if the non-static scenario is defined, RAN4 must verify that there is no performance degradation while UE is autonomously switching in between different AI/ML models due to the change of radio conditions.

When radio conditions change, UE will switch from one AI/ML model to another, and proper operation in non-static conditions needs to be ensured throughout testing.

RAN4 should define requirements/test procedures to ensure that no performance degradation occurs due to internal model switching under non-static conditions.

Tests for Generalization of Single AIML Model (No Model Switching)

When an AIML model trained for a certain static or non-static condition, no model switching may be required for the UE. In this case, the generalization should be tested to ensure that the local model of the UE is properly handling the changing conditions, and the performance impact is minimized.

When UE may have a well-generalized AIML model, no switching needs to be done for UE, and testing is needed to ensure that no performance degradation occurs.

In RAN4 requirement, the issue of overfitting UE models to the test environment needs to be considered. To address the overfitting problem, it is required to define and evaluate the generalization testes of a functionality. In the RAN4#110 meeting, the following agreements on Generalization were noted.

- Issue 1-1: Generalization update - Agreement: - For AI/ML generalization [tests/requirements] - RAN4 should discuss it and decide the requirements/tests for each AI feature in the case-by-case manner

Additionally, Agreement from RAN4#113 captures the channel model for the generalization test in CSI prediction.

- Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test.

For generalization testing, it was agreed to use TDL channel models for CSI prediction while using CDL for generalization test remains as an FFS. CDL models can be preferred for testing over TDL models due to the feature to more accurately represent complex, real-world environments. CDL models can capture spatial characteristics and multipath clustering. Therefore, CDL model can be used to describe environments with significant scattering and reflection, such as urban areas. CDL models can also simulate LoS/NLoS conditions. Considering spatial characteristics, CDL models can be also considered to model a more complex environment. Therefore, RAN4 may discuss whether CDL models can be used for generalization testing to ensure that practical scenarios can be simulated in a testing equipment.

CDL models can be used for generalization testing to ensure that practical scenarios can be simulated in a testing equipment.

The table below summarizes the parameters agreed upon for use in RAN1, which can be employed to verify various generalization scenarios.

Table 1: Parameters for Generalization Scenarios

| - Parameters | - Description |
| --- | --- |
| - UE Speed | - Slow / Medium / Fast |
| - SINR (Deployment Scenario) | - Good / Bad Radio conditions |
| - Carrier Frequency | - 2GHz / 3.5GHz |
| - Propagation Model | - Change of TDL/[CDL] model type, e.g., TDL-A, TDL-B, etc. |

When considering TDL model, prediction becomes challenges if TDL has a low correlation is considered. This is due to the fact that the spatial dimensions of the channel evolve “randomly”, or at least non-predictably. In TDL models, the speed-based Jakes maximum Doppler distribution is modeled, and no real velocity-based or physical model-based channel evolution is presented. In that case, the AI/ML-based CSI prediction can result in a random outcome due to the low correlation of TDL models.

Also, if TDL with medium or high correlation is used, the channel has a single spatial dimension (overlayed by small scale fading) that does not change during the test. Therefore, during the testing, the AI/ML model does not have much to learn and predict either. In that case if the spatial behavior of TDL models is unrealistic, the AI/ML-based CSI prediction can have only trivial outcomes. For instance, spatial characteristics of the channel could always be the same for medium or high correlation.

Therefore, low correlation in TDL models leads to unpredictable AI/ML-based CSI prediction, whereas medium or high correlation results in trivial predictions due to constant spatial dimensions. Considering the relationship between correlation and outcome of AIML, the TDL model can be used. In particular, the parameters specified in Clause 7.7 of TR 38.901 need to be selected by RAN4 to specify the testing environment. This includes selecting the delay spread, which determines the applicable delay profile configuration properties for the channel. Thus, RAN4 should study the expected outcomes of the AI/ML model when the TDL channel model is used.

RAN4 to define testing procedures/environment to ensure no performance degradation when running AIML model with TDL channel models for generalization testing.

Additionally, a new metric needs to be defined to ensure that the functionality is generalized. It is challenging to measure the degree of generalization when the functionality is running across different scenarios and conditions. Therefore, it first needs to identify and align the scenarios and conditions to define the KPIs for generalization.

We propose a new metric to indicate the generalization capabilities of an AI/ML functionality, that may be referred as “generalization score.” In case of CSI feedback enhancement use case, this may be used to verify/check whether the CSI feedback enhancement functionality (which may be developed for a specific scenario) generalize well to other scenario(s).

For example, if there are three scenarios, any of the three scenarios will be used as the baseline setup (depending on functionality implementation) and the remaining 2 scenarios will be used as the setting to test the generalization aspects. Then, an ‘overall generalization score’ can be calculated as a function of individual scenario’s verdict, e.g., a simple average or weighted average of the individual generalization verdict of each Scenario j (i.e., from Scenario 1 to Scenario N) where the verdict of Scenario j = 1 if the functionality generalizes to Scenario j; otherwise, the verdict becomes 0.

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML CSI prediction functionality, to verify the generalization performance of the functionality in different scenarios

AI/ML based Positioning enhancements

RAN4 agreements on testability and interoperability

During the Release-18 study on artificial intelligence (AI)/machine learning (ML) for NR air interface (FS_NR_AIML_Air) ‎[3], as mentioned in the extract from 3GPP TR 38.843 V18.0.0 ‎[4] below, following sub use cases for positioning accuracy enhancement were considered:

The following are selected as representative sub-use cases: - Direct AI/ML positioning: - AI/ML model output: UE location - e.g., fingerprinting based on channel observation as the input of AI/ML model - AI/ML assisted positioning: - AI/ML model output: new measurement and/or enhancement of existing measurement - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement More specifically, the following Cases are considered for the study: - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning One-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

During the SI, RAN4 studied different test metrics for performance requirements for AI/ML enabled positioning use case.  Here is an extract from the TR ‎[4] covering these aspects:

Both direct AI/ML positioning and AI/ML assisted positioning are considered. For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above The feasibility and testability of different options should be further justified in WI.

In RAN4#110, the following has been agreed for the Rel-19 WI on AI/ML positioning ‎[5] on the testability and interoperability issue ‎[6].

2.3 Testability and interoperability issues for positioning accuracy enhancement 2.3.1 Agreements in ad-hoc session (R4-240xxxx) Issue 3-2: Requirements for case 3a/3b RAN4 will not define positioning accuracy requirements for case 3a/3b Issue 3-6: Requirements for case 2a/2b RAN4 to come back to case 2a/2b based on progress in the other working groups

In RAN4#110bis, the following has been agreed on the testability and interoperability issue for AI/ML positioning ‎[7].

Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme(if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced

In RAN4#111, the following has been agreed on the testability and interoperability issue for AI/ML positioning ‎[8].

Issue 3-2: Requirements for case 2a Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups Issue 3-3: Requirements for case 2b Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups

In RAN4#112, the following has been agreed on the testability and interoperability issue for AI/ML positioning ‎[9].

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. Issue 3-6: Reported metrics for case 3a/3b Agreement: postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

In RAN4#112bis, the following has been agreed on the testability and interoperability issue for AI/ML positioning ‎[10].

Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.

In RAN4#113, no agreements on testability and interoperability issues were achieved ‎[11].

The issues on AI/ML based positioning require further discussion on selected sub use cases. In this paper, the performance requirements for prioritized Case 1 and Cases 3a/3b are discussed in regard to aspects of performance test metric, measurement accuracy requirements, testability, and LCM-related latency requirements.

UE-based positioning

This section discusses testability aspects and performance test metric in the use cases with a UE-side model. This section refers to Case 1, while UE-assisted positioning (Cases 2a/2b) is treated in the next section.

In view of testability aspects for UE-based positioning, it is a prerequisite that reporting of the estimated location coordinates is enabled. This is considered in the next section.

Reporting of estimated positioning coordinates by UE

In fact, reporting of estimated location coordinates by UE is supported in LPP spec TS 37.055. The spec refers to different location coordinate types based on geographic shapes as defined in TS 23.032:

#### LocationCoordinateTypes The IE LocationCoordinateTypes defines a list of possible geographic shapes as defined in TS 23.032. -- ASN1START LocationCoordinateTypes ::= SEQUENCE { ellipsoidPoint BOOLEAN, ellipsoidPointWithUncertaintyCircle BOOLEAN, ellipsoidPointWithUncertaintyEllipse BOOLEAN, polygon BOOLEAN, ellipsoidPointWithAltitude BOOLEAN, ellipsoidPointWithAltitudeAndUncertaintyEllipsoid BOOLEAN, ellipsoidArc BOOLEAN, ..., [[ highAccuracyEllipsoidPointWithUncertaintyEllipse-r15 BOOLEAN OPTIONAL, -- Need ON highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15 BOOLEAN OPTIONAL -- Need ON ]], [[ ha-EllipsoidPointWithScalableUncertaintyEllipse-r16 BOOLEAN OPTIONAL, -- Need ON ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16 BOOLEAN OPTIONAL -- Need ON ]], [[ local2dPointWithUncertaintyEllipse-r18 BOOLEAN OPTIONAL, -- Need ON local3dPointWithUncertaintyEllipsoid-r18 BOOLEAN OPTIONAL -- Need ON ]] } -- ASN1STOP - NOTE 3: In this version of the specification, the GAD shapes local2dPointWithUncertaintyEllipse and local3dPointWithUncertaintyEllipsoid are supported for DL-TDOA and DL-AoD only.

That means aside the location coordinate types defined in Rel-15 and Rel-16, Rel-18 allows to configure / report 2D and 3D location coordinates, respectively, with a certain uncertainty, both for DL-TDOA and DL-AoD positioning methods. Thereagainst, for the multi-RTT positioning method, signaling of such location coordinates is not supported.

The estimated location coordinates are included in the IEs NR-DL-TDOA-LocationInformation, as shown below for DL-TDOA positioning method, and NR-DL-AoD-SignalMeasurementInformation, respectively, and are conveyed in the ProvideLocationInformation message to the LMF, if requested by the LMF in the RequestLocationInformation message.

#### NR-DL-TDOA-LocationInformation The IE NR-DL-TDOA-LocationInformation is included by the target device when location information derived using NR DL-TDOA is provided to the location server. -- ASN1START NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, ... } OPTIONAL, ..., [[ locationCoordinates-r17 LocationCoordinates OPTIONAL, -- Cond batch1 locationSource-r17 LocationSource-r13 OPTIONAL -- Cond batch2 ]] } -- ASN1STOP - Conditional presence; - Explanation. batch1; The field is mandatory present if the field nr-DL-TDOA-LocationInformationInstances is present in IE NR-DL-TDOA-ProvideLocationInformation; otherwise it is not present.. batch2; The field is optionally present, need ON, if the field nr-DL-TDOA-LocationInformationInstances is present in IE NR-DL-TDOA-ProvideLocationInformation; otherwise it is not present.. - NR-DL-TDOA-LocationInformation field descriptions. measurementReferenceTime This field specifies the time for which the location estimate is valid.. locationCoordinates This field provides a location estimate using one of the geographic shapes defined in TS 23.032. NOTE 1.. locationSource This field provides the source positioning technology for the location estimate. NOTE 1.. NOTE 1: In the case of locationCoordinates for multiple NR positioning methods are provided, the locationCoordinates and locationSource shall be present in only one of NR-DL-TDOA-ProvideLocationInformation or NR-DL-AoD-ProvideLocationInformation.

Most of the location coordinate types are based on an “ellipsoid point with uncertainty circle or ellipse”, as seen in Figure 1 below.

Figure 1. Description of an uncertainty Ellipse [TS 23.032]

Coding both of the point and of the uncertainty is also specified in clause 6 of TS 23.032. For both parameters, higher and lower accuracy definitions exist. For instance, for the uncertainty, for the case of high accuracy, the range 0m to 46.5m is defined in TS 23.023 with a non-equal granularity of 6mm or higher, as depicted in the table of TS 23.023 below. Other resolutions for the uncertainty are lower.

Table 6.2a-1: Example values for the high accuracy uncertainty function

| Value of K | Value of uncertainty |
| --- | --- |
| 0 | 0 m |
| 1 | 0.006 m |
| 2 | 0.01212 m |
| - | - |
| 20 | 0.14578 m |
| - | - |
| 40 | 0.36241 m |
| - | - |
| 60 | 0.68430 m |
| - | - |
| 80 | 1.16263 m |
| - | - |
| 100 | 1.87339 m |
| - | - |
| 120 | 2.92954 m |
| - | - |
| 127 | 3.40973 m |
| - | - |
| 255 | 46.49129 m |

In conclusion, reporting of location estimates is supported in the current specifications. Reuse of this reporting facility for Case1 is hence possible, given that a certain location coordinate type from the existing ones is selected. The UE then is enabled to report its local 3D coordinates with origin in a known reference location together with its estimated location uncertainty, both inferred through its AI/ML model.

Reporting of location estimates is supported in the current specifications.

Reuse of the reporting facility for Case 1 is possible, given that a certain location coordinate type from the existing ones is selected.

The UE is enabled to report its local 3D coordinates with origin in a known reference location together with its estimated location uncertainty, both inferred through its AI/ML model.

RAN4 to use existing location coordinate reporting to identify positioning accuracy requirements for Case 1.

Testability aspects in Case 1

In RAN4#110, we discussed the testability of Case 1 where the test metric in Case 1 can be the positioning accuracy.  Therefore, in this paper, we further discuss the testability aspects of positioning accuracy test metric. To define the feasible requirements for Case 1, it is useful to consider a possible test framework with the four essential steps of identification, setting, running, and evaluation as shown in Figure 2.

Figure 2. Test framework in Case 1

When AI/ML inference results consist of positioning coordinates, the AI/ML functionality can be verified based on the ground truth. The ground truth may consist of the location point(s) with known positioning co-ordinates, for example, in form of a test area/grid as shown in Figure 3. To know the co-ordinates of each location point, a PRU (i.e., Positioning Reference Unit location is known to gNB/LMF) is placed, or the position co-ordinates of each location point form a known reference location derived by, e.g., using GNSS based positioning. In that case, while UE moves from one location point to another location point, the AI/ML inference results at each location corresponding to the location point is reported to the test equipment and compared to the ground-truth information for verification of positioning accuracy.

Figure 3. Grid map providing a ground-truth information for inference evaluation

The test setup can be implemented for a field test. If testing is performed in a limited indoor space, the same test setup can be scaled down to be fitted into the laboratory space. For instance, the test map granularity may be further reduced to a certain level depending on the achievable precision of an AI/ML-based positioning technique, and configured bandwidth size, and channel conditions.

Positioning accuracy can be verified based on the ground truth which may consist of the location points with known positioning co-ordinates (e.g., PRU or GNSS based).

RAN4 should specify (i) the test mechanisms for positioning accuracy and (ii) the metric for positioning accuracy performance in Case 1.

Regarding test mechanisms for positioning accuracy, a grid map in a limited indoor space should be defined to derive the ground truth. The UE is placed on one of these location points and receives PRS resources from a set of TRPs with known location, the set size may be e.g. four. The PRS resources are on the same PFL. The measurement period is derived from the set size of TRPs for single PFL according to TS 38.133. The UE reports its position co-ordinates to the test system (emulating LMF), which compares the reported co-ordinates to ground-truth.

RAN4 to specify a test grid map with known location points in indoor space and an appropriate set of TRPs for the positioning measurement in Case 1.

Performance test metric in Case 1

Assuming the reuse of the existing location coordinate types, as described above, we observe the following issue depicted in Figure 4 related to how to calculate the positioning error.

Figure 4. Example of different calculation methods for the 2D positioning error depending on whether to account for UE location uncertainties Ux and Uy.

The above shown grid map provides the ground-truth information for inference evaluation. The UE reports both its estimated location point and the uncertainty ellipse in case of 2D reporting consisting of semi-axis Ux and Uy. Different calculation methods are possible.

- Error 1 calculation method takes into account the reported location uncertainty and calculates the distance of the closest point of the uncertainty shape to the actual UE location. This refers to a best-case scenario.

- Error 2 calculation method does not take into account the reported location uncertainty and calculates the distance of the estimated UE location (center of uncertainty shape) without location uncertainties to the actual UE location. This refers to an average scenario.

- Error 3 calculation method takes into account the reported location uncertainty and calculates the distance of the furthest point of the uncertainty shape to the actual UE location. This refers to a worst-case scenario.

On one hand, performance may be preserved if the actual UE location is covered by the uncertainty shape, on the other side, a UE may ensure this by simply increasing the location uncertainties. The objective, however, is to reduce the uncertainty shape as much as possible. Thus, error calculation methods 2 or 3 are preferred to be used for determining the performance metric for Case 1.

RAN4 to select either error calculation method 2 (without location uncertainty) or 3 (applying worst case uncertainty error) for deriving the performance metric for Case 1.

Based on the above and more generally, the metric for positioning accuracy for Case 1 identifies the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth, i.e. Δx, Δy and Δz, using a Local Coordinate System (LCS) or a Global Coordinate System (GCS) reference and

- the estimated location reported by UE without location uncertainties or

- the estimated location reported by UE offset by the 3D location uncertainties, such that the 3D error distance is maximized.

It can be derived according to Eq.1:

e= Δx2+Δy2+Δz2	 (Eq.1)

RAN4 to select the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth as performance metric for the positioning accuracy in Case 1.

- 2.4.3	UE-assisted positioning

This section discusses testability aspects and performance test metric in the use cases with a UE-side model (use cases 2a/2b).

Testability aspects and Performance test metric in Case 2a / Case 2b

RAN4#112 discussed reported metrics for Case 2a and Case 2b and achieved following agreements.

Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported.

Case 2a

RAN1#116 meeting agreed that LOS/NLOS indicator and/or timing information are supported for reporting in AI/ML assisted positioning Case 2a.

Agreement For AI/ML assisted positioning Case 2a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via DL RSTD or UE Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

LOS/NLOS indicator can be considered as an intermediate performance metric for case 2a - UE-assisted/LMF-based positioning with UE-side model, AI/ML-assisted positioning.

LOS/NLOS reporting from UE to LMF is already supported for NR positioning in Rel-17. However, when the AI/ML model/ functionality inference is the LOS/NLOS indicator, it should be treated as an intermediate performance metric and specify the minimum performance requirements. Otherwise, LOS/ NLOS indicator used as one of the inputs to the positioning algorithm at LMF will have impact on the positioning accuracy.

LOS/NLOS indicator should be considered as an intermediate performance metric for case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML-assisted positioning).

LOS/NLOS indicator can be verified based on single path channel and/or multipath channels considering the path loss thresholds and exponents etc. One way of extracting the ground truth is by emulating single path channel for LOS and multipath channel for NLOS by placing the obstructions between UE and the TRP(s) and also by emulating the different propagation conditions as shown in Figure 5. This intermediate KPI verification will ensure that the minimum performance requirements are met by the AI/ML model/functionality.

Figure 5: Emulating single path channel for LOS (left) and multipath channel for NLOS (right) by placing obstructions between Transmitter and DUT.

Validation of LOS/NLOS inference can be performed based on the labelled data. Labelled data can be generated by extracting the ground truth based on legacy LOS/NLOS estimation methods using a calibrated device capable of precisely indicating the ratio of received LOS/NLOS signals.

LOS/NLOS as an intermediate metric/KPI can be considered for assisted AI/ML positioning. LOS/NLOS metric/KPI validation requires labelled data.

LOS/ NLOS metric/ KPI labelled data to be generated for Case 2a based on the ground truth extraction using the legacy methods with the help of a well calibrated device capable of precisely indicating the ratio of received LOS/NLOS signals.

Case 2b

RAN1#119 has achieved further agreements on sample-based measurements for Case 3b. For Case 2b the discussion is ongoing in RAN1, whether sample-based measurements for channel response, PDP and DP should be introduced in addition to existing legacy Rel-18 path-based measurements.

Thus, RAN4 needs to wait on further progress in RAN1 to address testability aspects for Case 2b related to existing path -based and, if agreed, sample-based measurements.

RAN4 to wait on further progress in RAN1 to address testability aspects for Case 2b related to existing path-based and, if agreed, sample-based measurements.

- 2.4.4	gNB assisted positioning

This section discusses testability aspects and performance test metric in the use cases with a gNB-side model (use cases 3a/3b).

Testability aspects and Performance test metric in Case 3a / Case 3b

Based on the earlier RAN4 #110 agreement not to specify positioning accuracy requirements for Case 3a and Case 3b, no testability aspects were considered for Case 3a and Case 3b, except if RAN1 specified new measurement types for AI/ML-based positioning or enhances existing measurement types. Meanwhile, RAN1 has agreed to introduce LOS/NLOS indicator measurement for Case 3a and sample-based measurements for Case 3b. Thus, there are testability issues for Case 3a (gNB-side model) and Case 3b (direct positioning, related to gNB measurement output), which need to be investigated by RAN4, as depicted in the following.

For Case 3a, RAN1 #119 agreed to report the LOS/NLOS indicator together with other measurements.

Conclusion​ For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, ​ - LOS/NLOS indicator can’t be reported independently from other measurements​

Thus, RAN4 needs to wait on further progress in RAN1 to address testability aspects for the LOS/LOS indicator together with other measurement(s).

RAN4 to wait on further progress in RAN1 to address testability aspects for the LOS/LOS indicator together with other measurement(s) for Case 3a.

For Case 3b, RAN1#117 agreed to introduce, aside the existing legacy Rel-18 path-based measurements, sample-based measurements of the estimated channel response in time domain. RAN1#119 further agreed the starting point and the length Nt of continuous samples as well as reporting of Nt’ samples with timing information reported relative to a reference time. The values for Nt and Nt’ are still under discussion in RAN1.

Agreement​ For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.​ - starting time = first detected path rounded down with timing granularity T.​ Note: UE-side measurement is a separate discussion​

Agreement​ For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, ​ - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. ​ - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. ​ - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.​ - The timing information is defined relative to a reference time, same as the path-based measurement. ​ - The Nt’ selected time domain channel measurement values are expected to be those with the highest power.​ - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.​ - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:​ - Nt'<=24. FFS: Nt' values. ​ - Nt = {32, 64, 128}​ - FFS: k ​ - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement​ - FFS: whether transmit offset from gNB to LMF​ Note: measurement by UE is a separate discussion.

Note, the discussion is ongoing in RAN1, whether sample-based measurements for PDP and DP should also be introduced

For Case 3b, RAN1 will define path-based and sample-based measurements. Thus, RAN4 should consider testability aspects applicable to path-based and sample-based measurements, for latter awaiting further details from RAN1.

RAN4 to consider testability aspects for Case 3b both for path-based and sample-based measurements, for latter awaiting further details from RAN1.

- 3	Conclusion

In this paper, we provided our view on testability and interoperability aspects for all one-sided models uses cases:

- AI/ML Beam Management

- AI/ML based CSI Prediction

- AI/ML based Positioning enhancements

In the paper, the following Observations and Proposals were made:

Proposal 1: Clarify the questions for the test setup with UE rotations: (1) How many Set A and/or Set B beams can be generated inside the test chamber at the same time? (2) How many test instance/iteration/intervals should be defined based on total number of Set A/B beams?

Proposal 2: UE vendors to clarify if the Set A and/or Set B beams emulated inside the test chamber can’t arrive at the DUT from different AoAs, will it be possible for the DUT to distinguish between the different beams which may have different Beam IDs but will arrive at the DUT from the same AoA.

Observation 1: Reported RSRP values as approximated ground truth are error-prone and have several uncertainties.

Proposal 3: To obtain a more accurate ground truth, RAN4 should consider the case where UL measurements received at TE/NW can be used as ground truth for DL measurements in case when Beam correspondence is ensured between UL and DL beams.

Observation 2: A combination of AWGN/TDL based test system and UE rotation can be a potential candidate for the conformance testing of AI/ML based BM use case.

Proposal 4: RAN4 to discuss and explore the feasibility of performing beam sweeping in multiple iterations during the conformance testing of AI/ML based BM use case.

Proposal 5: For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in Table 1.

Proposal 6: RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML BM functionality, to verify the generalization performance of the functionality in different scenarios.

Observation 3: When radio conditions change, UE will switch from one AI/ML model to another, and proper operation in non-static conditions needs to be ensured throughout testing.

Proposal 7: RAN4 should define requirements/test procedures to ensure that no performance degradation occurs due to internal model switching under non-static conditions.

Observation 4: When UE may have a well-generalized AIML model, no switching needs to be done for UE, and testing is needed to ensure that no performance degradation occurs.

Observation 5: CDL models can be used for generalization testing to ensure that practical scenarios can be simulated in a testing equipment.

Proposal 8: RAN4 to define testing procedures/environment to ensure no performance degradation when running AIML model with TDL channel models for generalization testing.

Proposal 9: RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML CSI prediction functionality, to verify the generalization performance of the functionality in different scenarios

Observation 6: Reporting of location estimates is supported in the current specifications.

Observation 7: Reuse of the reporting facility for Case 1 is possible, given that a certain location coordinate type from the existing ones is selected.

Observation 8: The UE is enabled to report its local 3D coordinates with origin in a known reference location together with its estimated location uncertainty, both inferred through its AI/ML model.

Proposal 10: RAN4 to use existing location coordinate reporting to identify positioning accuracy requirements for Case 1.

Observation 9: Positioning accuracy can be verified based on the ground truth which may consist of the location points with known positioning co-ordinates (e.g., PRU or GNSS based).

Proposal 11: RAN4 should specify (i) the test mechanisms for positioning accuracy and (ii) the metric for positioning accuracy performance in Case 1.

Proposal 12: RAN4 to specify a test grid map with known location points in indoor space and an appropriate set of TRPs for the positioning measurement in Case 1.

Proposal 13: RAN4 to select either error calculation method 2 (without location uncertainty) or 3 (applying worst case uncertainty error) for deriving the performance metric for Case 1.

Proposal 14: RAN4 to select the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth as performance metric for the positioning accuracy in Case 1.

Proposal 15: LOS/NLOS indicator should be considered as an intermediate performance metric for case 2a (UE-assisted/LMF-based positioning with UE-side model, AI/ML-assisted positioning).

Observation 10: LOS/NLOS as an intermediate metric/KPI can be considered for assisted AI/ML positioning. LOS/NLOS metric/KPI validation requires labelled data.

Proposal 16: LOS/ NLOS metric/ KPI labelled data to be generated for Case 2a based on the ground truth extraction using the legacy methods with the help of a well calibrated device capable of precisely indicating the ratio of received LOS/NLOS signals.

Proposal 17: RAN4 to wait on further progress in RAN1 to address testability aspects for Case 2b related to existing path-based and, if agreed, sample-based measurements.

Proposal 18: RAN4 to wait on further progress in RAN1 to address testability aspects for the LOS/LOS indicator together with other measurement(s) for Case 3a.

Proposal 19: RAN4 to consider testability aspects for Case 3b both for path-based and sample-based measurements, for latter awaiting further details from RAN1.

- References

- RP-243245, “New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm Incorporated, RAN#106 meeting, 9 – 12 December 2024, Madrid, Spain.

- R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm Incorporated, RAN4#113 meeting, 18th – 22nd November 2024, Orlando, USA.

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#96, Budapest, Hungary, June 06 – June 09, 2022.

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- R4-2406617, WF on AI/ML, Qualcomm, RAN4#110bis, Changsha, China, April 15 - 19, 2024.

- R4-2403712, WF on AI/ML, Qualcomm, RAN4#110, Athens, Greece, February 26 – March 1, 2024.

- R4-2410570, WF on NR ALML air interface, Qualcomm, RAN4#111, Fukuoka, Japan, May 20 – 24, 2024.

- R4-2404323, WF on NR ALML air interface, Qualcomm, RAN4#112, Maastricht, Netherlands, Aug. 19 – 23, 2024.

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm, RAN4#112bis, Hefei, China, Oct. 14 – 18, 2024.

- R4-2420336, WF on AI/ML air interface part 2, Ericsson, RAN4#113, Orlando, USA, Nov. 18 – 22, 2024.