3GPP TSG-RAN WG4 Meeting #114	R4-2501981

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Agenda item:	7.17.3

Document for:	Discussion

Introduction

In the last RAN4 meeting most of the issues for Fast Scell activation were closed. This contribution discusses about the remaining issues.

Discussion

Issue 1-2-4: UE capability Agreement: Discuss the UE capability by the end of core part.

In the release 18 EMR, the following capabilities were defined:

| measValidationReportEMR-r18 Indicates whether the UE supports measurement validation and report based on EMR measurement during connection setup/resume for fast CA/DC setup. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. A UE supporting this feature shall also indicate support of idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| measValidationReportReselectionMeasurements-r18 Indicates whether the UE supports measurement validation based on reselection measurements during IDLE/INACTIVE state and reporting for fast CA/DC setup. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |

We think that these capabilities are sufficient when considering the scope of the WI. Therefore, we do not see a need for further capability definitions.

No need for further capability definitions for rel-19 Fast SCell activation for UE supporting Rel-18 EMR

Conclusion

The following Observations and Proposals were made:

Proposal 1: No need for further capability definitions for rel-19 Fast SCell activation for UE supporting Rel-18 EMR