# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2501994

# Athens, Greece, 17 – 21 February 2025

# Title:	1UL CC MSD clean-up for intra-band non-contiguous NRCA

# Agenda Item:	6.8.2

# Source:	Skyworks Solutions, Inc., T-Mobile USA.

# Document for:	Approval

# Introduction

This document proposes to clean-up the 1UL CC MSD requirements for intra-band non-contiguous NR-CA. A companion CR is proposed [2].

# Discussion

## On Downlink (DL) PCC/SCC placement relative to the UL band

Contrary to MSD requirements for intra-band contiguous CA, we observe that the requirements on DL PCC/SCC placement relative to the Uplink (UL) band are:

- Not specified in the core-requirement of clause 7.3A.2.2

- Captured using NOTE 4 and NOTE 7 with contradictory requirements in Table 7.3A.2.2-1 (PC3 requirements)

  - NOTE 4: “The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band”

  - NOTE 7: “The carrier center frequency of SCC in the DL operating band is configured closer to the UL operating band”,

- Captured withe NOTE 7 Table 7.3A.2.2-1a (PC2 requirements) which is identical to Table 7.3A.2.2-1 Note.

This usage of footnotes may lead to confusion.

Observation 1: Contrary to 1UL CC MSD requirements for intra-band contiguous NR-CA, the requirements for DL PCC/SCC placement are specified with contradictory footnotes. This may cause confusion on respective applicability. The requirements for DL PCC/SCC placement, relative to the UL band, should be specified in the core requirement text, rather than using footnotes. Footnotes should only be used for exception cases.

## On SCC MSD requirements for cases with more than one DL SCC

1UL CC MSD requirements for intra-band non-contiguous DL CA were introduced in Rel-16. Guidelines for MSD test point selection were never officially captured in TR 38.846, as requirements have been studied on a band-combination to band-combination basis. At this meeting, we have summarized, and to the best of our knowledge, the most up-to-date RAN4 guidelines for all 1UL CC MSD requirements in [1]. We reproduce below our understanding of RAN4 conventions for intra-band non-contiguous CA 1UL CC MSD test points [1]:

For intra-band non-contiguous NR-CA reference sensitivity with one UL carrier:

- It is sufficient to specify only one MSD test point per Bandwidth Combination Set (BCS).

- If a test point can only be supported by a given BCS, a footnote is needed to indicate this restriction.

- The MSD test point is specified for one configured DL SCC for which the reference sensitivity is increased by ΔRIBNC.

- The requirements apply with all downlink carriers active. For operation with more than one non-contiguous DL SCC, ΔRIBNC applies to all secondary component carriers [8].

- Unless otherwise noted, the downlink SCC carrier center frequency shall be configured closer to the uplink operating band than the downlink PCC center frequency.

- The parameter “Wgap” is specified at the maximum value that can be configured to account for the DL band BW, PCC CBW and SCC CBW, i.e. Wgap = DL_BandBW – (PCCCBW + SCCCBW).

- For UL RB allocation,

  - The PCC UL band parameter “LCRB” is equal to the UL RB configuration specified in Table 7.3.2-3 Uplink configuration for reference sensitivity for the PCC DL channel bandwidth.

  - The RBstart is specified to maximize the DL SCC overlap by the lowest dual-uplink IMD order.

Regarding the MSD requirements of DL SCC:

- RAN4 MSD requirements are specified for only 1 DL SCC. This is reflected with Wgap specified at the maximum valued supported for a given DL band and DL PCC/SCC CBW configuration,

- The DL SCC MSD is specified using parameter ΔRIBNC.

- Even though never explicitly specified or even discussed, our understanding is that for the case when more than 1 DL SCC is configured ΔRIBNC applies to all configured DL SCCs.

- For CA_n25(2A): two companies contributed to finalizing the MSD requirements during the electronic meeting #101e, and specifically during the TP for TR flagging process. The MSD test point was designed for CA_n25(2A) but was not meant intended to apply to for CA_n25(3A), since the DL cell configuration does not permit configuring a 2nd DL SCC. For some this reason, CA_n25(3A) is captured in Table 7.3A.2.2-1.

Observation 2:

- For intra-band non-contiguous DL-CA configurations with more than one DL SCC, RAN4 MSD requirements are not clearly defined,

- MSD requirements for DL CA_n25(2A) can not apply to CA_n25(3A). Brackets on PC3 and PC2 MSD requirements for CA_n25(2A) may be removed.

- To our understanding, ΔRIBNC applies to all DL SCCs, hence CA_n25(3A) and CA_n66(3A) may be removed from Table 7.3A.2.2-1 and Table 7.3A.2.2-1a.

- Editorial: Some empty rows for CA_n71(2A) may be removed from Table 7.3A.2.2-1.

- Note 1 of Table 7.3A.2.2-1a may be corrected to “Applicable to UE supporting PC2 with 1Tx.”

Based on these observations, we make the following proposals.

Proposal 1: Adopt the following changes to clause 7.3A.2.2.

For aggregation of two or more downlink FDD carriers with one uplink carrier and unless otherwise noted, the downlink SCC carrier center frequency shall be configured closer to the uplink operating band than the downlink PCC center frequency. For power class 3, the reference sensitivity is increased by ΔRIBNC only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. For power class 2, the reference sensitivity power level is increased by ΔRIBNC for specific uplink and downlink test points which are specified in Table 7.3A.2.2-1a. The requirements apply with all downlink carriers active. For downlink configurations with more than one non-contiguous DL SCC, ΔRIBNC applies to all secondary component carriers. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Proposal 2: Adopt the following changes to Table 7.3A.2.2-1 and Table 7.3A.2.2-1a.

Table 7.3A.2.2-1: Power class 3 intra-band non-contiguous CA reference sensitivity with one uplink carrier.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 24.6 8 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |
| CA_n66(2A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 | FDD |
|  |  | 25MHz + 5MHz11 | Wgap = 5.0 | 20 (RBstart = 19) | 25.1 |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: Void. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 NOTE 11: Applicable only to Bandwidth Combination Set 4 and 5 and for UEs supporting the symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

Table 7.3A.2.2-1a: Power class 2 intra-band non-contiguous CA reference sensitivity
with one uplink carrier.

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 24.8 | 29.3 | FDD |
|  |  | 25MHz + 5MHz1 | Wgap = 5.0 | 20 (RBstart = 8) | 27.23 | 31.83 |  |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 7.3 | 10.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 27.6 8 | 32.68 | FDD |
| CA_n66(2A) | N/A | NOTE 4 | NOTE 5 | NOTE 6, NOTE 7 | 0.0 | 0.0 | FDD |
| NOTE 1: Applicable to UE supporting PC2 with 1Tx. NOTE 2: Applicable to UE supporting PC2 with 2Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. NOTE 4: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 5: All applicable sub-block gap sizes. NOTE 6: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 7: void. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1. |  |  |  |  |  |  |  |

# Conclusion

This contribution summarizes our views on clarifying and simplifying the 1UL CC MSD requirements for intra-band non-contiguous NR-CA.

Observation 1: Contrary to 1UL CC MSD requirements for intra-band contiguous NR-CA, the requirements for DL PCC/SCC placement are specified with contradictory footnotes. This may cause confusion on respective applicability. The requirements for DL PCC/SCC placement, relative to the UL band, should be specified in the core requirement text, rather than using footnotes. Footnotes should only be used for exception cases.

Observation 2:

- For intra-band non-contiguous DL-CA configurations with more than one DL SCC, RAN4 MSD requirements are not clearly defined,

- MSD requirements for DL CA_n25(2A) can not apply to CA_n25(3A). Brackets on PC3 and PC2 MSD requirements for CA_n25(2A) may be removed.

- To our understanding, ΔRIBNC applies to all DL SCCs, hence CA_n25(3A) and CA_n66(3A) may be removed from Table 7.3A.2.2-1 and Table 7.3A.2.2-1a.

- Editorial: Some empty rows for CA_n71(2A) may be removed from Table 7.3A.2.2-1.

- Note 1 of Table 7.3A.2.2-1a may be corrected to “Applicable to UE supporting PC2 with 1Tx.”

Based on these observations, we make the following proposals.

Proposal 1: Adopt the following changes to clause 7.3A.2.2.

For aggregation of two or more downlink FDD carriers with one uplink carrier and unless otherwise noted, the downlink SCC carrier center frequency shall be configured closer to the uplink operating band than the downlink PCC center frequency. For power class 3, the reference sensitivity is increased by ΔRIBNC only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. For power class 2, the reference sensitivity power level is increased by ΔRIBNC for specific uplink and downlink test points which are specified in Table 7.3A.2.2-1a. The requirements apply with all downlink carriers active. For downlink configurations with more than one non-contiguous DL SCC, ΔRIBNC applies to all secondary component carriers. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Proposal 2: Adopt the following changes to Table 7.3A.2.2-1 and Table 7.3A.2.2-1a.

Table 7.3A.2.2-1: Power class 3 intra-band non-contiguous CA reference sensitivity with one uplink carrier.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 125 | 4.7 | FDD |
|  |  |  | Wgap = 45.0 | 255 | 0.0 |  |
| CA_n5(2A) | 15/15 | 15MHz + 5MHz | Wgap = 5.0 | 55 | 6.3 | FDD |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n12(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 7.0 | 5 (RBstart=12) | 3 | FDD |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 24.6 8 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |
| CA_n66(2A) | N/A | NOTE 1 | NOTE 2 | NOTE 3, NOTE 4 | 0.0 | FDD |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 22.2 |  |
|  |  | 25MHz + 5MHz11 | Wgap = 5.0 | 20 (RBstart = 19) | 25.1 |  |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 2: All applicable sub-block gap sizes. NOTE 3: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 4: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 5: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 6: Wgap is the sub-block gap between the two sub-blocks. NOTE 7: Void. NOTE 8: For operation with three or more non-contiguous component carriers, ΔRIBNC applies to all secondary component carriers. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 NOTE 11: Applicable only to Bandwidth Combination Set 4 and 5 and for UEs supporting the symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

Table 7.3A.2.2-1a: Power class 2 intra-band non-contiguous CA reference sensitivity
with one uplink carrier.

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 24.8 | 29.3 | FDD |
|  |  | 25MHz + 5MHz1 | Wgap = 5.0 | 20 (RBstart = 8) | 27.23 | 31.83 |  |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 7.3 | 10.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 27.6 8 | 32.68 | FDD |
| CA_n66(2A) | N/A | NOTE 4 | NOTE 5 | NOTE 6, NOTE 7 | 0.0 | 0.0 | FDD |
| NOTE 1: Applicable to UE supporting PC2 with 1Tx. NOTE 2: Applicable to UE supporting PC2 with 2Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. NOTE 4: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 5: All applicable sub-block gap sizes. NOTE 6: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 7: void. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1. |  |  |  |  |  |  |  |

# References

- R4-2501963 Input to RAN4 PRD on rules for 1UL CC MSD requirements, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc.

- R4-2501951 (HPUE_NR_CADC_SUL_R19) Cat F CR for TS 38.101-1-j00 Corrections to intra-band non-contiguous MSD, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc., T-Mobile USA.