# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2501996

# Athens, Greece, 17 – 21 February 2025

# Title:	On PC2 for the HD-FDD RedCap UE

# Agenda Item:	7.7.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document discusses PC2 RedCap requirements for HD-FDD focusing on A-MPR.

# Discussion

In the Way Forward (WF) [1], it was agreed that, quoting:

- “PC2 RedCap HPUE requirements can be specified in the band agnostic way at least for TDD and HD-FDD

  - The band agnostic way means the PC2 requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.”

RAN4’s method for handling PC2 FDD requirements have continuously evolved since its first introduction in Rel-17 for Bands n1 and n3. We summarize our views on the latest RAN4 consensus regarding PC2 A-MPR requirements in the following observation.

Observation:

- For NS cases where a filter was assumed to evaluate the FDD PC2 A-MPR of the non-RedCap UE, RAN4 assumed that the PC3 filter assumptions also apply to PC2 operation.

- All completed PC2 FDD bands for the non-RedCap UE are also completed for the PC3 HD-FDD RedCap UE,

- There is no reason to adopt a different set of assumptions for either the FDD non-RedCap UE or for the HD-FDD RedCap UE.

Based on this observation, it may be concluded that the PC2 A-MPR requirements specified for the FDD non-RedCap UE may be re-used for the HD-FDD RedCap UE under the condition that the same PC3/PC2 filter assumptions also apply to the HD-FDD RedCap UE.

Proposal: The PC2 A-MPR requirements specified for the non-RedCap UE can be re-used for the PC2 HD-FDD RedCap UE under the condition that, for NS cases where a filter is assumed, the assumptions of the PC3 HD-FDD filter are also valid for PC2 HD-FDD operation in the completed PC2 FDD bands.

# Conclusion

This contribution summarizes our views regarding the A-MPR requirements for the PC2 HD-FDD RedCap UE.

Observation:

- For NS cases where a filter was assumed to evaluate the FDD PC2 A-MPR of the non-RedCap UE, RAN4 assumed that the PC3 filter assumptions also apply to PC2 operation.

- All completed PC2 FDD bands for the non-RedCap UE are also completed for the PC3 HD-FDD RedCap UE,

- There is no reason to adopt a different set of assumptions for either the FDD non-RedCap UE or for the HD-FDD RedCap UE.

Proposal: The PC2 A-MPR requirements specified for the non-RedCap UE can be re-used for the PC2 HD-FDD RedCap UE under the condition that, for NS cases where a filter is assumed, the assumptions of the PC3 HD-FDD filter are also valid for PC2 HD-FDD operation in the completed PC2 FDD bands.

# References

- R4-2306650 WF on RedCap HPUE, 3GPP TSG-RAN WG4 Meeting #106-bis-e, electronic meeting, China Telecom.