3GPP TSG-RAN WG4 Meeting #114	R4-2502012

# Athens, Greece, 17 – 21 February 2025

Agenda item:		7.29.2.2

Source: 		Qualcomm Inc.

Title:	 	NR NTN UL Capacity Enhancements

Document for:		Approval

## 1	Introduction

NR NTN UL capacity enhancement work is on-going in RAN1. RAN4 has had initial discussion on whether any specified scheme results in RAN4 specification impact. In this contribution RAN4 specification impact is further discussed.

## 2	Discussion

Agreement

RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows:

- Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs.

- Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences

- RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs.

- RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs.

Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact.

- RAN1 assumes no specification impact for CFO grouping

- RAN1 does not pursue closed-loop frequency adjustment commands.

- RAN1 assumes that RAN4 does not define new UE requirements for CFO.

In RAN4#113 it was agreed that there is no specification impact from inter-slot OCC over 2 slots. In RAN1#119 in November following agreement was reached (yellow highlights added only for this document).

Based on the RAN1 agreement pre-DFT OCC is no longer pursued. Therefore, only inter-slot OCC lengths 2 and 4 are discussed further. No conclusions was made in RAN4 yet for inter-slot OCC with length 4, but it can be seen that RAN1 does not pursue new frequency adjustment commands and expects no new frequency error requirements. Network implementation shall take care of pairing UEs with similar frequency error to minimize any negative performance impacts.

Given the progress in RAN1, it is proposed that there is no specification impact in RAN4 UE RF from NR NTN UL capacity enhancements work.

Observation 1: RAN1 has scoped down the supported OCC schemes and formally agreed that they do not expect new frequency error requirements to be defined in RAN4.

Proposal 1: There is no RAN4 UE RF specification impact from NR NTN UL capacity enhancements and further discussion on UL capacity enhancements is not needed.

## 3	Conclusion

In this contribution NR NTN UL capacity enhancements were discussed and following observation and proposal was made:

Observation 1: RAN1 has scoped down the supported OCC schemes and formally agreed that they do not expect new frequency error requirements to be defined in RAN4.

Proposal 1: There is no RAN4 UE RF specification impact from NR NTN UL capacity enhancements and further discussion on UL capacity enhancements is not needed.

## 4	References