3GPP TSG-RAN WG4 Meeting #114            	R4-2502030

Athens, GR, 17th-21th, Feb, 2025

Agenda item:	7.10.4

Source:	Samsung

Title:	Discussion on NTN testing NGSO channel model

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the NGSO channel model for NTN testing was under discussion, the related agreements are captured in the WF [1] as following

- TE emulated channel model for satellite mobility - Methodology for Time varying Doppler Shift and propagation delay modelling - For Keplerian definition motion base method, consider the following general procedure to generate the time varying Doppler shift and propagation delay. The detail formulars for each step can be considered as reference in Appendix 1.4. - Step 1: Set the initial ephemeris information parameters with format of position/velocity state vector in Earth-centered earth-fixed frame (ECEF) format according to the existing RAN5 test and initial UE location parameters with the following tables - Step 1.1: covert the initial ephemeris information in ECEF format in table 1 into the format of orbital parameters (Keplerian orbital elements) in ECI (Earth-centered inertial) format in table 2 for orbit generation based on two-body Keplerian model at epoch time T0 - Step 1.2: covert the configured UE location in a format of latitude, longitude, and altitude in table 3 into the ECEF format in table 4 to align with the satellite position/velocity - FFS on the value in table 1, 3 for initial ephemeris information and UE location information - Step 2: Generate the estimated satellite position/velocity vector based on Keplerian model at the time T1 with the initial ephemeris information provided in step 1 - Step 2.1: Calculate the Mean Anomaly value - Step 2.2: Derive the Eccentric Anomaly at time T1 by solving Kepler’s equation - Step 2.3: Derive the True Anomaly at time T1 - Step 2.4: Covert the orbital elements in ECI format into the satellite position/velocity state vector in ECEF format with considering the earth rotation - Note: The detail numerical methods to solve the Kepler’s equation in step 2.2, including number of iterations, accuracy of requirement, can be up to implementation, for example - Numerical method - Newton-Raphson method - Number of iterations - Option 1: 4 - Accuracy requirement E-E0>ϵ - Option 1: ϵ=10-8 - Option 2: ϵ=10-6 - Step 3: Obtain the propagation delay and Doppler shift trajectory based on the interpolation operation at the anytime between T0 and time T1. - Step 3.1: Obtain the satellite position/velocity at time T1 - Step 3.2: Obtain the propagation delay and Doppler shift trajectory between time T0 and time T1, (T0 <= T1) - Note: The update granularity of propagation delay and Doppler shift should be [1] slot - Note: The details interpolation method can be up to implementation, with meets some accuracy requirement, for example - Numerical method - Option 1: cubic spline interpolation - Option 2: linear interpolation - Option 3: calculate propagation delay and Doppler shift at any time instant - Other options are not precluded - Accuracy requirement - Option 1: - Max¦delay-error¦ ~ 3e-13 s - Max¦delay-rate error¦ ~ 3.28e-12 - Other options not precluded - Note: RAN4 will conduct the calibration work for each step in post-meeting. Companies are encouraged to provide the calibration and comparison results with GAMT before RAN4#114 meeting. Based on calibration and comparison results, further discuss how to capture the time varying Doppler shift and propagation delay derivation procedure and formulars into specification in next meeting. - The proponents to use the equation of motion to derive the satellite position/velocity in Steps 1/2, as an alternative method as the Keplerian model, is encouraged to provide more detail procedure and the comparison results with the Keplerian model in the next meeting - NGSO NTN test environment - RAN4 to consider the test environment for NGSO NTN UE testing - Assumption of Satellite altitude - Considering satellite types both LEO-600 and LEO-1200 for TE-emulated time varying doppler shift and delay shift modelling for satellite motion trajectory in this WI - The satellite configuration to be used in each individual performance test shall follow pre-existing conditions for altitude, and as a baseline, no new test cases introduced in this WID to cover for different satellite altitudes - Assumption of UE motion - It is common understanding that the earth rotation should be considered when deriving TE-emulated varying Doppler shift and propagation delay between the satellite and DUT(UE) - Not to consider UE motion during RRM transmit timing test case based on pre-existing test case - Assumption of updated granularity for propagation delay and Doppler shift calculation - The Doppler shift and propagation delay should be calculated and updated based on agreed TE emulated channel model per slot - TE vendors are encouraged to provide the feedback about the implementation feasibility to update the Doppler shift and propagation delay per slot - Assumption of updated granularity for Satellite ephemeris information - Unless otherwise stated on existing test configuration, the TE is expected to refresh satellite ephemeris in SIB 19 at least once every 10.24s - For RF and demodulation requirement, use 10.24s to update the satellite ephemeris information - For RRM requirement, use 2.56s to update the satellite ephemeris information - Assumption of Multiple satellite scenario - Focus on TE emulated channel modelling for single satellite scenario, whether and how to extend multiple satellites scenario pending on RRM session - Assumption of Elevation angle range at time 0 - Set more satellite position/velocity depending on test purpose and pre-existing test configuration - For demodulation test - Option 1: starting from [30°] for both NR and IoT NTN - Option 2: - starting from [30°] for NR NTN - starting from [10°] for IoT NTN - For RF test - starting from [10°] for both NR and IoT NTN with considering maximum positive/negative doppler - starting from [60°] for both NR and IoT NTN with considering the half maximum positive/negative doppler - Initial ephemeris information at time 0 - Option 1 - Consider the NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point to specify the TE-emulated Doppler shift and propagation delay model - Option 2 - GMAT - Other options are not precluded - Time varying of doppler shift and propagation delay generation procedure - Set the configured UE location to Longitude: 121.56076999, Latitude: 25.08439333, and Altitude: 0.0 as same as TS 38.508-1 and TS36. 521-4. - RAN4 requirements - UE assumption on propagator model for satellite motion for RAN4 simulation purpose - Option 1: Consider Keplerian model for UE assumption on the propagator model - Option 2: - Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span. - Option 3: It is up to UE implementation what method is used to estimate the doppler shift and propagation delay based on the signalled ephemeris information - Impact on RAN4 demodulation requirements - Option 1: - RAN4 NTN UE demodulation requirements specification need to configure the period of ephemeris information update - RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - Inclusion in Specification - RAN4 to incorporate the TE-emulated channel model with time varying Doppler shift and propagation delay into the annex of TS 38.101-5 and TS 36.102 - FFS on how to capture into specification

In this contribution, the view on the remaining issue of NTN NGSO testing channel model and related requirements is provided.

# 2	Channel Model

Methodology for Time varying Doppler Shift and propagation delay modelling

In the last meeting, RAN4 agreed to general procedure for generating the time varying Doppler shift and propagation delay based on Keplerian definition motion method.  In order to facilitate the TE implementation and alignment for channel model, companies provided the calibration results in post-meeting based on detail formular for each step to generate the doppler shift and propagation delay. Based on calibration results in [2], the result among companies are aligned well with following each steps

- Step 1: Set the initial ephemeris information parameters with format of position/velocity state vector in Earth-centered earth-fixed frame (ECEF) format according to the existing RAN5 test and initial UE location parameters

  - Step 1.0: covert the initial ephemeris information in ECI format in table 1 with considering the earth rotation

  - Step 1.1: covert the initial ephemeris information in ECI format into the format of orbital parameters (Keplerian orbital elements) in ECI (Earth-centered inertial) format in table 2 for orbit generation based on two-body Keplerian model at epoch time T0 =0

  - Step 1.2: covert the configured UE location in a format of latitude, longitude, and altitude in table 3 into the ECEF format in table 4 to align with the satellite position/velocity

- Step 2: Generate the estimated satellite position/velocity vector based on Keplerian model at the time T1 with the initial ephemeris information provided in step 1

  - Step 2.1: Calculate the Mean Anomaly value at time T1 (T1=1s)

  - Step 2.2: Derive the Eccentric Anomaly at time T1 by solving Kepler’s equation

  - Step 2.3: Derive the True Anomaly at time T1

  - Step 2.4-1 (Optional): Covert the orbital elements in ECI format into the satellite position/velocity state vector in perifocal frame

  - Step 2.4-2 (Optional): Covert the orbital elements in ECI format into the satellite position/velocity state vector in ECI format

  - Step 2.4: Covert the orbital elements in ECI format into the satellite position/velocity state vector in ECEF format

- Step 3: Obtain the propagation delay and Doppler shift trajectory based on the interpolation operation at the anytime between T0 and time T1.

  - Step 3.1: Obtain the satellite position/velocity at time T1

  - Step 3.2: Obtain the propagation delay and Doppler shift trajectory between time T0 and time T1, (T0 <= T1)

Regarding the detail numerical methods to solve the Kepler’s equation in step 2.2, including number of iterations, accuracy of requirement, can be up to implementation. Companies provide the results with different value of accuracy of requirement as

  - Accuracy requirement E-E0>ϵ

    - Option 1: ϵ=10-7

    - Option 2: ϵ=10-10

    - Option 3: ϵ=10-12

  - Number of iterations: 4

Considering the tradeoff of complexity for New Raphson method and accuracy, Option 1 can be considered as baseline assumption for TE emulated channel model implementation.

Regarding the detail interpolation method for obtain the propagation delay and Doppler shift trajectory between time T0 and time T1, the accuracy of cubic spline interpolation and linear interpolation is similar. Considering the complexity, we propose to apply the linear interpolation as baseline assumption for TE emulated channel model implementation

Proposal 1: Considering the following as baseline assumption for TE emulated channel model implementation

  - Accuracy requirement E-E0>ϵ

    - Option 1: ϵ=10-7

  - Number of iterations: 4

  - Interpolation method: linear interpolation

TE vendors are encouraged to provide the input about the implementation feasibility

Assumption of Elevation angle range

Regarding the evaluation angle, it will impact on the delay trajectory and doppler trajectory visible range.  With small evaluation angle, the worst-case scenario can be covered, while it requires long testing time to cover the visible satellite coverage range. The testing time should be considered. With larger evaluation angle, the test coverage time can be reduced, where the doppler shift and delay shift are relatively small.

Therefore, the proper evaluation angle should be selected considering the test time and satellite motion trajectory. Based on the profile provided for doppler shift and propagation delay with satellite type of both LEO 600 and LEO 1200 with 88o orbit inclination in [2] as following

Figure 1:  LEO 600km with 88o orbit inclination

Figure 2:  LEO 1200km with 88o orbit inclination

Observation 1: Based on elevation range of [30°~150°], around 300s elapsed time is required for LEO 600 and around 500s elapsed time is required for LEO 1200. Based on elevation range of [10°~170°], around 500s elapsed time is required for LEO 600 and around 900s elapsed time is required for LEO 1200.

For demodulation test, in Rel-17 NR NTN channel model discussion, the evaluation angle with 30°was selected for NTN-TDLA channel case. To align with Rel-17 NTN channel model discussion, evaluation angle start from  30° can be considered as the proper range for test with affordable test timing for NR NTN

Meanwhile, since TE-emulated channel model will also be applied for IoT NTN. Different with NR, larger number of repetitions transmission is required, there, the large number of testing time is required. In current IoT NTN test, the maximum number of repetition transmission is configured as 128. Therefore, larger testing time is need, the evaluation angle starts from 10° can be considered to derive the converged requirement for IoT NTN testing.

For in current RF test, the elevation range of [10 °~170°] was considered, considering the maximum doppler shift.

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 |  |  |
| positionY-r17 | 4550419 |  |  |
| positionZ-r17 | 852799 |  |  |
| velocityVX-r17 | 6164 |  |  |
| velocityVY-r17 | -19424 |  |  |
| velocityVZ-r17 | 124281 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 |  |  |
| positionY-r17 | 3404229 |  |  |
| positionZ-r17 | 3535794 |  |  |
| velocityVX-r17 | 35394 |  |  |
| velocityVY-r17 | -74414 |  |  |
| velocityVZ-r17 | 94682 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

Proposal 2: For demodulation test, to align with Rel-17 NR NTN channel model discussion, the elevation angle start from 30°can be considered for testing. Large testing period, i.e., elevation angle starts from 10° can be considered for IoT NTN testing.

Initial ephemeris information at time 0

According to RAN5 test, position velocity state vectors are used for Ephemeris info signaling

#### 5.6.2.0 Assumptions for Ephemeris generation This section provides the assumptions used to generate Ephemeris information for GSO and NGSO orbits: 1. Ephemeris files are generated for 3 types of satellites: GSO (inclination equal to 7º), LEO-600 and LEO-1200. 2. UE location is as defined in section 5.6.1. 3. PositionVelocity state vectors are used for EphemerisInfo signaling. 4. For GSO, an altitude of 35,786 km is considered. For NGSO, 2 different altitudes are considered: 600 km and 1200 km. 5. One-way delay between UE and satellite to be in the range of 119.375 ms to 128.79 ms for GSO and between 2 ms (lowest value for LEO orbit 600 km) and 6.67 ms (highest value for LEO orbit 1200 km)

Based on different test purpose of RF, RRM and demodulation, the initial ephemeris information at time 0 provided is different in current NTN test. Since the channel study is more generic, we prefer to apply the NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point to specify the TE-emulated Doppler shift and propagation delay model. Pending the test purpose, the existing condition can be reused during individual test.

Proposal 3: Considering NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point for initial ephemeris information at time 0 to specify the TE-emulated Doppler shift and propagation delay model. Pending on the test purpose, the existing condition can be reused during individual test.

# 3	RAN4 requirement impact

According to the WID of this WI as following, NGSO testing is one of major objective

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

Moreover, based on RAN4#112 meeting, RAN4 agreed to focus on channel model discussion in initial stage for NGSO testing. Once sufficient progress made for channel model introduction, RAN4 can start the discussion on the impact to UE demodulation, RF/RRM test cases.

Agreement in RAN4#112meeting Issue 1-1-1: Work plan on channel model and requirements - Agreement - Focus on TE-emulated channel model in initial stage for NGSO testing - Once sufficient progress made for channel model introduction, RAN4 can start the discussion on corresponding requirements and tests according to WI objective.

RF and RRM core requirements impact

Rel-17 RAN4 RF/RRM/Demod requirements are specified based on the assumption with UE pre-compensation on doppler shift and time delay. From RAN4 core and performance requirements perspective, these requirements can be applicable for both non-zero/zero doppler/delay conditions; and which also irrespective to constant or time varied delay/ Doppler condition which related to Satellite motion.

Observation 2: RAN4 Rel-17 IoT over NR and NR NTN requirements (RRM, RF and performance) are specified based on the assumption that UE shall be capable of pre-compensation of doppler shift and time delay for Satellite access link; and requirements agonistic to test conditions of doppler and time delay due to Satellite motion.

As captured in the WID, RF and RRM core requirements shall be maintained regardless channel condition. Meanwhile introducing variable time/frequency to model NGSO motion trajectory will bring challenge for testing and UE to pass corresponding RF (frequency error) and UL timing RRM test cases.

Proposal 4: RAN4 shall consider additional TT and/or MU for RF (frequency error) and UL timing RRM test cases if variable channel model introduced without change on the existing core requirements.

Demodulation requirements impact

Regarding the demodulation requirement, the requirements for NR PDSCH are also defined based on the assumption that UE capable of pre-compensation of doppler shift and time delay. As specified in the TS 38.508, the NTN tests defined, corresponding to zero Doppler and constant delay for the type of satellite under test, keeping such information constant throughout the duration of each measurement in the test. Meanwhile, the EphemerisInfo-r17 values in SIB19 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 5.6.2 (kept constant throughout the duration of each measurement in the test).

Observation 3: NTN tests defined, corresponding to zero Doppler and constant delay for the type of satellite under test, and EphemerisInfo-r17 values in SIB19 kept constant throughput the duration of each measurement in the test.

To check whether the existing NR NTN and IoT NTN can be applied, we see one possible way is to add some residual frequency offset and time offset with considering the estimation/pre-compensation errors. Similar as IoT over NTN for SAN requirement, we suggest to consider the fixed frequency offset and time offset during the test.

Regarding the value of frequency error and timing error, the worst case of 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s can be considered to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply, compared to ideally pre-compensated, according to core requirement. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

Table 7.1C.2-1: Te_NTN Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR1-NTN; 15; 15; 29*64*Tc. 30; 24*64*Tc. 60; N/A. 30; 15; 24*64*Tc. 30; 22*64*Tc. 60; N/A. Note 1: Tc is the basic timing unit defined in TS 38.211 [6].

Proposal 5: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

UE assumption on propagator model for satellite motion for RAN4 simulation purpose

Regarding UE assumption on propagator model for satellite motion, one option is to consider Keplerian model, Keplerian propagator model as it offers a good trade-off between implementation complexity and performance. With this assumption, UE can do pre- compensation based on Keplerian model, although it does not prevent UE from implementing a more accurate propagator model than Keplerian model. In general, since it is related to how to compensate the time/frequency shift in the UE receiver side, we think it should be up to UE implementation. Meanwhile, considering RAN4 has already agreed to apply the Keplerian model for TE implementation in the last meeting, we think there is no necessary to be aligned. So, there is no UE assumption needed.

Regarding option 2, evaluate the performance based on agreed TE emulated channel model firstly, according to the test environment of NGSO NTN UE testing as following, agreed in the last meeting

The UE Doppler and propagation delay estimation, and pre-compensation will be verified before the baseband processing, where the estimation is not reference signal based. After the baseband, only residual frequency offset error or time offset can be handled by reference signal, such as TRS for NR and CRS for LTE eMTC/NB-IoT.

Observation 4: The UE Doppler and propagation delay estimation, and pre-compensation will be verified before the baseband processing, where the estimation is not reference signal based. After the baseband, only residual frequency offset error or time offset can be handled by reference signal

Therefore, we may need to further discuss how to incorporate the TE emulated channel model for simulation

Proposal 6: No need the UE assumption on propagator model for satellite motion for RAN4 simulation

Proposal 7: RAN4 may further discuss how to incorporate the TE emulated channel model for simulation

# 4	Specification of Channel Model

Furthermore, RAN4 agree to incorporate the TE-emulated channel with time varying Doppler Shift and propagation delay into the annex of TS 38.101-5 and TS 36.102 for both NR NTN and IoT NTN.

Regarding how to capture into specification, RAN4 should further discuss.

In order to facilitate TE-emulated channel and also simulation alignment, the detail channel model for demodulation test was descripted into specification in the Annex (normative), for example, in B.2.1 of 38.101-4 for delay profiles generation, and high-speed train scenario in B.3

For delay profiles generation, it was simplified from the TR 38.901 TDL model, considering the implementation of TE-vendor, where the detail step of simplification was specified.

For high-speed train scenario, the propagation channel model was specified with the detail formula and the corresponding trajectory, which was defined as specific scenario.

Observation 5: For channel model delay profiles generation, it was simplified from the TR 38.901 TDL model, considering the implementation of TE-vendor, where the detail step of simplification was specified.

Observation 6: For high-speed train scenario, the propagation channel model was specified with the detail formula and corresponding trajectory

For NTN NGSO channel model, in our understanding, was derived by Keplerian definition motion base method, which was public materials, can be referred the book or academic paper.  It is not clear whether to specify the detail formula directly into the specification is allowed for 3GPP specification drafting rule, based on TR 21.801

Meanwhile, based on WID, there is no TR reserved for NTN channel model output

Observation 7: Based on WID of NTN NGSO testing, there is no new TR reserved

Therefore, regarding how to capture into specification, we see they may be one possible way, to specify the detail procedure for each step to generate the time varying doppler shift and propagation delay, similar as delay profiles generation, without explicitly specifying the detail formulas. To facilitate TE implementation and calibration, capture the output for each step and final doppler shift trajectory and propagation delay trajectory under the agreed NGSO test conditions.

Proposal 8: FFS to specify the detail formula directly into specification for NTN NGSO channel model, considering the Keplerian definition motion base method was public materials, can be referred the public book or academic paper.

Proposal 9: RAN4 can further discuss whether it is feasible to specify the detail procedure for each step to generate the time varying doppler shift and propagation delay, without explicitly specifying the detail formula, capture the output for each step, and final doppler shift trajectory and propagation delay under the agreed NGSO test conditions.

# 5	Conclusion

In this contribution, the view on the NTN testing NGSO channel model is provided.

Proposal 1: Considering the following as baseline assumption for TE emulated channel model implementation

  - Accuracy requirement E-E0>ϵ

    - Option 1: ϵ=10-7

  - Number of iterations: 4

  - Interpolation method: linear interpolation

TE vendors are encouraged to provide the input about the implementation feasibility

Observation 1: Based on elevation range of [30°~150°], around 300s elapsed time is required for LEO 600 and around 500s elapsed time is required for LEO 1200. Based on elevation range of [10°~170°], around 500s elapsed time is required for LEO 600 and around 900s elapsed time is required for LEO 1200.

Proposal 2: For demodulation test, to align with Rel-17 NR NTN channel model discussion, the elevation angle start from 30°can be considered for testing. Large testing period, i.e., elevation angle starts from 10° can be considered for IoT NTN testing.

Proposal 3: Considering NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point for initial ephemeris information at time 0 to specify the TE-emulated Doppler shift and propagation delay model. Pending on the test purpose, the existing condition can be reused during individual test.

Observation 2: RAN4 Rel-17 IoT over NR and NR NTN requirements (RRM, RF and performance) are specified based on the assumption that UE shall be capable of pre-compensation of doppler shift and time delay for Satellite access link; and requirements agonistic to test conditions of doppler and time delay due to Satellite motion.

Proposal 4: RAN4 shall consider additional TT and/or MU for RF (frequency error) and UL timing RRM test cases if variable channel model introduced without change on the existing core requirements.

Proposal 5: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

Observation 4: The UE Doppler and propagation delay estimation, and pre-compensation will be verified before the baseband processing, where the estimation is not reference signal based. After the baseband, only residual frequency offset error or time offset can be handled by reference signal

Proposal 6: No need the UE assumption on propagator model for satellite motion for RAN4 simulation

Proposal 7: RAN4 may further discuss how to incorporate the TE emulated channel model for simulation

Observation 5: For channel model delay profiles generation, it was simplified from the TR 38.901 TDL model, considering the implementation of TE-vendor, where the detail step of simplification was specified.

Observation 6: For high-speed train scenario, the propagation channel model was specified with the detail formula and corresponding trajectory

Observation 7: Based on WID of NTN NGSO testing, there is no new TR reserved

Proposal 8: FFS to specify the detail formula directly into specification for NTN NGSO channel model, considering the Keplerian definition motion base method was public materials, can be referred the public book or academic paper.

Proposal 9: RAN4 can further discuss whether it is feasible to specify the detail procedure for each step to generate the time varying doppler shift and propagation delay, without explicitly specifying the detail formula, capture the output for each step, and final doppler shift trajectory and propagation delay under the agreed NGSO test conditions.

# References

R4-2419786, Way Forward for [113][323] NTN_testing_NGSO_channel_model

R4-2502031, Calibration results summary for NTN testing NGSO channel model