3GPP TSG-RAN WG4 Meeting #114	R4-2502073

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5422 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR introducing test case for Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introducing test case for Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In RAN4#110bis meeting in Changsha RAN4 agreed on the list of test cases to be developed for NR_FR1_lessthan_5MHz_BW. For RLM RAN4 decided to have a split between DRX and non-DRX: - RLM-1: Radio Link Monitoring (SSB-based, FR1): DRX, Out-of-sync, 12 PRBs - RLM-2: Radio Link Monitoring (SSB-based, FR1): DRX, In-sync, 12 PRBs - RLM-3: Radio Link Monitoring (SSB-based, FR1): Non-DRX, Out-of-sync, 15 PRBs - RLM-4: Radio Link Monitoring (SSB-based, FR1): Non-DRX, In-sync, 15 PRB However, by accident both RLM-2 and RLM-4 were implemented as non-DRX into a single test case. This CR split the test case A.6.5.1.15 Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode, according to the agreement and introduces test case RLM-2 Test case A.6.5.1.16 Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode is introduced. TBDs updated according to the baseline test cases A.6.5.1.2.1 and A.6.5.1.4.1 respectively. A.3.1.2.1-1, and A.3.1.3.1-1 and A.3.10.1.13-1 used with the corresponding PRB configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR_FR1_lessthan_5MHz_BW are not implemented according to agreement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.1.15, A.6.5.1.16 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR was agreed in RAN4#112 (R4-2412788). However, the cover page showed wrong release (Rel-17). This CR is a resubmission of R4-2412788 with no technical changes compared to the agreed CR R4-2412788. 12 PRB configuration has been removed. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change#1>

#### A.6.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode

##### A.6.5.1.15.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting [FG-x capability] properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell operating on a 3MHz channel bandwidth. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

Supported test configurations are specified in Table A.6.5.1.15.1-1. General test parameters as specified in Table A.6.5.1.2.1-2 with config 1 apply to this test for both config 1 and 2, except those specified in Table A.6.5.1.15.1-2. Cell specific test parameters as specified in Table A.6.5.1.2.1-3 apply except those specified in Table A.6.5.1.15.1-3.

The test procedure specified in A.6.5.1.2.1 applies to this test.

Table A.6.5.1.15.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 3 MHz (15PRB) |
| Note: The UE is required to pass the test with configuration 1 for every supported 3MHz band in FR1. |  |

Table A.6.5.1.15.1-2: General test parameters for 15 PRB FR1 in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| BWchannel | Config 1 | MHz | 3: NRB,c = 15 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.3 FDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.7 FDD |
| SSB Configuration | Config 1 |  | SSB.13 FR1 |
| In sync transmission parameters | Number of Control OFDM symbols, Config 1 |  | 3 |
|  | Number of Control OFDM symbols, Config 1 |  | 2 |
|  | Aggregation level, Config 1 | CCE | 4 |
|  | Mapping from REG to CCE, Config 1 |  | Distributed |
| Out of sync transmission parameters | Number of Control OFDM symbols, Config 1 |  | 3 |
|  | Aggregation level, Config 1 | CCE | 8 |
|  | Mapping from REG to CCE, Config 1 |  | Non -Distributed |

Table A.6.5.1.15.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |

##### A.6.5.1.15.2	Test Requirements

Test requirements specified in Clause A.6.5.1.2.2 apply to this test.

#### A.6.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode

##### A.6.5.1.16.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting [FG-x capability] properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell operating on a 3MHz channel bandwidth. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

Supported test configurations are specified in Table A.6.5.1.16.1-1. General test parameters as specified in Table A.6.5.1.4.1- with config 1 apply to this test, except those specified in Table A.6.5.1.16.1-2. Cell specific test parameters as specified in Table A.6.5.1.4.1-3 apply except those specified in Table A.6.5.1.16.1-3.

The test procedure specified in A.6.5.1.4.1 applies to this test.

Table A.6.5.1.16.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 3 MHz (12 PRB) |
| Note: If the UE indicates the support for band n100, the UE is also required to pass the test with 12 PRB configuration. |  |

Table A.6.5.1.16.1-2: General test parameters for FR1 in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| BWchannel | Config 1 | MHz | 3: NRB,c = 12 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.2 FDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.6 FDD |
|  | Config 1 |  | SSB.13 FR1 |
| SSB Configuration |  |  |  |
| In sync transmission parameters | Number of Control OFDM symbols, Config 1 |  | 2 |
|  | Aggregation level, Config 1 |  | 2 |
|  | Mapping from REG to CCE, Config 1 |  | Distributed |
| Out of sync transmission parameters |  |  |  |
|  | Number of Control OFDM symbols, Config 1 |  | 2 |
|  | Aggregation level, Config 1 |  | 4 |
|  | Mapping from REG to CCE, Config 1 |  | Distributed |

Table A.6.5.1.16.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |

##### A.6.5.1.16.2	Test Requirements

Test requirements specified in Clause A.6.5.1.4.2 apply to this test.

<End of Changes>