3GPP TSG-RAN WG4 Meeting #114 	R4-2502085

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item: 7.28.2

Source: MediaTek inc.

Title: Discussion on RRM requirements for XR

Document for: Discussion

# Introduction

The work item (WI) for XR (eXtended Reality) for NR Phase 3 has the following objectives [1]:

The Rel-19 XR ph3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. - Note: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - If justified, define a mechanism for transmitter to inform the receiver of SN gap (or missing SNs) in PDCP. - Specify Core requirements related to the above objectives as necessary [RAN4]

In this contribution, the potential directions that leads to enhance the gap cancellation mechanisms are discussed then other RRM impact issues are raised, which requires some studies.

# Discussion

RAN4 RRM has discussed and identified the impact on RRM requirements as below.

# Discussion on scenarios for measurement skipping

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [2].

Issue 1-1: Deployment scenarios:

< Agreement>: from two meetings ago: - Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation < Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID <Way forward>: - Further discussion needed for - FR2-2 - NR-DC

Due to the limited TU for this WI, RAN4 should de-prioritize FR2-2 and NR-DC.

- RAN4 should de-prioritize FR2-2 and NR-DC.

Issue 1-3/1-4: L3/L1 Measurements without gaps with scheduling restrictions:

< Agreement> from meeting RAN4#112: - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed. < Way forward> from meeting RAN4#113: Issue 1-3: L3 Measurements without gaps with scheduling restrictions <Way forward>: - Option 1: Skipping of measurements without gaps with scheduling restrictions are supported - Option 1a: Support skipping of measurements without gaps - Option 1b: Support skipping of measurements without gaps for 100MHz CBW band - Option 2: Skipping of measurements without gaps with scheduling restrictions are deprioritized or not supported - Option 2a: Measurement skipping does not apply for measurements without gaps - Option 2b: Deprioritize Tx/Rx in L3 measurement without gap with scheduling restriction. Issue 1-4: L1 measurements with scheduling restrictions and procedures <Way forward>: - Option 1: Support skipping of L1 measurements and procedures. - Option 1a: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 1b: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD. - Option 1c: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 2: Not to support or deprioritize skipping of L1 measurements and procedures. - Option 2a: Measurement skipping does not apply for L1 measurements and procedures - Option 2b: RAN4 should keep the scheduling restriction impact on XR traffic FFS for now until there are sufficient progress in MG cancellation impact.

RAN4 RRM agreed to discuss the impact of L3 measurements occasions on XR Tx/Rx. Meanwhile, the impact of L1 measurements remains for further study. The main reason is that when L3 measurements is based on SSB outside the active BWP, the UE requires Tx/Rx interruption over a slot-level for all CCs in the form of MG or NCSG or NeedForInterruption (NFG). Out of the measurements criteria, MG can have the most severe impact on Tx/Rx due to its long interruption period, which can be up to 6ms. On contrary, scheduling restriction causes interruption on a symbol-level for a specific serving cell CC. In addition, L3/L1 based measurements without gaps with scheduling restrictions include serving cell measurements. Hence, cancelling a serving cell measurement occasion might impact mobility performance and/or RLM. Therefore, given the workload of this WI compared to the TU allocated for it, we suggest to focus the RRM discussion on L3 measurements impact due to MG.

Observation 1: Measurement gap based L3 measurements is the main cause of reducing XR system capacity, while the scheduling restriction impact of L1/L3 can be tolerated.

Observation 2: L3/L1 based measurements without gaps with scheduling restrictions include serving cell measurements, where cancelling a serving measurement occasion results in UE mobility performance degradation.

- RAN4 should deprioritize the scheduling restriction impact on XR traffic for both L1 and L3 measurements.

Issue 1-5: L1/L2 triggered mobility measurements and procedures:

<Way forward>: - Option 1: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM - Option 3: Only L1 measurements for LTM within gaps are supported for gap skipping. - Option 4: For L1 measurement for mobility, apply same decision as normal L1 measurement.

When a NW configure a UE with MG, the UE shall use the configured gap for all reported frequency layers with ‘gap’ including L1 measurements with gaps. Now, provided that there is no clear indication on which MG occasion to be used for L1 measurements with gaps, therefore, LTM should be supported with gap skipping.

Observation 3: there is no clear indication on which MG occasion to be used for L1 measurements with gaps (LTM) or for L3 based measurements.

- Only L1 measurements for LTM within gaps are supported for gap skipping.

# Discussion on measurements with gaps

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [2].

Issue 2-1: General approach for defining requirements for measurements with gaps:

Issue 2-1: General approach for defining requirements for measurements with gaps: <Way forward>: - Option 1: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type. - Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS. Issue 1-5: L1/L2 triggered mobility measurements and procedures: <Way forward>: - Option 1: support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

In our view, the above two issues should be merged. Besides, when it comes to discuss the priority of different gaps and interruptions, we should take into consideration the workload and necessity to introduce such enhancements. Also, RAN4 should consider whether the solution for a specific feature can be extended to newer features. For example, whether the solution for Type-1 MG can be applied for concurrent gaps, NCSG, Pre-MG, etc. Therefore, RAN4 RRM should focus on the solution mechanism at this stage of discussion then RAN4 can work the solutions for a specific feature if it is found that some gap features cannot be supported.

- RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

Issue 2-2: Types of measurement gaps to consider:

In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. <Way forward>: Please discuss the priorities for RAN4 to work based on the following table: Cap types; First prio; Second prio; Don’t include. Type 1. Type 2. NCSG. MUSIM. Positioning. Rel 18 concurrent gap. PreConfigured.

RAN4 shall decide on whether the UE is expected to do all the measurements while operating with XR data traffic. RAN4 has already agreed that Type-1 should be considered with high priority, while all other measurements gaps/interruption can be second priority. Besides, RAN4 should discuss the necessity of having positioning and MUSIM measurement gaps while the UE is operating with XR traffic.

In addition, the solution of Type-1 can be applied for other measurement gap types at a later stage. Yet, RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types.

- RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic.

- RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types.

# Discussion on XR skipping requirements impact

In the previous RRM meeting, issues related to the scenarios are captured in the way forward [2].

Issue 3-1: How to capture impact of measurement skipping <Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay <Way forward>: Proposals related to measurement delay requirements: - Option 1: Proposals without distinction of XR and legacy requirements - Option 1a: As minimum requirements, cell detection, index reading, and measurement period is extended. - Option 1b: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic. - Option 1c: extend the L3 requirements with MGs such as cell identification and measurements delays when one or more MGs are deactivated by the network. - Option 1d: RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR. - Option 2: Proposals with distinction of conditions where measurement impact is not needed - Option 2a: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information. - Option 2b: - Apply legacy measurement delay and accuracy requirements for the following condition: - the skipped measurement occasion(s) indicated by network is the unused measurement occasion reported by UAI - Otherwise, extend measurement delay while keep legacy accuracy requirements unchanged Issue 3-2: Principles for measurement delay calculation <Way forward>: - Option 1: Options based on scaling factor - Option 1a: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. - Option 1b: Introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window. - Option 1c: introduce scaling factor to make sure enough measurement samples. Besides the ratio between applicable chances and total chances as in legacy, some channel quality could be when determining the exact scaling factor. - Option 1d: pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor). - Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. - Option 2a: dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs. Issue 3-3: Conditions for measurement skipping <Way forward>: - Option 1: max number of occasions cancelled during a measurement period shall be defined. - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration. - Option 3: discuss MG configurations relevant for MG cancellation, e.g., - minimum MGL (e.g., MGL≥3 ms, due to low gain with MG skipping), - maximum MGL (e.g., MGL≤6 ms), - maximum MGRP (e.g., MGRP≤320 ms to avoid too long delays), etc. - Option 4: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

Furthermore, RAN4 discussed the impact on the measurements delay requirements due to MG cancellation. The number of samples for a single layer measurement was decided based on simulation evaluation for different channel environment. Given that the XR is meant to operate with NR devices then the focus shall be on the impact of measurement gap cancellation and how to compensate the dropped measurement samples. In our view, when the XR data leads to MG cancellation then the total measurement delay shall be extended by a puncturing scaling factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

- RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

- Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

- RAN4 shall introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

Furthermore, cancelling multiple measurement gaps occasions might result in mobility performance degradation. Therefore, RAN4 should discuss how to limit such impact. One option is to limit the max number of cancelled gap occasions within a specific time window.

- RAN4 shall discuss and define the max number of occasions cancelled during a measurement period.

# Discussion on need/feasibility of UE assistance information

Issue 5-1: General on UE assistance information (UAI):

Issue 5-1: General on UAI <Way forward>: - Companies are tasked to discuss the need and feasibility of UAI, based on the following options - Proposals related to measurement occasions - Option 1: Define UAI related to measurement occasions - Option 1a: number of occasions which can be skipped - Option 1b: number of measurement occasions that are needed for measurement - Option 1c: UE indicates NW a status whether it can tolerate measurement cancellation. - Option 1d: patterns of gaps positions of MG that can be skipped - Option 1e: study how UE assistance information/UE capability is reported when DRX is configured. - Option 1f: Maximum number of consecutive MGs that can be skipped - Option 2: - Option 2a: Do not define UAI related to measurement occasions. - Option 2b: RAN4 shall follow RAN1 agreement to deprioritise the UAI based solution - Option 2c: RAN4 will not specify any XR-related requirement, based on XR UAI. - Option 3: The information about gaps not used for measurement are reported as UE capability. - Option 4: The impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side, should also be considered in UAI discussions. - Proposals related to channel conditions - Option 1: For UEs configured with search threshold (s-MeasureConfig), the UE informs the network that it is stopping doing non-serving cell measurements because the s-MeasureConfig condition is no longer fulfilled.

In existing legacy NR spec, the UE assistance information (UAI) is a special corresponding message to the RRC reconfiguration, which is sent from the UE to the NW, in which, the UE inform the NW with its internal status. In addition, not all the UAI messages have defined corresponding actions from the NW nor from the UE, yet, some UAI messages indicate the initiation of a different UE behaviour. For example, A UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state shall initiate the procedure for providing an indication of its relaxation state for RLM measurements (a UEAssistanceInformation message with rlm-MeasRelaxationState) upon being configured to do so, and upon change of its relaxation state for RLM measurements in RRC_CONNECTED state.

For Rel-19 XR, RAN1 has discussed different possibilities for UAI, which are summarised in their agreements listed in the LS sent to RAN4 [3]. This includes: (i) UE assistance information related to measurement occasions, (ii) UE assistance information related to channel conditions, (iii) UE assistance information related to traffic, and (IV) UE assistance information related to UE mobility. In addition, RAN1 has deprioritised UAI related to channel condition, traffic and UE mobility. Besides, RAN1 mentioned that there is no consensus on whether to support UAI related to measurements occasions. Given that this topic is within RAN4 area of expertise, then RAN4 shall down-select the options to a lesser number of possibilities.

Observation 3: RAN1 discussed UAI in length without clear outcome on how to use it.

- RAN4 shall try to down-select the potential solution using UAI.

# Discussion on additional skipping schemes

Issue 6-1: Semi-static schemes:

<Way forward>: - Option 1: Follow RAN1 working assumption and consider DCI only. - Option 2: also consider RRC/semi-static approach. - Option 2a: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario. - Option 2b: RAN4 suggest RAN2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps, and UE can have a stable measurement pattern/period. Whether UE supports both approaches can be up to UE capability discussion. - Option 2c: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period. - Option 2d: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. - Option 2e: In addition to DCI solution, it is proposed to introduce semi-static schemes. - Option 3: - RAN4 agrees to introduce indication via MAC-CE command and/or RRC signaling for gap skipping and restriction skipping due to L3 measurements outside gap (if supported). - RAN4 agrees to use indication via DCI for restriction skipping due to L1 measurements (if supported).

From implementation point of view, semi-static approach that NW indicate which occasions to be skipped using RRC and MAC-CE is preferable.

- RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 should de-prioritize FR2-2 and NR-DC.

Proposal 2: RAN4 should deprioritize the scheduling restriction impact on XR traffic for both L1 and L3 measurements.

Proposal 3: Only L1 measurements for LTM within gaps are supported for gap skipping.

Proposal 4: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

Proposal 5: RAN4 shall discuss whether the UE is required to perform positioning and MUSIM measurements while the UE is operating with XR data traffic.

Proposal 6: RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types.

Proposal 7: RAN4 shall keep the number of measurement samples for a frequency layer for L3 measurements unimpacted due to XR traffic.

Proposal 8: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

Proposal 9: RAN4 shall introduce a scaling factor to extend the total measurement delay by a factor of Ntotal/Navailable, where Ntotal is equal to all gap occasions within the total measurements delay for all frequency layers, while Navailable is the number of gap occasions after gap cancellation for the same time window.

Proposal 10: RAN4 shall discuss and define the max number of occasions cancelled during a measurement period.

Proposal 11: RAN4 shall try to down-select the potential solution using UAI.

Proposal 12: RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

# References

- RP-234080, “New WID: XR (eXtended Reality) for NR Phase 3”, MediaTek (Moderator), Dec 2023.

- R4-2420103, “WF on RRM requirements for XR_Ph3”, Orlando, USA, Nov, 2024.

- R1-2405736, “LS on UE assistance information”, Nokia, May, 2024.