# 3GPP TSG-RAN WG4 Meeting # 114						R4-2502100

# Athens, Greece,17th–21st February 2025

Source:		Ericsson

Title:			A-IoT device requirement overview

Agenda item:		7.24.2.2

Document for:		Discussion

# Introduction

In this paper, we present our view on the A-IoT device RF requirements.

# RF requirement for A-IoT UE

## Tx requirement

In WF[3], for device 1, only a few transmitter requirement are relevant to the discussed, namely the backscattering power loss, transmit signal quality (modulation quality requirement) and unwanted emission (SEM, spurious).

For the backscatter power loss, this could be one factor impacted the link budget in RAN1, Table 4.3.2-1 [2] is quoted for example.

Table 4.3.2-1 [2] : Link budget template

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | OOK: 6 dB PSK: 0 dB FSK: Y dB It is applicable for device 1 and 2a. Companies to report and justify their assumptions for Y. Companies to report in row 3D if they assume any additional related loss. |

RAN4 could discuss how this parameter can be modeled and tested. The amount of power backscattered can be described using the concept of a cross-section which is used in radar theory. A scattering cross-section can be defined as the ratio of the power radiated by the device to the power density incident on it.

Assume the power density of the incident radio wave on the device antenna is S

S=Pt∙Gt4πr2 					(1)

Where the Pt is transmission power of a CW signal and Gt is the transmitter antenna gain, r is the distance between test equipment to the device.

The received power on device antenna is Pa relating to the effective antenna aperture:

Pa=λ2∙Gd4π∙S 					(2)

Where the λ is the radio wavelength and Gd is the device antenna gain.

The power backscattered back in the direction of the transmitter is Pbackscatter:

Pbackscatter=K∙Pa∙Gd=4∙Ra2Za+Zc1,c22  ∙ λ2∙Gd24π∙ S 		(3)

Where the Za=Ra+jXa  is complex antenna impedance and Zc1,c2=Rc1,c2+jXc1,c2 is complex chip load in different modulation status. The antenna impedance and chip load are based on Thevenin equivalent circuit for antenna.

The backscatter efficiency of the device 1/2a can be defined as

σ=PbackscatterS=Ra2Za+Zc1,c22  ∙ λ2∙Gd2π 			(4)

The received backscattered signal at the test device:

Preceived=Pbackscatter∙Gr4πr2=Pt∙Gt4πr2 ∙Gr4πr2∙σ 				(5)

With the above measurement procedure, the backscatter efficiency can be derived from the received modulation signal power and transmission power of the CW signal. As the received power only relevant to the sideband of received signal, this signal power corresponding to the differential vector RCS of two different modulation status.

In [4], the modulation efficiency is also defined which is proportional to differential radar cross section in two different modulation status. In our opinion the modulation efficiency or backscatter efficiency can be defined as requirement.

In the test equipment receiver, the received power of modulated signal can be measured and therefore, the modulation efficiency or the backscatter efficiency can be derived.

The modulation efficiency or backscatter efficiency can be derived with proper measurement.

- Proposal 1: Specify the backscatter efficiency requirement for device 1/2a.

For modulation signal quality requirement, the SNR degradation of a signal can be contributed by intrinsic noise from the hardware (e.g the intermodulation product from non-linear PA) and EVM can be measured for legacy system. For the backscatter signal modulation, modulation is done by switching different antenna impedance and therefore, the lowest power for decoding can be tested with a minimum received power using the equation (5), assuming the σ is fixed and all distance and transmit power is known. As the backscattering efficiency is derived from (5) and once σ is known and other parameter in (5) is fixed, so such minimum received power can be derived. So there is dependency between modulation signal quality and backscatter efficiency. If the min received power and a required reader range is specified, the backscatter efficiency can be decided and specified as requirement, if so, no need to specify the modulation signal quality.

The modulation signal quality can depend on the backscatter efficiency if backscatter efficiency should be specified according to a predefined reader range and min power for decoding of D2R signal.

- Proposal 2: discuss whether the modulation signal quality requirement can be depending on backscatter efficiency.

## Rx requirements

In WF [3], the REFSENS, maximum input power and interference rejection to be decided.

For device 1 harvesting the RF energy, it may be useful to discuss whether to have EH sensitivity as one requirement. As the energy harvesting sensitivity may be different with the REFSENS, for D1T1-B, the device can be illuminated with CW or other RF signal but still have a different REFSENS specified and as such, the OTA test can be set up with the knowledge both EH sensitivity and REFSENS.

The EH sensitivity and REFSENS both should be known for the RF test.

According to the coexisting assumption in SI phase, the CW node is co-located with reader, implying the EH sensitivity also play role in the reader coverage and therefore, RAN4 needs to discuss whether a EH sensitivity should be specified. Our opinion is that if coexisting simulation is not changed, EH sensitivity should be specified otherwise the system may not work as expected.

Energy harvesting sensitivity relevant to the reader range for D1T1-B deployment

- Proposal 3: Specify both the REFSENS and EH sensitivity.

For the REFSENS requirement, the SNR of the R2D signal should be agreed. This may involve the LLS simulation. As this is first meeting and R2D signal design is not clear, RAN4 can wait Ran1 further progress on the simulation assumption discussion. Meanwhile, RAN4 can discuss if the 24 dB noise figure in Table 6.6.2.3-2 in TR 38.769 can be working assumption for further REFSENS discussion.

- Proposal 4: Wait RAN1 progress for the LLS simulation assumption discussion.

- Proposal 5: Confirm the 24 dB to be the noise figure of device 1.

Table 6.6.2.3-2: A-IoT device RF parameters

| A-IoT device parameters | Device 1 | Device 2a |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | 0 |
| A-IoT device reflection （backscatter）loss (dB) | OOK: -6 dB | OOK: -6 dB |
| A-IoT device power gain of reflection amplifier (dB) | N/A | 10(M) |
| A-IoT Device receiver sensitivity (dBm) Use this value to determine whether device can camp on the cell. | -36 | -45 |
| A-IoT device noise figure (dB) | 24 | 20 |
| Guard band | 0PRB | 0PRB |

For the maximum input power, legacy requirement relates to the saturation of the receiver RF chain. For device 1, the diode will be used and it is a question if the square-law relation between input power and output voltage can be kept within a pre-defined range. As the square law is assumed in the ED modeling in LLS, if the component deviates this behavior beyond certain input power, it should be specified as a requirement. Figure 1 gives one example of one diode from one manufacture and shows the square law only applies within a certain range of input power. RAN4 can start to collect this parameter before specify the maximum input power.

- Proposal 6: Discuss the necessity of specifying the maximum intput power by collecting the square law appliance range.

Figure 1: detector output characteristic

For the interference rejection, this may be referred to the intermodulation requirement. As the RF ED has no inband selection, we donot think the IM requirement relevant to device 1. In other words, device cannot reject the high power CW and coexisting NR signal in other channel.

There is no IM rejection capability in RF ED device.

- Proposal 7: no need to specify the intermodulation requirement for device 1.

# Conclusions

In this contribution, we present our overview on the A-IoT device RF with below proposal and observations:

Observation 1 The modulation efficiency or backscatter efficiency can be derived with proper measurement.

Observation 2 The modulation signal quality can depend on the backscatter efficiency if backscatter efficiency should be specified according to a predefined reader range and min power for decoding of D2R signal.

Observation 3 The EH sensitivity and REFSENS both should be known for the RF test.

Observation 4 Energy harvesting sensitivity relevant to the reader range for D1T1-B deployment

Observation 5 There is no IM rejection capability in RF ED device.

- Proposal 1: Specify the backscatter efficiency requirement for device 1/2a.

- Proposal 2: discuss whether the modulation signal quality requirement can be depending on backscatter efficiency.

- Proposal 3: Specify both the REFSENS and EH sensitivity.

- Proposal 4: Wait RAN1 progress for the LLS simulation assumption discussion.

- Proposal 5: Confirm the 24 dB to be the noise figure of device 1.

- Proposal 6: Discuss the necessity of specifying the maximum intput power by collecting the square law appliance range.

- Proposal 7: no need to specify the intermodulation requirement for device 1.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- TR 38.769

- R4-2420470, WF on RF requirements for A-IoT, Huawei

- Pavel V. Nikitin and K. V. S. Rao, “Theory and measurement of backscattering from RFID tags” IEEE Antennas and Propagation Magazine (Volume: 48, Issue: 6, December 2006)

- Christoph Degen, Patrick Bosselmann, “On Complex Radar Cross Section and Backscatter Modulation Efficiency in RFID Systems”, 2020 14th European Conference on Antennas and Propagation (EuCAP)