3GPP TSG-RAN WG4 Meeting #114	R4-2502157

Athens, Greece, February 17th – 21st 2025

Agenda Item:	7.23.2.2

Source:	Ericsson

Title:	Impact on SBFD BS RF TX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF TX requirements for SBFD slots.

Discussion on existing BS RF RX requirements for SBFD slots, potentially new requirements for SBFD slots, and SBFD general aspects are detailed in separate submissions [1-3].

# 2	Discussion

Unwanted emissions

# Issue 2: Unwanted emissions Agreement: - FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

In the approved WF [4] in RAN4#113, it was agreed to further study whether to introduce the additional unwanted emission requirement for FR1 WA BS. The proposals involve a new co-existence scenario: a SBFD-capable BS co-exists with another operator’s SBFD-capable BS. The proponents recommend that RAN4 consider more stringent ACLR and/or OBUE requirements to ensure the co-existence of these two SBFD-capable BSs. In our view, the co-existence of two SBFD-capable BSs is a corner case based on the assumption that one band is dedicated to the operation of SBFD capable BSs and there is no legacy TDD BSs operation. More importantly, this new scenario was not considered in the co-existence study performed in the SI phase, if there is any additional unwanted emission requirement to be introduced in the specification in WI, it should first have been studies and identified as a potentially new requirement in SI and shall not be embedded in existing requirement discussions. Therefore, we believe it is unnecessary to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

RAN4 do not introduce the new ACLR and/or OBUE requirements for FR1 WA BS.

Measurement of Averaged EVM

# Issue 3: Joint measurement for normal DL symbols/slots and SBFD symbols/slots Agreement: - FFS on the Proposal 3 from R4-2419221.

In the approved WF [4] in RAN4#113, companies are encouraged to further study the Proposal 3 from R4-2419221, which propose to change the granularity from slot to symbol.

In our view, changing the time granularity from slot to symbol would lead to significant variation in symbol EVM. Even when transmitting at lower power, the resulting EVM remains substantial. Therefore, we cannot support the modification proposed in Proposal 3 of R4-2419221 [5].

Additionally, as outlined in the approved WF R4-2416585 [6] from the RAN4#112-bis meeting, the agreement, as shown below, is to separately measure the averaged EVM for SBFD and non-SBFD, and the averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The solution is to divide all symbols into SBFD symbols and non-SBFD symbols, and then spread non-SBFD symbols across multiple frames. For example, for the case of three SBFD symbols and one non-SBFD symbol, take four frames for averaging non-SBFD symbols, instead of taking one frame. This approach preserves the same number of symbols for non-SBFD as is done today.

## 1.2. Joint measurement for normal DL symbols/slots and SBFD symbols/slots Agreement: - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots.

Below is the proposed text proposal, which is based on the existing version in TS 38.104 [7].

The following text proposal is adopted for averaged EVM calculation:

- ---------------------------------------------------------Start of Text Proposal -------------------------------------------------------

# B.7	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval. For SBFD, only resource blocks allocated to downlink subbands shall be measured.

For FDD the averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step.

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

-	Thus   is calculated using in the expressions above and  is calculated using  in the  calculation.

-	Thus we get:

EVM=maxEVMframe,l,EVMframe,h

For TDD or SBFD, let NdlTDD be the number of slots with downlink symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

- ---------------------------------------------------------End of Text Proposal---------------------------------------------------------

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 do not introduce the new ACLR and/or OBUE requirements for FR1 WA BS.

Proposal 2	The following text proposal is adopted for averaged EVM calculation:

---------------------------------------------------------Start of Text Proposal -------------------------------------------------------

# B.7	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval. For SBFD, only resource blocks allocated to downlink subbands shall be measured.

For FDD the averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step.

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

-	Thus   is calculated using in the expressions above and  is calculated using  in the  calculation.

-	Thus we get:

EVM=maxEVMframe,l,EVMframe,h

For TDD or SBFD, let NdlTDD be the number of slots with downlink symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

---------------------------------------------------------End of Text Proposal---------------------------------------------------------

# References

R4-2502158, Impact on SBFD BS RF RX requirements, Ericsson

R4-2502156, Potentially new SBFD BS RF requirements, Ericsson

R4-2502155, Discussion on SBFD general aspects, Ericsson

R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF

R4-2419221, Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS

R4-2416584, Way forward for [112bis][306] NR_duplex_evo_General

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP