3GPP TSG-RAN4 Meeting #114	R4-2502177

# Athens, Greece, 17th - 21st February, 2025

Agenda item:		7.14.2.2

Source:				Keysight Technologies UK Ltd, Samsung, Nokia, Verizon Wireless, AT&T, Mediatek, ZTE, Ericsson, ETS-Lindgren, Rohde&Schwarz, OPPO, Xiaomi, CAICT

Title:					On Circular Polarization for FR1 NTN OTA Conformance Testing

Document for:		Approval

# Introduction

This contribution addresses how to resolve the UE antenna polarization based on the Rel-19 OTA WI in [1], i.e., whether conformance test shall utilize circular polarization (CP) testing or the conventional linear polarization (LP) testing.

# Discussion

The RAN4#112 WF [2] included a discussion topic for test equipment and test system vendors related to CP measurements, i.e.,

Issue 3-3-4: How to resolve UE antenna circular polarization issue Agreements: - FFS RAN4 how to accommodate different UE polarization implementations, i.e., either circular polarization or cross polarization.

Two contributions in RAN4#112-bis addressed this topic [3], [4] with the recommendation not to pursue CP testing further in 3GPP. Based on online and offline discussions, no definite agreements related to the use of CP for NTN OTA conformance testing could be made and the further discussions were deferred to RAN4#113 with the following agreements captured in the WF [5].

Issue 3-3-4: How to resolve UE antenna circular polarization issue Agreements: - RAN4 to further study the need for circular polarization for NR NTN conformance testing - Feedback from OEMs and Satellite Vendors on the need for CP testing and test system impacts to support CP OTA conformance testing from TE/System vendors is requested.

No progress was made in RAN4#113 [6], [7], [8], [9] and further deferred to RAN4#114 [10].

Issue 3-2-4: How to resolve UE antenna circular polarization issue Proposals - Proposal 1: Use circular polarisation test antennas to assess the performance of the UE. This enables us to characterise the exact loss seen by the UE when operating in a system that utilises circular polarisation antennas, as all satcom systems do. - If we ignore circular polarisation testing at this stage, it may preclude the ability of an operator to leverage XPD to increase the capacity of their system. - Proposal 2: The NR FR1 NTN conformance testing should focus on LP rather than CP assessments. - WF - FFS

This contribution is a continuation of [9].

# Use of CP OTA Testing so Far

As illustrated in Figure 1, circular polarization CP requires the definition of the handedness, i.e., clock vs counter-clock wise or left-handed circular polarization (LHCP) vs right-handed circular polarization (RHCP).

Figure 1: Left and Right Handedness of Circular Polarization

The assessment of polarization is pretty common when just the antenna sub-component of DUTs is tested in antenna measurement systems. In such systems, with an example system visualized in Figure 2, the signal paths between the input of the AUT to the input/outputs of the Vector Network Analyzer (VNA) ports connected to the measurement probe ports, must be determined separately for each of the probe’s orthogonal polarizations, q and f. Each path can readily be characterized for magnitude and phase with the VNA using full 2-port calibrations. The corresponding S31 and S21 passive/non-signaling measurements of the AUT can be performed simultaneously with the VNA and the CP components, axial ratio, XPDs, etc. can then be determined using well known equations.

Figure 2: Sample antenna measurement system to characterize antennas

Observation 1: Circular polarization testing of just the antenna sub-system is relatively common using antenna measurement systems and passive/non-signaling testing.

# Use of CP from a System Test System Perspective

The use of circular polarization in OTA test systems for NR FR1 NTN conformance testing, specifically related to test system aspects, is discussed next. It should be noted that conventional linear polarization (LP) has been employed in traditional OTA conformance test systems for SISO and MIMO across all frequency ranges, as well in other radiated test systems like EMC and RSE.

Leaving aside the test system implementation to introduce circularly polarized DL signals for now, this section is focused on whether a circularly polarized antenna (RHCP or LHCP) used for both UL and DL can be used as measurement antenna. It should be noted that circular polarization in wireless telecommunications is very untypical on the UE side; the use of linear polarizations is much more predominant.

To assess the UE DL performance for circular polarization reception, the UE has to be exposed to the corresponding CP (LHCP, RHCP) polarization, i.e., the performance cannot be assessed with post-processing of two LP measurements, i.e., q and f, since DL signaling measurements do not readily provide phase information. On the other hand, to assess the UE UL performance for CP transmission, the UE does not necessarily have to be exposed to the corresponding CP polarization as the performance could be assessed with post-processing of two LP measurements (q and f) provided that the phase difference between the two q and f UL paths has been measured, e.g., using a signal analyzer or a scope, and that the measurement paths have been calibrated for magnitude and phase.

Observation 2: To assess the UE DL performance for reception of a circularly polarized wave, the UE has to be exposed to the corresponding CP (LHCP, RHCP) polarization

Observation 3: To assess the UE UL performance for circular polarization reception, the performance could be assessed with post-processing of two linear-polarization (q and f) measurements

Observation 4: To assess the UE UL performance for circular polarization reception using post-processing of two-linearly polarized measurements, more complex test system measurement equipment is needed to assess the phase difference between UL signals and a more complex calibration will have to be performed.

Introducing circular polarization in the DL can be achieved by either

- Using circularly polarized antennas, such as (conical) spiral or helix antennas

- Using dual linearly (orthogonal) polarized antennas with input signals that are in phase quadrature

Native, CP antennas generally support just a single polarization (LHCP or RHCP) which is problematic as all other NR FR1 OTA conformance test cases [6], [12] require the use of linear polarizations to be introduced in the DL. Such native, CP antennas cannot readily support switching between CP and LP polarizations, i.e., conformance test systems supporting NR FR1 OTA test cases would have to rely on mechanical switching between CP and LP measurement antennas which is possible but highly undesirable as the system and all system paths would require a new calibration from scratch which can take up to a day depending on the calibration approach employed by the lab.

Observation 5: The use of antennas with native circular polarization is not suitable for FR1 OTA conformance test cases as the mechanical switching between CP (LHCP, RHCP) and LP probe antennas requires calibrations performed from scratch for every probe change.

The use of dual linearly polarized antennas with orthogonal polarizations initially seems the more practical approach if CP had to be introduced for NR NTN conformance test cases. Suitable implementations would be dual polarized horn or Vivaldi antennas. Either would require the input signals to be controlled individually in amplitude (due to antenna gain differences in the two respective polarizations) and in phase, i.e., achieve the relative +90o or -90o phase differences.

Observation 6: The use of dual, linearly polarized with orthogonal polarizations, e.g., horn or Vivaldi antennas, requires frequency-dependent amplitude compensations and each polarization to be fed in phase quadrature.

If we assume that the gain difference between two polarizations is insignificant, the DL signal could be split in front of the antenna inputs and a ±90o phase shifter needs to be inserted at one input to support both LHCP and RHCP. Adjustable 90o and -90o phase shifters are readily, commercially available. However, such phase shifters would introduce additional insertion loss and reduction of SNR. Additionally, the operation of CP (LHCP, RHCP) and LP polarization in the same test system would require further complexities in the switching network and thus impact dynamic range even more. This is further illustrated in Figure 3. In the top, the conventional OTA SPDT switching of the two orthogonal probe polarizations (q and f) is shown which allows both polarizations to be measured one by one sequentially. In the bottom, a sample extension of the required switching and splitter network is displayed that allows the sequential operation of LP (q and f), RHCP, and LHCP. Clearly, a significant reduction in dynamic range can be observed; additionally, the required HW upgrades of existing systems would be significant as well, especially for the multi-probe SISO chambers.

Figure 3: Sample extension of the DL paths and switching/splitter network to support LP (q and f), RHCP, and LHCP operation in the DL

Observation 7: The use of discrete 90o phase shifters/polarizers/splitter inserted in front of dual-linearly polarized antennas to generate CP is not recommended given the significant impact in dynamic range and the significant upgrades required for existing OTA systems.

An alternate approach would be to individually adjust the magnitudes and phases of the DL signals connected to the dual-linearly polarized antenna at the output of the gNB emulator. In order to feed the antenna in quadrature, the phase differences between gNB emulator ports have to be controllable individually and accurately. However, such adjustability has never been made a requirement in 3GPP for UE RF testing and therefore not considered in gNB emulators architectures so far. Performing phase calibrations before and/or during each UE RF test is impractical as well.

Observation 8: The need for fixed phase differences between gNB emulator ports has not been made a requirement in 3GPP for UE RF testing and therefore has not yet been taken into account for gNB emulator architectures.

Observation 9: Performing phase calibrations before or during each UE RF test is impractical

The three approaches to upgrade/extend existing LP OTA systems to support CP as well are summarized in Table 1.

Table 1: Overview of approaches in order to generate LP and CP in the DL

| Probes | Switching Network | gNB Output Phase Control | Calibration | Concern |
| --- | --- | --- | --- | --- |
| Multiple Probes (changed mechanically between RHCP, LHCP, dual-pol probes) | No changes required compared to legacy OTA Systems | N/A | Requires calibrations performed from scratch for every probe change for LP (q and f), RHCP, LHCP operation | Time-intensive calibrations required which increase overall testing time |
| Single Dual-Pol Probe | Extension of the switching/splitter network required | N/A | Requires LP (q and f), RHCP, LHCP calibrations on a regular basis but not every time polarization is switched | Switching network extension leads to significant insertion losses and degradation in dynamic range and significant HW upgrades |
| Single Dual-Pol Probe |  | For CP operation, phases at input of probe must be in quadrature, i.e., precise phase control at output of gNB | Requires phase calibrations for q and f paths on a very regular basis due to phase drifts in the DL paths | Phase control at output of gNB has never been a requirement, i.e., not supported |

If we assumed that the generation of signals in quadrature at the input of the antenna was feasible in principle, some related testing aspects need to be considered as well. The deviation from the ideal ±90o phase differences at the gNB emulator outputs need to be considered together with the axial ratio, i.e., deviation from the ideal ±90o phase differences, in the far field of the antenna (assuming the antenna was fed in perfect quadrature). Such axial ratio assessment might potentially also be necessary in off-axis directions. More importantly, the relative phase differences for two orthogonal polarizations inside the quiet zone need to be determined at various discrete locations caused by reflections/standing waves from support structures, positioners, absorbers, the deviation in AR of the feed antenna (especially off-axis directions), etc. Additionally, the relative amplitude differences for two orthogonal polarizations inside the quiet zone need to be determined as well. All of the above degradations and coupling between polarizations need to be considered with an additional MU element related to UEs receiving downlink signals with an axial ratio of less than 1 (an axial ratio of 1 which corresponds to circular polarization). These three phase deviation terms (output phase of the gNB emulator, antenna axial ratio variation, and the quality of the quiet zone) could change the intended circular polarization to elliptical polarization. The change from circular to elliptical and even to linear polarization due to changes in phase is illustrated in the Poincare sphere in Figure 4.

Observation 10: New and modified quality of quiet zone procedures are needed for relative phase and amplitude

Observation 11: Additional MU element(s) are needed to account for the relative phase variation (due phase variation of the gNB or in-line components, antenna axial ratio, quality of quiet zone).

Observation 12: An intended circularly polarized signal could become elliptically or even linearly polarized due to cumulative relative phase difference changes anywhere within the quiet zone.

Figure 4: Poincare Sphere and Changes in Polarization due to relative phase differences

Another approach to potentially assess CP performance of a device is by performing a series of magnitude only LP measurements along three different directions within a plane, e.g., Figure 3 of [17]

Suitable approaches to perform three LP measurements were provided in Figure 9 from [17]

Most OTA systems readily support two orthogonal polarizations (q and f) already, but the third LP measurement cannot readily be supported with existing test systems, i.e., test systems cannot support any option of Figure 9 from [17]. One suitable upgrade option of existing OTA systems to support multiple LP measurements within a plane is to install a roll motor behind the probe antenna as illustrated schematically in Figure 5.

Figure 5: Antenna/Probe System to support multiple LP measurements along different directions within a fixed plane.

However, the following concerns can be noted for such upgrade:

- Existing single-probe systems can hardly be retrofitted with a roll motor next or behind the measurement probe given weight and/or space concerns to fit an additional motor on measurement booms or chamber walls. Without an additional positioning system on the probe side, this approach is not feasible

- Multi-probe systems would require N (N equals the number of probes in the mutli-probe systems) roll motors, i.e., one motor for each probe. The same weight/space concerns as for single-probe systems applies but adding N additional motors is also cost prohibitive.

- Performing 3 single-pol measurements means a test time increase of at least 50% compared to LP or RHCP+LHCP testing

Another alternative to the roll motor approach is to place a polarizer in front of the measurement probe at a fixed angle offset from either q or f polarization and to subsequently perform separate q and f measurements to assess the LP component along the plane of the polarizer. However, such polarizer will introduce some additional uncertainties due to losses, scattering and frequency dependent behavior, requires manual intervention to install and remove the polarizer, additional cost (especially for multi-probe systems). The chamber likely requires separate ripple test evaluation given the support structures holding the polarizer which will result in additional costs and chamber downtime.

Regardless, three separate LP measurements merely allow the assessment of the Axial Ratio (AR) and not the handedness of the polarization which is needed to determine TRPRHCP vs TRPLHCP. To determine the handedness requires the measurement probe antenna to be replaced by a pure LHCP and RHCP probe and a single-directional measurement as outlined in Figures 4 and 15 from [17]. As such, proper LHCP and RHCP assessment still require approaches and the corresponding concerns outlined in Table 1.

For the TRS metric, the approaches outlined in [16], [17] are not be applicable for devices that use using receiver switching and/or proprietary algorithms to shift phases between simultaneous dual-receiver measurements, as the algorithms seem to be applicable for static antennas only.

Observation 13: A series of magnitude only LP measurements along three different directions within a plane might be able to assess the AR but the use of CP antennas is still required for RHCP and LHCP assessments

Observation 14: The assessment of AR only using LP measurements along three different directions has several complexity, MU, upgradeability challenges and is not applicable to active antenna systems

Given the many observations and impact on test equipment and MUs, it is proposed not to consider the introduction of circular polarization for NR NTN measurements.

From a system vendor perspective, the NR FR1 NTN conformance testing should focus on LP rather than CP assessments.

Proposal 1: From a test system perspective, the NR FR1 NTN conformance testing should focus on LP rather than CP assessments, see Observation 2 throughObservation 14.

# Use of CP for Smartphone UEs

For conformance/performance testing, i.e., for device certification purposes, CP has not been utilized so far for OTA testing purposes, not even for GNSS OTA DL/sensitivity testing which has been established in CTIA for many years [13][14]. Assisted-GNSS (A-GNSS) or standalone GNSS operation assumes transmission of CP from satellite vehicles; however, as discussed below, smartphone OEMs typically do not integrate native CP antennas but rather linearly-polarized (LP) antennas. An example OTA test system/setup is shown in Figure 6. Here, the two orthogonal polarizations of the measurement probe are connected sequentially to the output of a GNSS signal generator and for each DL polarization, q and f, and for each measurement grid point, the UE records the C/N information; the eNB/gNB for the A-GNSS operation is connected to the UE via a link antenna to improve sensitivity and to reduce the time to obtain fixes. Subsequent linearization measurements and a sensitivity search to map the peak C/N value to an EIS value then allow an assessment of the TRS performance.

Figure 6: Sample OTA system for A-GNSS measurements

Observation 15: A-GNSS or standalone GNSS OTA testing in CTIA is based on sequential, linearly-polarized testing with satellite vehicles antennas being CP while UEs include predominantly LP antennas.

While satellite vehicles commonly use circularly polarized antennas, the UEs typically leverage highly integrated antennas for NTN similar to GNSS antennas that are linearly polarized as the integration of CP antennas into handsets generally require more space and/or real estate not readily available for antennas. The CST simulation results of the sample NTN antennas integrated in a typical smartphone UE [15] presented in the Annex confirm this observation.

Observation 16: The NR FR1 NTN antennas in smartphone UEs are linearly polarized.

The intention of the OTA testing is to determine the overall/total performance of the UE which captured appropriately with TRP and TRS metrics for the transmitter and receiver. These full-sphere metrics or partial radiated metrics, still seem to be appropriate for smartphone UE conformance testing which focus on the entire TX/RX transceiver chains all the way to the baseband. The CP performance of the UE is specific to the antenna only, as such, the assessment of the CP performance of the antennas should be left to in-house R&D tests rather than certification tests.

Observation 17: The assessment of the CP performance of the antennas should be left to in-house R&D tests rather than certification tests

It should furthermore be highlighted that earlier studies during the development of the A-GNSS OTA testing in CTIA showed that there was no predominant polarization at the DUT antenna in challenging environments, e.g., indoor, blocking scenarios, scattered environments, etc., where REFSENS applies even with a CP transmission.

Observation 18: Studies during the development of the A-GNSS OTA testing in CTIA showed that there was no predominant polarization at the DUT antenna in challenging environments

Additionally, the integration of a CP antenna into a smartphone UE is not feasible for omnidirectional coverage. Assume the satellite is using RHCP, a smartphone with a CP antenna orientated with good RHCP performance towards the satellite (forward direction) achieves good polarization match. However, when the UE is held in the opposite direction with the back side of the antenna pointing towards the satellite, the antenna polarization is typically reversed (RHCP  LHCP) and consequently would not be able to communicate with the satellite due to the polarization mismatch.

Observation 19: The integration of a CP antenna into a smartphone UE is not feasible for omnidirectional coverage as the antenna typically reverses polarization in the forward vs backward direction.

From an OEM/smartphone UE perspective, the NR FR1 NTN conformance testing should focus on LP rather than CP assessments.

Proposal 2: From an OEM/smartphone UE perspective, the NR FR1 NTN conformance testing should focus on LP rather than CP assessments, see Observation 15 through Observation 19.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: Circular polarization testing of just the antenna sub-system is relatively common using antenna measurement systems and passive/non-signaling testing.

Observation 2: To assess the UE DL performance for reception of a circularly polarized wave, the UE has to be exposed to the corresponding CP (LHCP, RHCP) polarization

Observation 3: To assess the UE UL performance for circular polarization reception, the performance could be assessed with post-processing of two linear-polarization (q and f) measurements

Observation 4: To assess the UE UL performance for circular polarization reception using post-processing of two-linearly polarized measurements, more complex test system measurement equipment is needed to assess the phase difference between UL signals and a more complex calibration will have to be performed.

Observation 5: The use of antennas with native circular polarization is not suitable for FR1 OTA conformance test cases as the mechanical switching between CP (LHCP, RHCP) and LP probe antennas requires calibrations performed from scratch for every probe change.

Observation 6: The use of dual, linearly polarized with orthogonal polarizations, e.g., horn or Vivaldi antennas, requires frequency-dependent amplitude compensations and each polarization to be fed in phase quadrature.

Observation 7: The use of discrete 90o phase shifters/polarizers/splitter inserted in front of dual-linearly polarized antennas to generate CP is not recommended given the significant impact in dynamic range and the significant upgrades required for existing OTA systems.

Observation 8: The need for fixed phase differences between gNB emulator ports has not been made a requirement in 3GPP for UE RF testing and therefore has not yet been taken into account for gNB emulator architectures.

Observation 9: Performing phase calibrations before or during each UE RF test is impractical

Observation 10: New and modified quality of quiet zone procedures are needed for relative phase and amplitude

Observation 11: Additional MU element(s) are needed to account for the relative phase variation (due phase variation of the gNB or in-line components, antenna axial ratio, quality of quiet zone).

Observation 12: An intended circularly polarized signal could become elliptically or even linearly polarized due to cumulative relative phase difference changes anywhere within the quiet zone.

Observation 13: A series of magnitude only LP measurements along three different directions within a plane might be able to assess the AR but the use of CP antennas is still required for RHCP and LHCP assessments

Observation 14: The assessment of AR only using LP measurements along three different directions has several complexity, MU, upgradeability challenges and is not applicable to active antenna systems

Observation 15: A-GNSS or standalone GNSS OTA testing in CTIA is based on sequential, linearly-polarized testing with satellite vehicles antennas being CP while UEs include predominantly LP antennas.

Observation 16: The NR FR1 NTN antennas in smartphone UEs are linearly polarized.

Observation 17: The assessment of the CP performance of the antennas should be left to in-house R&D tests rather than certification tests

Observation 18: Studies during the development of the A-GNSS OTA testing in CTIA showed that there was no predominant polarization at the DUT antenna in challenging environments

Observation 19: The integration of a CP antenna into a smartphone UE is not feasible for omnidirectional coverage as the antenna typically reverses polarization in the forward vs backward direction.

Proposal 1: From a test system perspective, the NR FR1 NTN conformance testing should focus on LP rather than CP assessments, see Observation 2 throughObservation 14.

Proposal 2: From an OEM/smartphone UE perspective, the NR FR1 NTN conformance testing should focus on LP rather than CP assessments, see Observation 15 through Observation 19.

# References

- RP‑240841, New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3, vivo, CAICT, RAN meeting #103, Mar 2024

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo, 3GPP TSG-RAN WG4 Meeting #112, August 2024

- R4-2415268, on NTN OTA test, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #112bis, October 2024

- R4-2416480, On NTN OTA Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #112bis, October 2024

- R4-2416590, Way Forward for [112bis][323] TRP_TRS_Ph3, vivo, 3GPP TSG-RAN WG4 Meeting #112bis, October 2024

- R4-2418826, OTA Conformance Testing with Circular Polarisation, Inmarsat, Viasat, 3GPP TSG RAN WG4 #113, November 2024

- R4-2417591, on NTN OTA tests, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #113, November 2024

- R4-2417757, On NTN TRP TRS test methodology, Apple, 3GPP RAN4 Meeting #113, November 2024

- R4-2419396, On Circular Polarization for FR1 NTN OTA Conformance Testing Keysight Technologies UK Ltd, Samsung, Nokia, Verizon Wireless, AT&T, Mediatek, ZTE, Ericsson, ETS-Lindgren, Rohde&Schwarz, Apple, OPPO, Xiaomi, CAICT, 3GPP TSG-RAN4 Meeting #113, November 2024

- R4-2419808, WF for [113][325] TRP_TRS_Ph3, vivo, 3GPP TSG-RAN WG4 Meeting #113, November 2024

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

- TS 38.161, User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios

- CTIA 01.20, Test Methodology, SISO (Single Input Single Output), Anechoic Chamber

- CTIA 01.51, Wireless Technology, Location Based Technologies

- R4-2416480, On NTN OTA Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #112bis, October 2024

- Improved Measurement Method of Circularly-Polarized Antennas Based on Linear-Component Amplitudes, D. Wang, et al, Open Journal of Antennas and Propagation, Vol.5, pp.36-45, March 2017, DOI: https://doi.org/10.4236/ojapr.2017.51004

- Determining Antenna Polarization Using Amplitude-Only Measurements With Linear-Polarized Antenna in Three Positions, R. Zentner, et al, IEEE Access, 2024, Vol.12, pp.136542-136564, DOI: 10.1109/ACCESS.2024.3463477

# Annex: Smartphone UE Simulations of Sample NR FR1 NTN antennas

This annex is investigating sample antenna patterns for smartphone UEs simulated in CST Studio Suite. Here, a full smartphone model including most components was investigated first with the device held in the hand and is shown in Figure 7

Figure 7: CST Studio Suite smartphone model in hand right (HR) talk condition

Two different antenna locations for NTN antennas were investigated, i.e., one at the top (with frequency ~1.6 GHz, n255) and one on the side (with frequency range ~2.1 GHz, n256) as further illustrated in Figure 8.

Figure 8: NTN antennas investigated on top (left) and on side (right)

The simulated antenna patterns are shown in Figure 9.

Figure 9: NTN antennas patterns for antenna location on top (left) and on side (right)

From the patterns shown in Figure 9, it can be observed that the simulated antennas yield fairly omnidirectional patterns even in the HR talk condition.

The CP antenna patterns, plotted separately for LHCP and RHCP, are shown in Figure 10 for the NTN antenna integrated on the top of the UE and in Figure 11 for the antenna integrated on the side.

Figure 10: NTN CP antennas patterns for antenna location on top. LHCP on left, RHCP on right

Figure 11: NTN CP antennas patterns for antenna location on side. LHCP on left, RHCP on right

These results show that the antenna patterns are indeed predominantly linearly polarized.

Figure 12: NTN antennas pattern axial ratios (15dB dynamic range) for antenna location on top (left) and on side (right)