3GPP TSG-RAN WG4 Meeting #114	R4-2502180

Athens, Greece, February 17 – 21, 2025

Agenda Item:	7.17.2.1

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The objective of R19 NR RRM Phase-5 work item is captured below in the WID [1]. In this paper, we continue the discussion on the first objective, particularly on optimizing Rx beam sweeping factor for L3 measurement delay reduction when UE support multiple-Rx simultaneous reception.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In the last meeting, RAN4 discussed the conditions to apply the enhancement of R19 L3 measurement delay reduction (SSB based) in FR2-1 for UE supporting multiple-Rx simultaneous reception. It was agreed that the signal quality of the serving cell can be used as the triggering condition to activate/deactivate multi-Rx for faster beam sweeping in L3 measurement as captured in the WF [2].

Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19.

Note that, given the agreed scenarios (1/2/3/4/5/7) in RAN4#112 meeting as captured below [3], the above condition will only apply to scenarios (1/2/3/4). It does not apply to scenarios (5/7), i.e., handover and RRC Re-establishment, where multi-Rx is always activated for those events if supported by the UE.

- Scenario 1: SSB based Intra-frequency measurement without MG,

- Scenario 2: SSB based Intra-frequency measurement with MG,

- Scenario 3: SSB based Inter-frequency measurement without MG,

- Scenario 4: SSB based Inter-frequency measurement with MG,

- Scenario 5: Handover

- Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

Observation 1: The agreed triggering conditions to activate/deactivate multi-Rx for L3 measurement only applies to scenarios (1/2/3/4).

UE power consumption due to activating multiple panels for simultaneous DL reception is an important aspect to discuss and consider in RAN4. Even if UE fulfills the condition to enable FR2-1 SSB based L3 measurement delay reduction by activating multi-Rx, UE should also have the capability to indicate to the NW whenever UE needs to deactivate multi-Rx, for example, due to overheating resulting from activating multiple panels for simultaneous reception for long time. Note that such indication was also introduced in R18 multi-RX as shown in 38.331 spec below, wherein UE can indicate “single” to deactivate multi-Rx or “multiple” to re-enter multi-Rx mode after sending deactivation indication. Therefore, similar indication can be introduced for R19 L3 measurement delay reduction for UE supporting multi-Rx simultaneous reception.

multiRx-PreferenceFR2 Indicates the UE's preference on single FR2 Rx operation to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2..

Proposal 1: RAN4 to use same principle as R18 UAI indication for multi-Rx to deactivate/activate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

Since RAN4 agreed on the applicability of R19 L3 measurement delay reduction for UE supporting multi-Rx as given in the scenarios (1/2/3/4/5/7) above, RAN4 need to discuss how to apply beam sweeping optimization on the agreed scenarios. It is noted that:

- For scenarios (1/2/3/4), the beam sweeping parameter used to scale L3 measurements for intra/inter-frequency (w/o gap) in FR2 is given by Mpss/sss_sync and Mmeas_period for PSS/SSS detection and measurement period, respectively.

- For scenario 5, the beam sweeping parameter used to scale handover (HO) delay when the target cell is unknown in FR2 is given by N in the Tsearch.

- For scenario 7, the beam sweeping parameter used to scale RRC Re-establishment/RRC Connection Release is included in Tidentify_intra and Tidentify_inter delays.

Therefore, for UE supporting multi-Rx in R19, L3 measurement delay reduction for the aforementioned parameters needs to be enhanced.

Proposal 2: The following delay requirements/parameters of L3 measurement can be reduced by optimizing beam sweeping factor:

- For scenarios (1/2/3/4): Mpss/sss_sync and Mmeas_period used in intra/inter-frequency (w/o gap) in FR2

- For scenario (5): N in Tsearch used in handover delay when target cell is unknown in FR2

- For scenario (7): Tidentify_intra and Tidentify_inter used to scale RRC Re-establishment/RRC Connection Release

The solution to optimize beam sweeping factor for the aforementioned L3 measurements scenarios is by considering a smaller RX beam sweeping factor X than the legacy factor 8 (e.g., 6 or 4). In intra/inter-frequency measurements, for instance, instead of Mpss/sss_sync =40 when UE support FR2-1 power class 1 or 5, it can become 20 for UE supporting multi-Rx simultaneous reception. If RAN4 decides to use a single value for the reduced beam sweeping factor then we support either 6 or 4, otherwise if RAN4 decides to use multiple candidate values then (6 and 4) can be supported as UE capability

Proposal 3: On the solution to achieve reduced beam sweeping factor (BSF) as UE capability, RAN4 to decide to use either:

- Single value for reduced BSF (e.g., 6 or 4)

- Multiple candidate values for BSF (e.g., 6 and 4)

# Summary

In this contribution, the following observations and proposals are concluded:

Observation 1: The agreed triggering conditions to activate/deactivate multi-Rx for L3 measurement only applies to scenarios (1/2/3/4).

Proposal 1: RAN4 to use same principle as R18 UAI indication for multi-Rx to deactivate/activate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

Proposal 2: The following delay requirements/parameters of L3 measurement can be reduced by optimizing beam sweeping factor:

- For scenarios (1/2/3/4): Mpss/sss_sync and Mmeas_period used in intra/inter-frequency (w/o gap) in FR2

- For scenario (5): N in Tsearch used in handover delay when target cell is unknown in FR2

- For scenario (7): Tidentify_intra and Tidentify_inter used to scale RRC Re-establishment/RRC Connection Release

Proposal 3: On the solution to achieve reduced beam sweeping factor (BSF) as UE capability, RAN4 to decide to use either:

- Single value for reduced BSF (e.g., 6 or 4)

- Multiple candidate values for BSF (e.g., 6 and 4)

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple.