3GPP TSG-RAN WG4 Meeting # 114	R4-2502183

Athens, GR, Feb 17 – Feb 21, 2025

Agenda Item: 7.22.3.2

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for other RRM requirements

Document for: Discussion

# Introduction

RAN#105 agreed on the revised WID “NR_MIMO_Ph5” [1]. According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WID [1] is divided into two items to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “Other RRM requirements”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul.

- Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

## 2.1 UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

In the last meeting, RAN4 reached the following agreements as captured in the WF [2]:

Issue 2-1: On the simulation assumption for CJT calibration Agreement: - Agree with the following table for delay/frequency offset reporting. FFS on phase offset. Parameter; Value. Waveform; OFDM. Duplex, SCS; - FDD, 15kHz SCS - TDD, 30kHz SCS. Carrier Frequency; 3.5 GHz. Channel Model; TDLC300-10. Delay Spread; 300ns. antenna configuration; 1X2. TRP number; 2. TRS Bandwidth; 48RB. Numerology; 14 OFDM symbol per slot. UE speed; 3km/h. SNR; -3dB, 0dB, 3dB. Frequency offset between TRP (only for freq. offset simulations); 0.1ppm, 0.2ppm 2nd priority: 0.02 ppm, 0.05ppm. Delay difference between TRP (only for delay offset simulations); 0.5CP, CP. Number of samples (TRS resource sets) for measurement; 1, 3, 5 2nd priority: 2, 4, 10. CSI-RS config; Cjtc-d and cjtc-f: periodic TRS (‘CSI-RS for tracking’). CSI-RS periodicity (TRS); 20ms / 40 slots for 30kHz; 20 slots for 15kHz. CSI-RS offset; Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz. Issue 2-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration? Way forward: - Option 1: Define accuracy requirements. - Option 2: FFS. Issue 2-3: Whether report mapping tables should be specified by UE reporting enhancement for CJT calibration? Way forward: - Option 1: Define mapping tables for frequency/delay/phase offset reporting in performance part.

RAN1 has already specified some mechanism of UE reporting enhancement for CJT deployments including timing misalignment, frequency offset and phase offset reporting. In our understanding, these are new feature for reporting in terms of the compensation of delay, frequency, and phase offset.

Based on RAN1 agreement for CJT calibration reporting, RAN4 to discuss whether there’s any RRM impact based on feasibility study. Table 1 shows the simulation parameters based on simulation assumption agreed in last RAN4 meeting.

- To estimate TO between two TRPs, the FO is set to zero and the TO setting is either 0.5 CP or one CP. This is a kind of delay difference between two TRPs.

- To estimate FO, the TO is set to zero and the FO setting between two TRPs is either 350Hz or 700Hz in terms of 0.1 ppm or 0.2 ppm frequency offset assumption.

We will provide simulation result based on 24 test set shown in Table 1 for different SCS, SNR, TO and FO setting. In the end, RAN4 could decide whether to define the requirements for CJT calibration report according to the simulation results from companies.

Table 1. Simulation parameters for timing and frequency offset estimation

| Test set | SCS | SNR | Timing offset (TO) setting b/w two TRPs | Frequency offset (FO) setting b/w two TRPs (Unit: Hz) |
| --- | --- | --- | --- | --- |
| #1 | 15KHz | -3 | 0.5CP | 0 |
| #2 | 15KHz | 0 | 0.5CP | 0 |
| #3 | 15KHz | 3 | 0.5CP | 0 |
| #4 | 15KHz | -3 | CP | 0 |
| #5 | 15KHz | 0 | CP | 0 |
| #6 | 15KHz | 3 | CP | 0 |
| #7 | 15KHz | -3 | 0 | 350 |
| #8 | 15KHz | 0 | 0 | 350 |
| #9 | 15KHz | 3 | 0 | 350 |
| #10 | 15KHz | -3 | 0 | 700 |
| #11 | 15KHz | 0 | 0 | 700 |
| #12 | 15KHz | 3 | 0 | 700 |
| #13 | 30KHz | -3 | 0.5CP | 0 |
| #14 | 30KHz | 0 | 0.5CP | 0 |
| #15 | 30KHz | 3 | 0.5CP | 0 |
| #16 | 30KHz | -3 | CP | 0 |
| #17 | 30KHz | 0 | CP | 0 |
| #18 | 30KHz | 3 | CP | 0 |
| #19 | 30KHz | -3 | 0 | 350 |
| #20 | 30KHz | 0 | 0 | 350 |
| #21 | 30KHz | 3 | 0 | 350 |
| #22 | 30KHz | -3 | 0 | 700 |
| #23 | 30KHz | 0 | 0 | 700 |
| #24 | 30KHz | 3 | 0 | 700 |

Proposal 1: RAN4 to consider the simulation parameters on Table 1 as one of candidates to provide the simulation result of timing and frequency offset estimation.

Based on the Table 1, we provide simulation result of TO estimation (TOE) and FO estimation (FOE) on Table 2 and Table 3. The TOE and FOE is the measured TO and FO from two TRS resource set between two TRPs. The TOE span is calculated by maximum absolute value of TOE (CDF 5%, CDF 95%) to the bias (TO setting between two TRPs). Therefore, TOE span is a kind of measured accuracy for timing offset between two TRPs. We average the number of samples (1, 3, 5) and show the result in below.

Table 2. Simulation result for timing offset estimation (TOE)

| Test set | SCS | SNR | Timing offset (TO) setting b/w two TRPs | TOE span bias to TO setting (CDF@5%-95%, unit: ns) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | #samples |  |  |  |
|  |  |  |  | #1 | #3 | #5 |
| #1 | 15KHz | -3 | 0.5CP | 524.56 | 342.56 | 327.56 |
| #2 | 15KHz | 0 | 0.5CP | 309.83 | 206.14 | 173.37 |
| #3 | 15KHz | 3 | 0.5CP | 168.04 | 126.60 | 103.43 |
| #4 | 15KHz | -3 | CP | 570.43 | 365.72 | 305.80 |
| #5 | 15KHz | 0 | CP | 271.56 | 181.29 | 179.89 |
| #6 | 15KHz | 3 | CP | 181.11 | 118.97 | 117.63 |
| #13 | 30KHz | -3 | 0.5CP | 269.36 | 170.89 | 156.11 |
| #14 | 30KHz | 0 | 0.5CP | 146.81 | 101.25 | 93.07 |
| #15 | 30KHz | 3 | 0.5CP | 122.00 | 79.12 | 76.30 |
| #16 | 30KHz | -3 | CP | 273.09 | 184.44 | 152.45 |
| #17 | 30KHz | 0 | CP | 159.45 | 118.06 | 108.78 |
| #18 | 30KHz | 3 | CP | 118.18 | 85.96 | 75.01 |

The FOE span is calculated by maximum absolute value of FOE (CDF 5%, CDF 95%) to the bias (FO setting between two TRPs). Therefore, FOE span is a kind of measured accuracy for frequency offset between two TRPs.

Table 3. Simulation result for frequency offset estimation (FOE)

| Test set | SCS | SNR | Frequency offset (FO) setting b/w two TRPs (unit: Hz) | FOE span bias to FO setting (CDF@5%-95%, unit: Hz) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | #samples |  |  |
|  |  |  |  | #1 | #3 | #5 |
| #7 | 15KHz | -3 | 350 | 168.34 | 107.22 | 76.30 |
| #8 | 15KHz | 0 | 350 | 99.84 | 65.79 | 46.59 |
| #9 | 15KHz | 3 | 350 | 70.71 | 43.38 | 32.34 |
| #10 | 15KHz | -3 | 700 | 156.79 | 105.98 | 72.21 |
| #11 | 15KHz | 0 | 700 | 98.34 | 65.10 | 47.93 |
| #12 | 15KHz | 3 | 700 | 66.84 | 43.85 | 33.01 |
| #19 | 30KHz | -3 | 350 | 364.17 | 194.62 | 163.63 |
| #20 | 30KHz | 0 | 350 | 228.90 | 132.68 | 101.18 |
| #21 | 30KHz | 3 | 350 | 143.94 | 84.92 | 65.44 |
| #22 | 30KHz | -3 | 700 | 297.08 | 186.14 | 134.26 |
| #23 | 30KHz | 0 | 700 | 179.32 | 102.40 | 76.00 |
| #24 | 30KHz | 3 | 700 | 123.37 | 70.13 | 51.66 |

Proposal 2: The simulation result of timing offset and frequency offset between two TRPs are provided on Table 2 and Table 3.

The calculation of TOE and FOE span is the maximum absolute value of (CDF 5%, CDF 95%) to the bias (TO and FO setting between two TRPs). This way is similar as the calculation of L1-RSRP accuracy. To define accuracy requirement of TO and FO between two TRPs, the common calculation for TO and FO between two TRPs need to be considered.

Proposal 3: RAN4 to consider the measurement accuracy as the maximum absolute timing offset (TO) and frequency offset (FO) value of (CDF 5%, CDF 95%) to the bias (TO and FO setting between two TRPs).

Based on above simulation result, we summarize the observation as below:

- The higher SNR, the less timing/frequency offset span between two TRPs.

- The higher SCS, the more frequency offset span between two TRPs.

  - Since the symbol length of SCS 30Khz is shorter than SCS 15KHz, it’s more sensitive for frequency offset measurement on time domain.

- The less SCS, the more timing offset span between two TRPs.

  - Since the subcarrier spacing of SCS 15Khz is smaller, it’s more sensitive for timing offset measurement on frequency domain.

Observation 1: Based on the simulation result on Table 2 and Table 3, we summarize the impact factor of TO and FO between two TRPs as below:

- The higher SNR, the less timing/frequency offset span between two TRPs.

- The higher SCS, the more frequency offset span between two TRPs.

- The less SCS, the more timing offset span between two TRPs.

In addition, there may be some problem even if UE provide timing offset or frequency offset of multi-TRP to NW. For instance, TOE span is 524.56 ns for test set#1 in Table 2. Since the range of TOE span is not small, it’s expected that there’s some bias after NW compensate TO. Since the measured accuracy of TO may not be so accurate and quantization also introduce bias. In our understanding, the performance degradation may be expected even if NW do pre-compensation on TO for multi-TRP. The same problem also existed after NW do pre-compensation on FO for multi-TRP.

Observation 2: The measured timing offset (TO) and frequency offset (FO) may not be so accurate enough. The performance degradation may be expected even if NW do pre-compensation for TO and FO under multi-TRP.

To minimize the range of TOE and FOE span, one way is to average more samples. For test set#1 in Table 2, the TOE span is reduced to 327.56ns when averaging the number of 3 samples. The averaging more samples, the more time is needed for CJT calibration. However, how many samples are feasible or accurate enough for CJT performance enhancement is out of scope from RRM perspective. Therefore, we still consider all of samples (1, 3, 5) for TO and FO measurement in accuracy requirement. How NW do the pre-compensation based on CJT calibration report is up to NW implementation.

Proposal 4: RAN4 to discuss on the number of samples used for the accuracy requirement for timing offset and frequency offset measurement between two TRPs.

## 2.2 Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

Based on previous RAN1 agreement, NW should indicate PL offset for UE to calibrate UL only TRP. In our understanding, the DL reference timing definition for PDCCH-order PRACH will be discussed in RAN1. We can wait for RAN1 progress on DL reference timing. The unified TCI state indication for the asymmetric DL sTRP/UL mTRP can totally reuse Rel-17 and Rel-18 unified TCI framework. Therefore, the following proposal is suggested.

Proposal 5: No RRM impact at least for unified TCI state switching since RAN1 agreed to reuse Rel-17/Rel-18 unified TCI framework to indicate TCI states of asymmetric DL sTRP/UL mTRP deployment scenarios.

It is noted that in the last meeting companies understand that UL TCI state can be associated to SRS. However, in the legacy MIMO requirements, there was no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS. Therefore, similar principle can be reused here in R19 MIMO.

Proposal 6: If UL TCI states can be associated to SRS, then reuse the same principle in Rel-17 which is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: RAN4 to consider the simulation parameters on Table 1 as one of candidates to provide the simulation result of timing and frequency offset estimation.

Proposal 2: The simulation result of timing offset and frequency offset between two TRPs are provided on Table 2 and Table 3.

Proposal 3: RAN4 to consider the measurement accuracy as the maximum absolute timing offset (TO) and frequency offset (FO) value of (CDF 5%, CDF 95%) to the bias (TO and FO setting between two TRPs).

Observation 1: Based on the simulation result on Table 2 and Table 3, we summarize the impact factor of TO and FO between two TRPs as below:

- The higher SNR, the less timing/frequency offset span between two TRPs.

- The higher SCS, the more frequency offset span between two TRPs.

- The less SCS, the more timing offset span between two TRPs.

Observation 2: The measured timing offset (TO) and frequency offset (FO) may not be so accurate. The performance degradation may be expected even if NW do pre-compensation for TO and FO under multi-TRP.

Proposal 4: RAN4 to discuss on the number of samples used for the accuracy requirement for timing offset and frequency offset measurement between two TRPs.

Proposal 5: No RRM impact at least for unified TCI state switching since RAN1 agreed to reuse Rel-17/Rel-18 unified TCI framework to indicate TCI states of asymmetric DL sTRP/UL mTRP deployment scenarios.

Proposal 6: If UL TCI states can be associated to SRS, then reuse the same principle in Rel-17 which is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

# Reference

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung

[2] R4-2420108, WF on RRM requirements for NR_MIMO_Ph5 (Part2)