3GPP TSG-RAN WG4 Meeting #114	R4-2502279

# Athens, Greece, 17th February – 21st February 2025

Title: 	Way Forward for [114][303] NR_BS_RF_Part2_CLTA

Agenda Item:	7.13.5

Source: 	Huawei, HiSilicon

Document for:	Approval

## Sub-topic 1-1: BS-to-BS isolation analysis

Way forward:

BS-to-BS isolation data collection to prioritize the following scenarios:

- Horizontal domain prioritized (vertical BS stacking analyses not precluded);

- Same-band analysis (inter-band analysis not precluded),

- Array-to-array case (array to sub-array not precluded);

- Same BS type for aggressor and victim (mixed cases of various BS types not precluded).

Guidance for collection of future results: Companies to clarify on the statistical distribution of the results provided (i.e., worst case, CDF, etc.)

For the derivation of the conclusion, [95-99th] percentile of CDF is expected to be used for simulations, or the worst-case results from measurements.

BS-to-BS isolation data (simulations, or measurements) to be collected on top of the data already collected in the following table formats:

- No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | ZTE (M, R4-2501875): -54 | ZTE (R4-2501875): -52 | ZTE (M, R4-2501875): -56 |  |
| Isolation in horizontal domain (dB) | ZTE (M, R4-2501875): -31 | ZTE (M, R4-2501875): -34 Huawei (M, R4-2502135): -46 | ZTE (M, R4-2501875): -46 |  |
| - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling |  |  |  |  |

- Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 6.7 GHz --> 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) |  |  |  |  |
| Isolation in horizontal domain (dB) |  | S: Ericsson (R4-2418398): A-to-SA: -55±20 dB |  | S: Ericsson (R4-2502160): A-to-A: -77±25; A-to-SA: -70±20 |

For the deprioritized scenarios, reuse table templates as above.

- Timeline for BS-to-BS isolation data collection: conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting (RAN4#116).

- Data collection for other carrier frequencies is not precluded.

- Any future conclusion on isolation numbers is subject to availability of sufficient amount of data from multiple companies and can be decided case-by-case.

## Sub-topic 1-2: Specifications set baseline

Way forward: Baseline set of specifications for implementation of the BS-BS isolation conclusions:

- TS 37.105

- TS 37.145-1/-2

- TS 38.104

- TS 38.141-1/-2

- TS 38.174

- TS 38.176-1/-2

- TS 38.106

- TS 38.115-1/-2

## Sub-topic 1-3: Requirements set baseline

Way forward: Baseline set of requirements for implementation of the BS-BS isolation conclusions:

- Transmitter OFF power requirement

- Co-location spurious emission requirement/Protection of the BS receiver of own or different BS

- Tx IMD requirement

- Co-located OOBB requirement

## Sub-topic 1-4: TX IMD co-location

Way forward:

Companies are encouraged to provide inputs on the handling of the intra-system TX IMD requirements.

## Sub-topic 1-5: Capturing CLTA-related agreements

Way forward:

As there is a dedicated CLTA section in TR 37.941 on RF conformance testing background for OTA BS requirements, consider TR 37.941 as the placeholder for the CLTA-related agreements collection. Rel-19 findings to be ca in a brand new sub-clause.

## Sub-topic 1-6: Horn antenna aspects

Way forward: Companies to investigate applicable frequency range for horn antenna studies (i.e., where the reference CLTA may not be available).

## Sub-topic 1-7: Assumptions for the beam steering scenario analyses

Way forward: To improve simulation results alignment, the following assumptions are taken for the beam steering consideration:

Data collection to prioritize the following scenarios:

- Beam steering range +/- [60] degrees

- Reuse CLTA arrangements, e.g., same plane orientation; 10 cm offset enclosure to enclosure, etc.