3GPP TSG-RAN WG4 Meeting #114	 														R4-2502579

Athens, Greece, 17th – 21st February, 2025

Agenda item:			7.27.3

Source:	Apple

Title:	Ad-hoc minutes for NR mobility enhancements

Document for:	Approval

# Topic #1: General aspects and work plan

#### Issue 1-1: scope of R19 NR mobility enhancements Phase 4

- Proposals:

  - P1: Not to consider MCG change with SCG change in RAN4 in R19. (MTK)

- Recommended WF:

  - Discuss if P1 is agreeable.

Discussion:

MTK: MCG change with SCG change is not in the WID.

E///: RAN2 agreed to support this scenario.

MTK: yes, they also agreed this can be revisited.

E///: we don’t need to specifically discuss this scenario. But no need to exclude.

Chair: no consensus on P1. E/// needs to further check with RAN2.

# Topic #2: Event triggered L1 measurement reporting

#### Issue 2-1: applicability of L1-RSRP reporting delay

<Candidate options>

- Option 1: (vivo)

  - RAN4 starts the discussion on L1-RSRP reporting delay with the follow assumptions:

    - No indicate TCI change during the evaluation period.

    - No configuration change w.r.t measurement RS during the evaluation period

    - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

<Recommended WF>

- Continue discussion.

Discussion:

QC: this has too much restriction.

E///: maybe we can consider this only in test case design. But not as assumptions in core requirements design.

HW: is there similar issue in legacy L3 measurement?

Chair: no consensus on option 1.

#### Issue 2-10: Impact on other LTM requirements

<Candidate solutions>

- Proposal 1: If the UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, the related delay requirements are applicable. (Nokia)

- Proposal 1a: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. (E///)

- Proposal 2: In addition to the existing conditions, TLTM_RRC-processing = 0 if the UE has sent event-triggered L1 report for the target cell before receiving the cell switch command. (Nokia)

- Proposal 3: RAN4 to consider that after event-triggered L1 reporting condition is fulfilled, the UE is allowed to prioritize L1 measurements on the candidate cell(s) fulfilling the reporting condition. (Nokia)

<Recommended WF>

- Continue discussion.

Discussion:

Proposal 1: If the UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, the related delay requirements are applicable.

MTK: RAN2 agreed on some enter condition and leaving condition.

Tentative agreement:

UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, [and leaving condition of the event is configured and corresponding condition is not met], the related cell switch delay and TCI state activation requirements are applicable.

- Further discuss how to capture this in RRM spec.

# Topic #3: CSI-RS based L1 measurement

#### Issue 4-4: Measurement type

- Candidate options:

  - Option 1: (Xiaomi, MTK, )

Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19.

  - Option 2: (ZTE, Huawei, China Telecom)

Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell.

  - Option 3: (CATT)

In Rel-19, if ’the CSI-RS of candidate cell outside active BWP’ is supported for LTM,

    - RAN4 need to discuss whether to introduce measurement gap for CSI-RS based L1 measurement.

    - RAN4 need to discuss whether to reuse type 1 MG in R18 LTM or introduce the new gap for CSI-RS based measurement.

  - Option 4: (CMCC, Ericsson)

    - CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario, and it is proposed to define requirements for CSI-RS based L1-RSRP measurement outside active BWP.

  - Option 5 (OPPO)

Support intra-frequency measurement and inter-frequency measurement and deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. Otherwise, an optional UE capability for supporting the case that CSI-RSs are outside active BWP shall be introduced.

  - Option 6: (Nokia)

CSI-RS based L1 measurement with gap is deprioritised out from rel-19
Consider also the scenario where CSI-RS based L1-RSRP measurement is outside of the active BWP.

  - Option 7: (Qualcomm, Nokia)

RAN4 to not consider scenarios where a UE needs measurement gap for CSI-RS measurements from LTM candidate cell.

  - Option 8: (Ericsson)

Measurement within active BWP, Measurements outside active BWP

RAN4 to define the gap-based CSI-RS L1-RSRP measurement which do not impact the legacy SSB based L1-/L3-RSRP measurement within gap.

  - Option 9 (vivo)

If RAN4 agree to define requirements for both within active BWP and outside active BWP, an optional UE capability for CSI-RSs are outside active BWP shall be introduced. Prefer to deprioritize the case where CSI-RS for LTM L1 measurements are outside active BWP(s).

RAN4 do not support CSI-RS based L1 measurement with gap in Rel-19

- Recommended WF:

  - Discuss the options.

Discussion:

- Option A: Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. (E///, CMCC, ZTE, CATT, China Telecom)

- Option B: Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. (QC, MTK, Nokia, Xiaomi, vivo, OPPO)

- Option C: Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell is subject to optional UE capability.

  - RAN4 to define RRM requirement for the gap-based CSI-RS L1-RSRP measurement.

#### Issue 4-10: Restriction on CSI-RS resource configuration

- Candidate options:

  - Option 1: (Apple)

It is proposed to introduce a CSI-RS based L1 measurement timing configuration.

  - Option 3: (Ericsson)

RAN4 not to define L1 measurement timing configuration for CSI-RS based L1-RSRP measurements for LTM

- Recommended WF:

  - Discuss the options.

Discussion:

Agreement:

In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration.

In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction:

- All CSI-RS resources on [one intra-frequency layer] are configured within up to two separate windows where each window is up to [5 ms]

# Topic #4: conditional LTM

Issue 1-4-2: The starting point of Subsequent conditional LTM delay

- Candidate solutions:

- Proposal 1: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM. (CTC, vivo, CATT, CMCC, Apple, Huawei, ZTE)

- Recommended WF

  - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

  - Revisit if RAN2 has more progress.

Discussion:

Agreement:

  - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM, [under assumption that UE will continue evaluating the CLTM event after the completion of previous CLTM].

  - Revisit if RAN2 has more progress.

Issue 1-3-2: The starting point of conditional LTM cell switch delay

- Candidate solutions:

- Proposal 1: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. (CTC, Xiaomi, CATT, Apple, ZTE)

- Proposal 2: The starting point of conditional LTM delay is the time the UE receives an RRC message implying conditional LTM. (vivo, CMCC, Huawei)

- Proposal 3: The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration. (ZTE)

- Proposal 4: Wait for RAN1/RAN2 more progress. (Nokia)

- Recommended WF

  - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM.

  - Revisit if RAN2 has more progress.

Discussion:

Agreement:

  - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM, [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM].

  - Revisit if RAN2 has more progress.

Issue 1-3-3: The components of conditional LTM cell switch delay

- Candidate solutions:

- Proposal 1: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. (CTC)

    - Proposal 1a: TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. (CTC)

- Proposal 2: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + … +TEarly_process. (E///)

- Proposal 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_interrupt + TCLTM_execution. (CTC, Xiaomi, vivo, CATT, CMCC, Apple, ZTE)

    - Proposal 3a: Whether/How to capture the time for early candidate TCI State activation/deactivation and early TA acquisition. (ZTE)

- Proposal 4: DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. (Huawei)

    - Proposal 4a: After TCHO_execution, the component TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused for R19 C-LTM. (Huawei)

- Proposal 5: Wait for RAN1/RAN2 more progress. (CATT, Nokia)

- Recommended WF

  - Discussion is needed.

Discussion:

Agreement:

Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc

TCLTM_interrupt = TLTM-processing + TLTM-IU ms

Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc

TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms

Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt

TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms