3GPP TSG-RAN WG4 Meeting #114	 														R4-2500529

Athens, Greece, 17th – 21st February, 2025

Agenda item:			7.11.7

Source:	Apple

Title:	WF on RRM requirements for NR_NTN_Ku_bands

Document for:	Approval

# Topic #1: RRM requirements for NR_NTN_Ku_bands

Issue 1-1: RRM scope for NR NTN on Ku bands

Agreement:

- For SCS, consider SCS=30kHz in FR1-NTN numerology and 120kHz in FR2-NTN numerology, following the discussion in BDaT session.

- Mobile VSAT UE shall be considered from RAN4 RRM requirement perspective. There are four cases shall be considered for mobile VSAT UE:

  - Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology

  - Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology

  - Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology

  - Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology

Issue 1-2: impact on RRM requirements from NR NTN on Ku bands

Proposals:

- Option 1: RAN4 RRM group to consider the sections identified in the following table and determine which of these can be reused unmodified, which need to be revised for editorial modification and which need new technical work.  (Eutelsat Group)

| Section Number | Section/ sub-section title |
| --- | --- |
| RRM Core Requirements |  |
| 4.2.2.12 | Measurements of inter-frequency NR cells with NTN carrier |
| 4.2C | Cell Re-selection for NR UE for Satellite Access |
| 4.2C.2 | Requirements |
| 4.2C.2.1 | UE measurement capability |
| 4.2C.2.2 | Measurement and evaluation of serving cell |
| 4.2C.2.3 | Measurements of intra-frequency NR cells |
| 4.2C.2.4 | Measurements of inter-frequency NR cells |
| 6.1C | Handover for SAN |
| 6.1C.1.2 | NR SAN FR1 – NR SAN FR1 Handover |
| 6.1C.1.3 | NR SAN FR2-NTN – NR SAN FR2-NTN Handover |
| 6.1C.2.4 | NR SAN FR2-NTN – NR SAN FR2-NTN conditional handover |
| 6.1C.3 | NR SAN Satellite switching with re-synchronization |
| 6.1C.3.2 | NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization |
| 7.1C | UE transmit timing for Satellite Access |
| 7.1C.1 | Introduction |
| 7.1C.2 | Requirements |
|  | Table 7.1C.2-1: Te_NTN Timing Error Limit |
|  | Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO |
|  | Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO |
|  | Table 7.1C.2-3: The Value of NTAoffset for VSAT in FR2-NTN |
| 7.1C.2.1 | Gradual timing adjustment |
|  | Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate |
| 7.2C | UE timer accuracy for satellite access |
| 7.3C | Timing advance for satellite access |
| 7.3C.2.2 | Table 7.3C.2.2-1: UE Timing Advance adjustment accuracy |
|  | Table 7.3C.2.2-2: UE Timing Advance adjustment accuracy for VSAT UE in FR2-NTN |
| 8.1C | Radio Link Monitoring for Satellite Access |
|  | Table 8.1C.1-2: Maximum number of RLM-RS resources NRLM |
| 8.1C.2 | Requirements for SSB based radio link monitoring |
| 8.1C.2.3 | Measurement restrictions for SSB based RLM |
| 8.1C.3 | Requirements for CSI-RS based radio link monitoring |
| 8.1C.3.2 | Minimum requirement |
|  | Table 8.1C.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for [FR2-NTN] |
| 8.1C.3.3 | Measurement restrictions for CSI-RS based RLM |
| 8.1C.7 | Scheduling availability of UE during radio link monitoring |
| 8.1C.7.1 | Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1 and [FR2-NTN] |
| 8.1C.7.2 | Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1 and [FR2-NTN] |
| 9.1C.2 | Measurement gap |
|  | Table 9.1C.2-2 |
|  | Table 9.1C.2-2a |
| 9.2C.2 | Requirements applicability |
| 9.2C.3 | Number of cells and number of SSB |
| 9.2C.4 | Measurement Reporting Requirements |
| 9.2C.4.1 | Periodic Reporting |
| 9.2C.4.2 | Event-triggered Periodic Reporting |
| 9.2C.4.3 | Event Triggered Reporting |
| 9.3C | NR inter-frequency measurements for SAN |
| 9.3C.2 | Requirements applicability |
| 9.3C.3 | Number of cells and number of SSB |
| 9.3C.3.1 | Requirements for FR1 |
| 10.1.3C | Intra-frequency RSRP accuracy requirements for FR2-NTN |
| 10.1.5C | Inter-frequency RSRP accuracy requirements for FR2-NTN |
| 10.1.5C.1 | Inter-frequency SS-RSRP accuracy requirements |
| 10.1.5C.1.1 | Absolute SS-RSRP Accuracy |
| 10.1.5C.1.2 | Relative SS-RSRP Accuracy |
| 10.1.8C | Intra-frequency RSRQ accuracy requirements for FR2-NTN |
| 10.1.8C.1 | Intra-frequency SS-RSRQ accuracy requirements in FR2-NTN |
| 10.1.8C.1.1 | Absolute SS-RSRQ Accuracy in FR2-NTN |
| 10.1.10C | Inter-frequency RSRQ accuracy requirements for FR2-NTN |
| 10.1.10C.1 | Inter-frequency SS-RSRQ accuracy requirements in FR2-NTN |
| 10.1.10C.1.1 | Absolute Accuracy of SS-RSRQ in FR2-NTN |
| 10.1.10C.1.2 | Relative Accuracy of SS-RSRQ in FR2-NTN |
|  | Table 10.1.10C.1.2-1: SS-RSRQ Inter frequency relative accuracy in FR2-NTN |
| 10.1.13C | Intra-frequency SINR accuracy requirements for FR2-NTN |
| 10.1.13C.1 | Intra-frequency SS-SINR accuracy requirements in FR2-NTN |
| 10.1.13C.1.1 | Absolute SS-SINR Accuracy in FR2-NTN |
|  | Table 10.1.13C.1.1-1: SS-SINR Intra frequency absolute accuracy in FR2-NTN |
| 10.1.15C | Inter-frequency SINR accuracy requirements for FR2-NTN |
| 10.1.15C.1 | Inter-frequency SS-SINR accuracy requirements in FR2-NTN |
| 10.1.15C.1.1 | Absolute Accuracy of SS-SINR in FR2-NTN |
|  | Table 10.1.15C.1.1-1: SS-SINR Inter frequency absolute accuracy in FR2-NTN |
| 10.1.15C.1.2 | Relative Accuracy of SS-SINR in FR2-NTN |
|  | Table 10.1.15C.1.2-1: SS-SINR Inter frequency relative accuracy in FR2-NTN |
| Performance Requirements for Information (to follow after the Ku band WID Core RRM work) |  |
| Annex |  |
| A.3.36 | Testing related to Satellite access |
| A.3.36.2 | Principle of testing different RRM requirements |
|  | Table A.3.36.2-1: Test cases for NTN specific requirements |
| A.3.36.4 | General setup for SIB19 |
|  | Table A.3.36.2-1: Test cases for NTN specific requirements |
| A.3.36.5 | Satellite specific parameters configuration |
| A.3.36.5.1 | Satellite specific configuration for serving cell |
|  | Table A.3.36.5.1-1: Satellite specific configuration pattern 1 for serving cell in GSO scenario |
|  | Table A.3.36.5.1-2: Satellite specific configuration pattern 2 for serving cell in NGSO scenario |
| A.3.36.5.2 | Satellite specific configuration for neighbour cell |
| A.3.15C | Angle of Arrival (AoA) for FR2-NTN RRM test cases |
| A.14.1.2 | Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements |
| A.14.2.1.7 | Intra-frequency intra-satellite Handover from FR2-NTN to FR2-NTN |
|  | Table A.14.2.1.7.2-1: Supported test configurations |
|  | Table A.14.2.1.7.2-2: General test parameters Intra-frequency intra-satellite handover from FR2-NTN to FR2-NTN |
|  | Table A.14.2.1.7.2-3: Cell specific test parameters for intra-frequency intra-satellite handover from FR2-NTN to FR2-NTN |
| A.14.2.1.9 | Intra-frequency inter-satellite handover from FR2-NTN to FR2-NTN |
|  | Table A.14.2.1.9.2-1: Supported test configurations |
|  | Table A.14.2.1.9.2-2: General test parameters |
|  | Table A.14.2.1.9.2-3: Cell specific test parameters for NR FR2-FR2 Intra frequency handover test case |
| A.14.3.1.2 | NR UE Transmit Timing Test for FR2-NTN |
|  | Table A.14.3.1.2.1-1: Supported test configurations for FR2-NTN PCell |
| A.14.3.2.3 | SA FR2-NTN timing advance adjustment accuracy |
|  | Table A.14.3.2.3.2-1: Timing advance supported test configurations |
| A.14.5.1.7 | SA event triggered reporting test with SSB time index reading without gap under non-DRX for FR2-NTN |
|  | Table A.14.5.1.7.2-1: Supported test configurations |
|  | Table A.14.5.1.7.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR2-NTN with SSB index reading |
|  | Table A.14.5.1.7.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR2-NTN with SSB index reading |
|  | Table 14.5.1.X.2-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with FDD PCell in FR2-NTN without gap without DRX |
| A.14.5.3.5 | SSB based L1-RSRP measurement when DRX is not used in FR2-NTN |
|  | Table A.14.5.3.5.1-1: Applicable NR configurations for FR2-NTN SSB based L1-RSRP test |
|  | Table A.14.5.3.5.2-1: General test parameters |
|  | Table A.14.5.3.5.2-2: SSB specific test parameters |
| A.14.6.1.3 | SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell |
|  | A.14.6.1.3.3 Test Requirements |
|  | Table A.14.6.1.3.3-1: SS-RSRP absolute accuracy test requirement |
| A.14.6.4.3 | SSB based L1-RSRP measurement for VSAT UE in FR2-NTN when DRX is not used |
|  | Table A.7.6.3.1.1-1: Applicable NR configurations for FR2 SSB based L1-RSRP test for VSAT UE |
| B.2.1.7 | Derivation of Minimum SSB_RP values for FR2-NTN for satellite access |
| B.2.18 | Conditions for NR inter-frequency measurements for satellite access |
| B.2.19 | Conditions for NR L1-RSRP reporting for satellite access |
| B.2.19.1 | Conditions for SSB based L1-RSRP reporting for satellite access |
| B.2.19.2 | Conditions for CSI-RS based L1-RSRP reporting for satellite access |
| B.2.20 | Conditions for RRC connection release with redirection to NR for satellite access |

- Option 1a: The potential RRM requirements could be seen as below. (ZTE)

| Requirements |  |
| --- | --- |
| Cell reselection | UE measurement capability |
|  | Measurements on intra/inter-frequency NR cells |
|  | ﻿Maximum interruption in paging reception |
|  | ﻿Measurements of intra/inter-frequency NR cells for UE configured with relaxed measurement criterion |
| Handover | Handover |
|  | NR conditional handover |
|  | ﻿RRC Connection Mobility Control: RRC re-establishment |
| Timing | UE transmit timing |
|  | UE timer accuracy |
|  | Timing advance |
| Signaling | RLM |
|  | BWP switch |
|  | TCI state switch |
| Measurement procedure | UE Measurement capability |
|  | Intra/inter-frequency measurements |
|  | L1-RSRP measurements |

- Option 2: (MTK)

  - For intra-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point.

  - For inter-Satellite RRM requirement for Ku-band, Ka band RRM requirements can be used as the starting point. Besides, RAN4 to discuss whether to reuse the same value of T_sat_beam for Ku band Handover requirement.

- Option 3: FR2 (Ka-band) NTN RRM requirements could be reused for Ku-band NTN type 4 and 5 (not to introduce separate RRM requirements for Ku-band NTN), except for timing error requirements. (LG)

- Option 4: R17 NR NTN related requirements shall be reused or as baseline when we define the FR1-NTN requirements for Ku bands. On the contrary, R18 NR NTN related requirements shall be reused or as baseline when we define the FR2-NTN requirements for Ku bands. (ZTE)

- Option 5: The following RRM requirements for VSAT UE in Ku band reuse the existing requirements defined for el-18 FR2-NTN: (QC)

- RRC Idle and Inactive mobility

- L1 measurements

- RLM

- RRC Re-establishment

- L3 measurements

- UE Measurement capability

- Measurement gap

- Scheduling and Measurement Restrictions

- Intra-satellite Handover

- Inter-satellite Handover

Recommended WF:

- Take option 1 as baseline. Discuss which requirements shall be removed and whether other requirements need to be added.

Issue 1-3: UL transmit timing requirement

Agreement:

  - RAN4 to define timing accuracy requirements for case 1, 2:

    - For FR1-NTN numerology, existing requirements in Table 7.1C.2-1 of 38.133 (FR1-NTN) are re-used.

      - For the applicable SCS, follow the agreement in BDaT session.

  - RAN4 to define timing accuracy requirements for case 3:

    - For FR2-NTN numerology, existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used.

      - For the applicable SCS, follow the agreement in BDaT session.

      - The endorsed CR with 7.5 Ts under the same side conditions as Rel-18 are considered.

  - RAN4 to define timing accuracy requirements for case 4:

    - For FR2-NTN numerology, FFS existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used.

      - For the applicable SCS, follow the agreement in BDaT session.

      - The endorsed CR with 7.5 Ts is considered. FFS whether the same or different side condition from Rel-18 can be used.

Issue 1-4: terminology related issue

Proposals:

- Option 1: An overarching task in RAN4 is to determine how terminology currently used in TS 38.133 can be aligned to, clearly and unambiguously, identify Ku bands defined for FR1-NTN and Ku bands defined for FR2-NTN. (Eutelsat Group)

Recommended WF:

- Continue discussion.