# 3GPP TSG-RAN WG4 Meeting #114	R4-2502592

# Athens, GR, 17th – 21st February, 2025

Agenda item:			7.17.4

Source:	Moderator (Apple)

Title:	WF on RRM requirements for NR_RRM_Ph5_Part1

Document for:	Approval

# Topic #1: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (7.17.2.2)

## Issue 1-1: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

- Option 1 (Apple, CMCC, HW, Ericsson, vivo): The enhanced CSSF apply to all the 3 aspects and do not define the priority.

- Option 2 (Nokia, ZTE, CATT, CTC, vivo): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion.

Agreement:

  - Option 1 is agreed

## Issue 1-3: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Issue 1-3-1: Tentative solution 1: measure one serving CC per band

Agreement:

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings,

  - Network indication of enable/disable this CSSF enhancement is per-UE.

  - Follow the following principle for serving and neighboring cell measurement

    - Measure PCC if PCC in band

    - Measure PSCC if PSCC in band

    - Measure SCC on which UE is configured to report SSB based measurements, if

      - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

  - Follow the network indication to measure a SCC, if

    - neither PCC nor PSCC is in band, and

    - if there is no SCCs on which UE is configured to report SSB based measurements in band, or

    - if there are multiple SCCs on which UE is configured to report SSB based measurements in band.

    - Network indication of a specific SCC per-band for measurement.

      - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band.

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

- Note: Following the WID, the above agreement applies for the measurement without gap.

Issue 1-3-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

Issue 1-3-3: Tentative solution 3: 3 searchers for CSSF

Agreement:

Support Solution 3 in Rel-19:

- FR1 scenario (CA [and NR-DC]) and FR1 + FR2 scenario (CA [and NR-DC]) only.

  - Define separate new UE capabilities for FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC).

  - For FR1+FR2 scenario, PCell is FR1

  - FFS whether NR-DC is included or not

Design for solution 3 (agreed in ad-hoc session)

- Option 3 is as a by-default for CSSF enhancement solution 3, unless RAN4 agreed to introduce a network indication for specific SCC to use 3rd searcher in the future.

    - Option 3: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule,

      - the 3rd searcher resource sharing is same as the second searcher sharing.

Not support solution 2 in Rel-19.

Issue 1-3-4: Others

Proposals (Nokia):

- RAN4 to define optimized CSSF assuming the UE is able to sample SSBs simultaneously from multiple CCs and process these SSB samples offline.

- The SCell activation delay requirement shall be improved considering the optimized measurement for CSSF reduction.

Agreement:

  - This issue is closed