3GPP TSG-RAN WG4 Meeting #114										                  R4-2502597

Athens, Greece, 17 – 21 February, 2025

Agenda item:			6.20.4

Source:	Huawei, HiSilicon, Apple

Title:	WF on RRM requirements for NR_duplex_evo

Document for:	Approval

Agreements

Topic 1: UE-to-UE CLI handling

- Sub-topic 1-1: General issues

#### Issue 1-1-1: Measurement methods

- Agreements (from Mon AH session)

  - RAN4 to define the CLI measurement requirements for Method #3 (UE measures RSSI within UL subband)

Issue 1-1-3: Measurement resources

- Agreements

  - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements.

  - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types.

Issue 1-1-4: Measurement reporting

- Agreements

  - Focus on aperiodic reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1.

  - L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the aperiodic L1 based CLI measurement reporting.

#### Issue 1-1-5: Scheduling restriction

- Agreements (from Thu AH session)

  - For L1-SRS-RSRP,

    - For UL, scheduling restrictions always apply, for both FR1 and FR2.

    - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies.

    - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements,

      - Note: this time has accounted for the UL/DL switching time.

    - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition.

  - For L1-CLI-RSSI Methods #1,

    - For UL, scheduling restrictions always apply, for both FR1 and FR2.

    - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2.

    - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed.

  - For L1-CLI-RSSI Method #3, scheduling restriction requirements are FFS

Sub-topic 1-2: L1-SRS-RSRP

Issue 1-2-1: Side conditions

- Agreements (from online session)

  - For SRS configuration, reuse R16 assumptions on SRS configuration for L1 SRS-RSRP measurements

- Agreements

  - Further discuss timing offset side condition for accuracy requirements in the performance part of the WI, including whether to consider intra-cell scenario only or both intra- and inter-cell scenarios for defining the side condition.

  - For SRS Es/Iot, further discuss the following options

    - Option 1: 1dB

    - Option 2: > 1dB

  - For SRS BW, further discuss the following options

    - Option 1: 24RB

    - Option 2: 48RB

Issue 1-2-2: Measurement period

- Agreements (from online session)

  - RAN4 to define one set of accuracy requirements based 1 sample.

  - Send LS to RAN1/2 on whether the NW configuration of timeRestrictionForChannelMeasurement or a similar configuration will apply for L1 SRS-RSRP and/or CLI-RSSI measurement.

    - From RAN4 requirement perspective: if it applies, RAN4 to define two sets of requirements for L1 SRS-RSRP measurement delay.

- Agreements

  - For aperiodic resources, measurement period is based on 1 sample.

Sub-topic 1-2: L1-CLI-RSSI

Issue 1-3-1: Measurement timing for Method #3

- Agreements (from online session)

  - Base on DL reference timing

Topic 2: SBFD operation

Sub-topic 2-1: RRM impacts of SBFD operation

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Agreements

  - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case, depending on UE measurement capability

    - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

    - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

  - FFS whether and how to account for the UE capability on CSI processing time.

  - Further discuss the accuracy requirements for the following cases

    - Case 1: X1 >= X3 and X2 >= X3

    - Case 2: X1 >= X3 and X2 < X3

    - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3

where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

Issue 2-1-2: Requirements for CSI-RS based L3 measurement

- Agreements (from Wed AH session)

  - For DU case:

    - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols.

  - FFS on DUD case

# Potential issues for further discussion

Topic 1: UE-to-UE CLI handling

- Sub-topic 1-1: General issues

Issue 1-1-2: Rx beam

Issue 1-1-6: Measurement restriction

Issue 1-1-7: Measurement capability

Issue 1-1-8: Report mapping

Issue 1-1-9: Requirement applicability

Issue 1-1-10: Spec structure

Sub-topic 1-2: L1-SRS-RSRP

Issue 1-2-3: Measurement accuracy

Sub-topic 1-3: L1-CLI-RSSI

Issue 1-3-2: Side conditions

Issue 1-3-3: Measurement period

Issue 1-3-4: Measurement accuracy

Issue 1-3-5: SCS for performing measurement

Topic #2: SBFD operation impacts

Sub-topic 2-1: RRM impacts of SBFD operation

Issue 2-1-1a: CSI-RS L1 measurement – collision with UL

Issue 2-1-1c: CSI-RS L1 measurement – different symbol types

Issue 2-1-3: Requirements for scheduling restriction

Issue 2-1-4: Requirements for RACH requirements

Issue 2-1-5: Requirements for UL PC and or spatial relation update

Issue 2-1-6: Requirements for MG and BWP switch

Issue 2-1-7: Requirements for UL resource muting