3GPP TSG-WG4 Meeting #114	 R4-2502840

# Athens, Greece, Feb 17 – 21, 2025

Title: 	WF on simultaneous Rx-Tx

Agenda Item:	6.1

Source: 	Huawei

Document for:	Approval

Background:

Simultaneous Rx/Tx feature with inter-band CA/DC:

The feature simultaneous Rx-Tx of inter-band CA/DC is quite important for the interaction between the network and the UE. For some band combinations that are not supportive of simultaneous Rx-Tx, there would be scheduling restriction from network side. RAN4 started discussing the rules for simultaneous Rx-Tx in Rel-17. The handling of BC with different duplex mode is different.

- For FDD-TDD and TDD-TDD band combination, the capability signalling simultaneousRxTxInterBandCA was designed.

  - The capability is mandatorily reported for FDD-TDD BC. It was agreed to capture in RAN4 spec that ‘Unless stated otherwise, simultaneous Rx/Tx capability is mandatory for FR1+FR1 FDD-TDD CA combinations’

  - The capability is optionally reported for TDD-TDD BC. Case by case analysis is required. However, in theory, if MSD is defined for a band combination, it is possible to support simultaneous Rx-Tx.

- For FDD-FDD band combination, there is no capability signalling designed for simultaneous Rx-Tx. There is no scheduling restriction for FDD-FDD band combination from network side. Both the network and UE assume simultaneous Rx-Tx is mandatorily supported without signalling, by default. However, for special case like CA_n5-n8, it is difficult for UE to implement the con-current operation between n5DL and n8UL. A method should be designed for interaction between network and UE, such as, signalling design or Note pre-defined in spec.

- For SUL band combinations, the capability simultaneousRxTxSUL is mandatorily reported for each one in current specification.

- For SDL band combination, the SDL band can be viewed as derived from FDD band.

  - The handling of FDD-SDL BC refers to that for FDD-FDD

  - The handling of TDD-SDL BC refers to that for TDD-FDD.

Simultaneous Rx/Tx for CA_n40A-n41A

The bands n40 and n41 may begrouped together due to their close frequency distance. This allows to reduce implementation complexity and antenna count. In case of concurrent 4-layer MIMO and simultaneous Rx/Tx the two Rx chains sharing antennas with Tx are expected to observe de-sense as well as the diversity antennas which is also demonstared due to high cross-band MSD .

- For a device experiencing strong noise imbalance between shared and diversity antennas it might be beneficial reduce MIMO rank. Then Tx and Rx could be handled on different antennas which reduces noise in the Rx due to the antenna isolation.

<Way forward>

Way forward on Simultaneous Rx/Tx feature with inter-band CA/DC:

- To clarify the handling of simultaneous Rx-Tx interaction between network and UE for band combinations with different types of duplex mode

  - For TDD-SDL band combination, simultaneous Rx-Tx is mandatory with signalling. Consider to reuse the capability of simultaneousRxTxInterBandCA referring to TDD-FDD.

  - For FDD-FDD band combinations, simultaneous Rx-Tx is mandatory without signalling.

    - Deviations to this due to UL CA_n5-n8 is TBD

  - For FDD-SDL band combinations, simultaneous Rx-Tx is mandatory without signalling.

    - Deviations to this above due to LB-LB RX switching including LB-LB configurations in RP-242532 are TBD.

- Consider to specify the requirements of simultaneous Rx-Tx for inter-band CA/DC using a new column in the two-band BC table(s), capturing the handling of each type of BC with different duplex modes.

  - Handling properly every Note for simultaneous Rx-Tx.

Way forward on Simultaneous Rx/Tx for CA_n40A-n41A:

It has been observed that RAN2 signalling might allow to indicate the same band combination twice with different capabilities by utilizing separate feature-sets. Companies are encouraged to check if current RAN2 framework allows to indicate different number of MIMO layers with and without simultaenous Rx/Tx.

Reference

[1] R4-2500146, Discussion on simultaneous Rx-Tx, Huawei, Hisilicon.

[2] R4-2501993, On simultaneous RxTx requirements, Skyworks Solutions, Inc., Nokia.