3GPP TSG RAN WG4 Meeting #114		R4-2502883

Athens, Greece, Feb 17th – 21st , 2025

Title: 	TP to TR 38.922 on parameters for 14.8 - 15.35GHz frequency range

Agenda Item:	7.2.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

This contribution merges TPs input to RAN4#114 on parameters for 14.8 – 15.35GHz frequency range including R4-2500774, R4-2501066 and R4-2502244 according to the worksplit.

# 2 Text Proposal

<Start of Change>

## 6.1	Co-existence study

### 6.1.1	Co-existence simulation scenarios

Table 6.1.1-1 summarizes the proposed initial simulation scenarios for 14800 - 15350 MHz.

Table 6.1.1-1: Summary of simulation scenarios for 14800 - 15350 MHz

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100/200 MHz | Same as aggressor | DL to DL | 15 GHz | Indoor hotspot | Down-prioritized |
| 2 | eMBB | NR, 100/200 MHz | Same as aggressor | DL to DL | 15 GHz | Urban macro |  |
| 3 | eMBB | NR, 100/200 MHz | Same as aggressor | DL to DL | 15 GHz | Dense urban | Down-prioritized |
| 4 | eMBB | NR, 100/200 MHz | Same as aggressor | UL to UL | 15 GHz | Indoor hotspot | Down-prioritized |
| 5 | eMBB | NR, 100/200 MHz | Same as aggressor | UL to UL | 15 GHz | Urban macro |  |
| 6 | eMBB | NR, 100/200 MHz | Same as aggressor | UL to UL | 15 GHz | Dense urban | Down-prioritized |
| Note: It is agreed to down-prioritize the dense urban and indoor hotspot scenarios in this coexistence study, as urban macro scenario is the worst case among the three simulated scenarios and will drive the ACIR requirements. |  |  |  |  |  |  |  |

<next change>

### 6.1.2	Co-existence simulation assumption

#### 6.1.2.1	Network layout model

##### 6.1.2.1.1	Urban macro

Details on urban macro network layout model are listed in Table 6.1.2.1.1-1 and 6.1.2.1.1-2.

Table 6.1.2.1.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 450 m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20/0% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 6.1.2.1.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | Coordinated operation (0% Grid Shift) and Un-coordinated operation (100% Grid Shift) |  |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |

Figure 6.1.2.1.1-1: Coordinated operation

Figure 6.1.2.1.1-2: Uncoordinated operation

##### 6.1.2.1.2	Dense urban

Details on dense urban network layout model are listed in Table 6.1.2.1.2-1 and 6.1.2.1.2-2.

Table 6.1.2.1.2-1: Single operator layout for dense urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 65.03 m |  |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi |  |
| Shadowing correlation |  | Between cite: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 450m is assumed. |  |  |  |

Figure 6.1.2.1.2-1: Network layout for dense urban

Table 6.1.2.1.2-2: Multi operators layout for dense urban

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note 1 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

##### 6.1.2.1.3	Indoor

Details on indoor network layout model are listed in Table 6.1.2.1.3-1 and 6.1.2.1.3-2.

Table 6.1.2.1.3-1: Single operator layout for indoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | 50m x 120m, 12BSs |  |
| Inter-site distance |  | 20m | Single sector per site |
| BS antenna height |  | 3 m | Mounted on ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office |  |
| Shadowing correlation |  | NA |  |

Figure 6.1.2.1.3-1: Network layout for indoor

Table 6.1.2.1.3-2: Multi operators layout for indoor

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Coordinated operation (0% Grid Shift) |  |

#### 6.1.2.2	Propagation model

##### 6.1.2.2.1	Pathloss

The pathloss models are summarized in Table 6.1.2.2.1-1 and the distance definitions are indicated in Figure 6.1.2.2.1-1 and Figure 6.1.2.2.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 6.1.2.2.1-1.

| Figure 6.1.2.2.1-1: Definition of d2D and d3D for outdoor UTs | Figure 6.1.2.2.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs. |
| --- | --- |

Note that

(6.1.2.2-1)

Table 6.1.2.2.1-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 6 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| UMa | LOS | , see note 1 |  |  |
|  | NLOS | for |  | Explanations: see note 3 |
|  |  | Optional |  |  |
| UMi - Street Canyon | LOS | , see note 1 |  |  |
|  | NLOS | for |  | Explanations: see note 4 |
|  |  | Optional |  |  |
| InH - Office | LOS |  |  |  |
|  | NLOS |  |  |  |
|  |  | Optional |  |  |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by , where . Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

##### 6.1.2.2.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 6.1.2.2.2-1.

Table 6.1.2.2.2-1: LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMi – Street canyon | Outdoor users: Indoor users: Use d2D-out in the formula above instead of d2D |
| UMa | Outdoor users: where and Indoor users: Use d2D-out in the formula above instead of d2D |
| Indoor – Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

##### 6.1.2.2.3	O-to-I penetration loss

The Path loss incorporating O-to-I building penetration loss is modelled as in the following:

PL = PLb + PLtw + PLin + N(0, σP2)

where PLb is the basic outdoor path loss given in Section 6.1.2.2.1. PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP  is the standard deviation for the penetration loss.

PLtw is characterized as:

is an additional loss is added to the external wall loss to account for non-perpendicular incidence;

, is the penetration loss of material i, example values of which can be found in Table 6.1.2.2.3-1.

pi is proportion of i-th materials, where ; and

N is the number of materials.

Table 6.1.2.2.3-1: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2⋅f |
| IRR glass | LIRRglass=23+0.3⋅f |
| Concrete | Lconcrete=5+4⋅f |
| Wood | Lwood=4.85+0.12⋅f |
| Note: f is in GHz |  |

Table 6.1.2.2.3-2 gives PLtw, PLin and σP  for two O-to-I penetration loss models. The O-to-I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 6.1.2.2.3-2 O-to-I penetration loss model

|  | Path loss through external wall: PLtw [dB] | Indoor loss: PLin [dB] | Standard deviation: σP [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 | 0.5d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for RMa, UMa and UMi-Street Canyon. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models, and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world.

The pathloss incorporating O-to-I car penetration loss is modelled as in the following:

PL = PLb + N(μ, σP2)

where PLb is the basic outdoor path loss given in Section 6.1.2.2.1. μ = 9, and σP = 5. Optionally, for metallized car windows, μ = 20 can be used. The O-to-I car penetration loss models are applicable for at least 0.6-60 GHz.

#### 6.1.2.3	Antenna and beam forming pattern modelling

##### 6.1.2.3.1		BS antenna modelling

|  | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| --- | --- |
|  | AEθ,φ=GE,max+Aθ,φ |
|  | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
|  | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 vm=expj2πm-1dv,subλcosθ |
|  | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
|  | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |

The antenna model for AAS BS is described in subclause 7.1.

In Table 6.1.2.3.1-1, representable parameter sets relevant for an AAS base station operating within 14800 - 15350 MHz are provided.

Table 6.1.2.3.1-1: Antenna array parameters

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 5 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 4 x 4 | 16 x 24 | 16 x 24 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V (Note 5) | 0.5 of wavelength for H, 2.8 of wavelength for V (Note 5) |
| Number of element rows in sub-array | N/A | 4 | 4 |
| Vertical element separation in sub-array (dv,sub) | 0.5 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 8 | 17.2 | 7.2 |
| Base station horizontal coverage range (degrees) | +/-90 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 0-180 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | N/A(Note 6) | 6 | 6 |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° i.e. 90° being at the horizon Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array assumes 16 × 24 sub-arrays and 2 polarizations for urban macro and dense urban cases; the conducted power per element assumes 4 x 4 elements and 2 polarizations for indoor case Note 4: 16 × 24 means there are 16 vertical and 24 horizontal radiating sub-arrays for urban macro and dense urban cases; 4 x 4 means there are 4 vertical and 4 horizontal radiating elements for indoor case. Note 5: For the case of 4 elements per sub array, dv will be 2.8 wavelengths. Note 6: Roof mounted equipment with antenna boresight pointing downward |  |  |  |

##### 6.1.2.3.2		UE antenna modelling

For the FR1-like RF design, the UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming (only for simulation and not reflecting deployment, coverage, implementation, or requirement aspects).

|  | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=25dB |
| --- | --- |
|  | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=25dB |
|  | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |

#### 6.1.2.4	Other simulation parameters

Table 6.1.2.4-1: Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Channel bandwidth | 100/200 MHz | 100/200 MHz | 100/200 MHz |
| Scheduled channel bandwidth per UE (DL) | 100/200 MHz | 100/200 MHz | 100/200 MHz |
| Scheduled channel bandwidth per UE (UL) | 100/200 MHz | 100/200 MHz | 100/200 MHz |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | 1 UE per slot (first priority) 3 UE per slot (second priority) | 1 UE per slot (first priority) 3 UE per slot (second priority) | 1 UE per slot (first priority) 3 UE per slot (second priority) |
| Traffic model | Full buffer | Full buffer | Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| BS max TX power in dBm (2 polarizations) | 23dBm | 46dBm | 36dBm |
| UE max TX power in dBm | 23dBm | 23dBm | 23dBm |
| UE min TX power in dBm | -40dBm | -40dBm | -40dBm |
| BS Noise figure in dB | 16 | 8 | 13 |
| UE Noise figure in dB | 13 | 13 | 13 |
| Handover margin | 3dB | 3dB | 3dB |

<next change>

### 6.1.3	Co-existence simulation results

#### 6.1.3.1	Urban Macro scenario

##### 6.1.3.1.1	Downlink

For Urban macro scenarios, simulation results for DL throughput loss with baseline assumption ACLR = 45 dBc at BS and ACS = 33 dBc at UE are summarized in Table 6.1.3.1.1-1/2/3/4/5. The following results are based on the FR1 like UE with omni-directional antenna and no beamforming.

Table 6.1.3.1.1-1: DL throughput loss for 14800 – 15350 MHz [Relative ACIR value]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia (R4-2417536) | Average throughput loss in % (15 GHz) | 0.17 |  | 0.28 |  | 0.45 |  | 0.70 |  | 1.06 | 1.55 |
|  | 5%-tile throughput loss in % (15 GHz) | 0.00 |  | 0.18 |  | 0.35 |  | 1.00 |  | 1.38 | 3.53 |
| ZTE | Average throughput loss in % (15 GHz) | 0.33 | 0.41 |  | 0.52 | 0.67 | 0.85 |  | 1.09 |  |  |
|  | 5%-tile throughput loss in % (15 GHz) | 2.50 | 2.75 |  | 3.41 | 4.38 | 4.97 |  | 5.89 |  |  |

Table 6.1.3.1.1-2: DL throughput loss for 14800 – 15350 MHz [Absolute ACIR value]

|  | ACIR (dB) | 5 | 10 | 15 | 17 | 18 | 19 | 20 | 21 | 25 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apple | Average throughput loss in % |  |  | 0.47 |  |  |  | 0.232 |  |  | 0.49 | 0.026 |
|  | 5%-tile throughput loss in % |  |  | 0.623 |  |  |  | 0.247 |  |  | 0.054 | 0.006 |
| CATT | Average throughput loss in % | 0.043 | 0.026 | 0.015 |  |  |  | 0.0082 |  | 0.0042 | 0.0023 |  |
|  | 5%-tile throughput loss in % | 0.16 | 0.15 | 0.14 |  |  |  | 0.11 |  | 0.10 | 0.046 |  |
| MediaTek | Average throughput loss in % | 7.41 | 4.61 | 2.74 |  | 1.97 |  |  | 1.39 | 0.86 | 0.44 |  |
|  | 5%-tile throughput loss in % | 35.10 | 22.03 | 12.45 |  | 8.57 |  |  | 5.90 | 3.78 | 1.70 |  |
| Vivo | Average throughput loss in % |  | 3.11 | 1.79 |  |  |  | 0.91 |  | 0.46 | 0.24 |  |
|  | 5%-tile throughput loss in % |  | 17.71 | 10.13 |  |  |  | 6.21 |  | 2.67 | 1.49 |  |
| Qualcomm | Average throughput loss in % | 6.4 | 3.8 | 2.2 |  |  |  | 1.2 |  | 0.7 | 0.4 |  |
|  | 5%-tile throughput loss in % | 34.5 | 20.4 | 10 |  |  |  | 5.4 |  | 2.5 | 1.5 |  |
| Ericsson | Average throughput loss in % |  |  |  | 0.0134 | 0.0121 | 0.01 | 0.009 | 0.008 |  |  |  |
|  | 5%-tile throughput loss in % |  |  |  | 8 | 7 | 6.24 | 5.25 | 4.6 |  |  |  |
| Huawei | Average throughput loss in % | 7.3 | 4.5 | 2.6 |  |  |  | 1.4 |  | 0.8 | 0.4 |  |
|  | 5%-tile throughput loss in % | 34.9 | 23.3 | 14.0 |  |  |  | 7.8 |  | 3.8 | 1.8 |  |
| Samsung | Average throughput loss in % | 4 | 3 | 2 |  |  |  | 1 |  | 0 | 0 |  |
|  | 5%-tile throughput loss in % | 34 | 20 | 12 |  |  |  | 7 |  | 3 | 2 |  |

<next change>

##### 6.1.3.1.2	Uplink

For Urban macro scenarios, simulation results for UL throughput loss with baseline assumption ACLR = 30 dBc at UE and ACS = 45 dBc at BS are summarized in Table 6.1.3.1.2-1/2/3/4/5. The following results are based on the FR1 like UE with omni-directional antenna and no beamforming.

Table 6.1.3.1.2-1: UL throughput loss for 14800 – 15350 MHz [Relative ACIR value]

| Company | Throughput loss | Relative ACIR offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -2 | -3 | -4 | -6 | -8 | -9 | -10 | -12 | -15 |
| Nokia (R4-2417536) | Average throughput loss in % (15 GHz) | 0.07 |  | 0.12 |  | 0.20 |  | 0.30 |  | 0.46 | 0.68 |
|  | 5%-tile throughput loss in % (15 GHz) | 1.14 |  | 2.21 |  | 3.52 |  | 3.55 |  | 3.61 | 3.73 |
| ZTE | Average throughput loss in % (15 GHz) | 0.26 | 0.30 |  | 0.34 | 0.39 | 0.46 |  | 0.55 |  |  |
|  | 5%-tile throughput loss in % (15 GHz) | 6.36 | 7.20 |  | 8.95 | 9.10 | 9.36 |  | 10.98 |  |  |

Table 6.1.3.1.2-2: UL throughput loss for 14800 – 15350 MHz [Absolute ACIR value]

|  | ACIR (dB) | 5 | 10 | 15 | 17 | 18 | 19 | 20 | 21 | 25 | 30 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apple | Average throughput loss in % |  |  | 0.47 |  |  |  | 0.232 |  |  | 0.49 | 0.026 |
|  | 5%-tile throughput loss in % |  |  | 0.623 |  |  |  | 0.247 |  |  | 0.054 | 0.006 |
| CATT | Average throughput loss in % | 0.043 | 0.026 | 0.015 |  |  |  | 0.0082 |  | 0.0042 | 0.0023 |  |
|  | 5%-tile throughput loss in % | 0.16 | 0.15 | 0.14 |  |  |  | 0.11 |  | 0.10 | 0.046 |  |
| MediaTek | Average throughput loss in % | 7.41 | 4.61 | 2.74 |  | 1.97 |  |  | 1.39 | 0.86 | 0.44 |  |
|  | 5%-tile throughput loss in % | 35.10 | 22.03 | 12.45 |  | 8.57 |  |  | 5.90 | 3.78 | 1.70 |  |
| Vivo | Average throughput loss in % |  | 3.11 | 1.79 |  |  |  | 0.91 |  | 0.46 | 0.24 |  |
|  | 5%-tile throughput loss in % |  | 17.71 | 10.13 |  |  |  | 6.21 |  | 2.67 | 1.49 |  |
| Qualcomm | Average throughput loss in % | 6.4 | 3.8 | 2.2 |  |  |  | 1.2 |  | 0.7 | 0.4 |  |
|  | 5%-tile throughput loss in % | 34.5 | 20.4 | 10 |  |  |  | 5.4 |  | 2.5 | 1.5 |  |
| Ericsson | Average throughput loss in % |  |  |  | 0.0134 | 0.0121 | 0.01 | 0.009 | 0.008 |  |  |  |
|  | 5%-tile throughput loss in % |  |  |  | 8 | 7 | 6.24 | 5.25 | 4.6 |  |  |  |
| Huawei | Average throughput loss in % | 7.3 | 4.5 | 2.6 |  |  |  | 1.4 |  | 0.8 | 0.4 |  |
|  | 5%-tile throughput loss in % | 34.9 | 23.3 | 14.0 |  |  |  | 7.8 |  | 3.8 | 1.8 |  |
| Samsung | Average throughput loss in % | 4 | 3 | 2 |  |  |  | 1 |  | 0 | 0 |  |
|  | 5%-tile throughput loss in % | 34 | 20 | 12 |  |  |  | 7 |  | 3 | 2 |  |

## 6.2	General parameters

### 6.2.1	Duplex mode

There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Even though FDD is not precluded, TDD is assumed as a baseline duplexing in this frequency range. SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [8].

### 6.2.2	Channel Bandwidth

While a number of channel bandwidth will be specified for this frequency range, 200 MHz is considered as a typical channel bandwidth. Other channel bandwidths compared to 200MHz are not precluded for this range.

### 6.2.3	Signal Bandwidth

The signal bandwidth for a 200MHz typical channel bandwidth signal is associated with both SCS and spectrum utilization, and can be calculated based on the spectrum utilization:

Signal bandwidth = NRB  x SCS x 12

with NRB: Number of Resource block for 200 MHz channel bandwidth and SCS to be specified when introducing new band(s) for this frequency range.

## 6.3	BS parameters

### 6.3.1	Transmitter characteristics

#### 6.3.1.1	Power dynamic range

#### There is no power control in downlink and fixed power per resource block is assumed during the study phase. Hence 0 dB power dynamic range was agreed for the LS reply [6].

#### 6.3.1.2	Spectral mask

Category A limits for Wide Area BS for AAS BS are listed in Table 6.3.1.2-1.

Table 6.3.1.2-1: Wide Area BS operating band unwanted emission limits for AAS BS (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic Limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz  f < 50MHz | -7dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz  f < min(100 MHz, fmax) | -14 dBm | 100 kHz |
| 100 MHz  f  fmax | -13 dBm | 1 MHz |
| NOTE: fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

#### Note: AAS BS Category B limits for Wide Area BS are not yet specified by regional regulatory bodies.

#### 6.3.1.3	ACLR

The ACLR should be higher than the value specified in Table 6.3.1.3-1.

Table 6.3.1.3-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 200 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 30 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 30 dB |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

#### 6.3.1.4	Spurious emissions

The Category A spurious emission limits applicable are extracted from TS 38.104 [9], as listed in Table 6.3.1.4-1.

Table 6.3.1.4-1: General BS transmitter spurious emission limits for 14800 - 15350 MHz operation (Category A)

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

#### Note: AAS BS Category B limits are not yet specified by regional regulatory bodies.

#### 6.3.1.5	Maximum output power

The maximum base station output power/ sector (e.i.r.p.) was provided in the antenna parameter table of the LS [6], as extracted in Table 6.3.1.5-1. It was agreed to be aligned with antenna characteristics.

Table 6.3.1.5-1: Maximum BS output power in 14800 to 15350 MHz

|  | Macro suburban | Macro urban | Micro urban | Indoor Hotspot |
| --- | --- | --- | --- | --- |
| Maximum base station output power/sector (e.i.r.p.) (dBm) | 84.3 | 84.3 | 74.3 | 40.0 |

The Total Radiated Power (TRP) for two polarizations was agreed as shown in Table 6.3.1.5-2 below.

Table 6.3.1.5-2: Total Radiated Power

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Indoor Hotspot |
| --- | --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 46 | 46 | 36 | 23 |

#### 6.3.1.6	Average output power

#### It was agreed the average output power won’t be mentioned in the reply LS in [6].

### 6.3.2	Receiver characteristics

#### 6.3.2.1	Noise figure

The BS typical noise figure is listed in Table 6.3.2.1-1, and BS class definitions can be seen in [9], § 4.4.

Table 6.3.2.1-1: Noise figure

| BS class | Noise figure (dB) |
| --- | --- |
| Wide Area | 8 |
| Medium Range | 13 |
| Local Area | 16 |

#### 6.3.2.2	Sensitivity

The OTA sensitivity requirement will be based on the NF and the antenna gain:

EISREFSENS=PkT+10*log10BW+NF+IM+SNR-G    (dBm)

Where:

-	BW is the configured bandwidth of the FRC,

-	NF is the noise figure,

-	IM is implementation margin not related to antenna array,

-	SNR is the required SNR to reach 95% throughput, and

-	G is the antenna gain including RF losses and 3dB off peak margin.

However, the sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter in LS [6].

#### 6.3.2.3	Blocking response

The out-of-band blocking requirement applies from 30 MHz to 2nd harmonic of the upper frequency edge of the operating band, excluding the in-band blocking frequency range, but including the downlink frequency range in case of an FDD operating band as listed in Table 6.3.2.3-1. It is agreed to adopt ΔfOOB = 100 MHz for AAS BS.

Table 6.3.2.3-1: OTA out-of-band blocking performance requirement for 14800 – 15350 MHz operation

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.36 | CW |
| 12750 to FUL,low – 100 | EISREFSENS + 6 dB | 0.1 | CW |
| FUL,high + 100 to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | 0.1 | CW |

The out-of-band blocking conductive requirements and in-band blocking requirements should be further studied in the WI phase.

#### 6.3.2.4	ACS

#### According to the simulation results in clause 6.1.3, it is agreed to specify 30 dB ACS.

<next change>

## 6.4	UE parameters

### 6.4.1	Transmitter characteristics

#### 6.4.1.1	Power dynamic range

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. Based on the study in sub-clause 6.5.2, the UE antenna type for 14800 – 15350 MHz can reuse existing FR1 antenna characteristics so that both nominal maximum and minimum output power refer to existing FR1 bands, i.e., power dynamic range is 56 dB with power class 3 (i.e. 23 dBm maximum output power) UE.

#### 6.4.1.2	Spectral mask

The UE spectral mask is described in TS 38.101-1 [12] clause 6.5.2.2 Table 6.5.2.2-1. Emission mask for 200 MHz BW is based on the CA mask as defined in TS 38.101-1 [12] clause 6.5A.2.2. The SEM requirements in this frequency range may be relaxed than FR1 intra-band CA with 200MHz aggregated bandwidth in the WI phase.

#### 6.4.1.3	ACLR

Considering that the UE needs to meet the Occupied bandwidth (OBW) requirements which is higher than the ACLR needed for co-existence as noted in sub-clause 6.1.3, it was concluded that 24 dB ACLR would be sufficient for 14800 – 15350 MHz. Thus, ACLR of 24 dB can be considered for the frequency range 14800 – 15350 MHz.

The actual ACLR for this frequency range or parts there-of should be further studied in the WI phase.

#### 6.4.1.4	Spurious emissions

The general spurious emissions defined in TS 38.101-1 [12] clause 6.5.3 Table 6.5.3.1-2 with OOB boundary given in Table 6.5A.3.1-1 can apply to the frequency range 14800 -15350 MHz.

#### 6.4.1.5	Maximum output power

Based on the study in sub-clause 6.5.2, the UE maximum output power for the considered frequency ranges could be 23 dBm. Other UE power classes, e.g., 20dBm, 26dBm and 29dBm, are not precluded (corresponding ACLR limit will be adapted accordingly to avoid additional interference).

#### 6.4.1.6	Average output power

NOTE:	This parameter was not mentioned in the previous response to ITU-R WP5D.

### 6.4.2	Receiver characteristics

#### 6.4.2.1	Noise figure

A noise figure of 11 dB was assumed for coexistence simulation in sub-clause 6.1.2. For the frequency range 14800 – 15350 MHz noise figure of 13 dB can be reused.

The actual noise figure for this frequency range or parts there-of to define RF requirements should be further studied in the WI phase.

#### 6.4.2.2	Sensitivity

The sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter.

#### 6.4.2.3	Blocking response

The blocking characteristic specified in clause 7.6A Tables 7.6A.2.1-1 and 7.6A.3-1 Bandwidth Class C and clause 7.7 Table 7.7A-1 Bandwidth class C for spurious response of TS 38.101-1 [12] could be applied for 200 MHz BW requirements of the frequency range 14800 – 15350 MHz.

The actual requirements for this frequency range or parts there-of may differ depending on the band plan and possible re-use of RF hardware components.

#### 6.4.2.4	ACS

According to the previous studies and simulation results in sub-clause 6.1.3, adjacent channel selectivity (ACS) is agreed as 24 dBc for 14800 – 15350 MHz.  Thus, ACS of 24 dB can be considered for the frequency range 14800 – 15350 MHz.

The actual ACS for this frequency range or parts there-of should be further studied in the WI phase.

## 6.5	Antenna characteristics

### 6.5.1	BS antenna characteristics

#### 6.5.1.1		Antenna model

The antenna model for AAS BS is described in subclause 7.1.

#### 6.5.1.2	Antenna parameters

In Table 6.5.1.2-1, representable parameter sets relevant for an AAS base station operating within 14800 - 15350 MHz are provided.

Table 6.5.1.2-1: Antenna array parameters

|  |  | Macro suburban | Macro urban | Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Table 3 |  |  | N/A |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 | 5 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | 90º for H 90º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 16x24 | 16x24 | 16x24 | 4x4 |
| 1.7 | Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 2.8 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V | 0.5 of wavelength for H, 2.8 of wavelength for V | 0.5 of wavelength for H, 0.5 of wavelength for V |
| 1.7a | Number of element rows in sub-array | 4 | 4 | 4 | N/A |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength for V | 0.7 of wavelength for V | 0.7 of wavelength for V | N/A |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | 3 | 3 | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | ## 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 17.2 | 17.2 | 7.2 | 8.0 |
| 1.10 | Base station horizontal coverage range (degrees) | ±60 | ±60 | ±60 | ±90 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 | 0-180 |
| 1.12 | Mechanical down-tilt (degrees) | 6 | 6 | 6 | N/A |
| 1.13 | Base station output power/sector (e.i.r.p.) (dBm) (Note 7) | 84.3 | 84.3 | 74.3 | 40.0 |

Note 1:	The vertical coverage range is given in global coordinate system, i.e., 90° being at the horizon. This range includes the mechanical down-tilt given in row 1.12.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.8 and is per polarization.

Note 3:	Conducted power values are per polarization. The conducted power per sub-array assumes 16 × 24 sub-arrays and 2 polarizations for the Macro Suburban, Macro Urban and Micro Urban cases; the conducted power per element assumes 4x4 elements for the Small cell indoor/ Indoor Urban case. This power is typical power, there is no upper limit for Wide Area Base station (For BS class definitions, see 3GPP TS 38.104 [1], § 4.4).

Note 4:	16 × 24 means there are 16 rows and 24 columns of radiating sub-arrays for Macro Suburban, Macro Urban and Micro Urban cases. 4x4 means there are 16 rows and 24 columns of radiating elements for Small cell indoor/ Indoor Urban case.

Note 5:   For the case of 4 elements per sub-array, dv will be 2.8 wavelengths.

Note 6:	The pre-set sub array down-tilt is a fixed design parameter for a base station. It is envisaged as a passive fixed (non-varying) electrical tilt within the sub-array elements.

Note 7:	The base station e.i.r.p per sector is calculated as total power (including power from two orthogonal polarizations).

Note 8:   Mechanical down-tilt is handled by a coordinate system transformation described in 3GPP TR 36.814 section A.2.1.6.2.

Note 9:   θetilt and φescan is the BS array antenna beam steering direction used in Table 3, they should be set so that the beam steering direction is within the vertical and horizontal coverage ranges in row 1.11 and row 1.10, respectively.

### 6.5.2	UE antenna characteristics

#### 6.5.2.1	General considerations

One of the key issues for this range is the UE RF architecture. While existing FR1 system parameters are defined for frequencies up to 7.1GHz, same FR1 principles are already assumed for e.g. 7.1-8.4GHz, which is however below the considered range of 14.8-15.3GHz. At the same time, the lower bound for the FR2 starts at 24.25GHz, which is notably higher than 15.3GHz. A choice for a particular UE RF architecture at these frequency ranges should consider implementation feasibility, e.g. physical constraints in case of handheld devices, and anticipated performance, such as coverage (as highlighted in Figure 6.5.2-1 in TR 38.820). The capabilities of PA technology in the FR2 frequency range motivated 3GPP to consider a UE architecture which relies on panels performing analog beam-forming, which was also considered as a feasible option from the handheld device implementation perspective. In contrast, panels and analog beam-forming are not feasible for lower FR1 frequencies.

In the context of the UE RF architecture, better UL coverage at FR2 frequencies was one of the key driving factors to consider the UE RF design with panels performing analog beam-forming. An FR2-like array antenna system incorporates analog beamforming into the array antenna to increase the coherent EIRP gain, thus helping to reduce the path loss between the UE and the base station. Furthermore, the FR2 array antenna, LNAs, PAs, beamformers, and PMIC are integrated into a single package, small form factor of which was also considered as a feasible option from the handheld device implementation perspective. As for the frequencies around 10-15GHz, previous work in TR 38.820 studied feasibility of FR2 principles in hand-held devices operating in that frequency range. One of the main challenges is the size of potential FR2 panel and, as noted in TR 38.820, "… we can see that the antenna module doubles in size going from 28 GHz to 20 GHz and further doubles at 14 GHz". Figure 6.5.2.1-1 presents an example of the patch width and ground plane extension based on the microstrip patch array design assuming four elements with half-wavelength spacing.

Figure 6.5.2.1-1: Patch array dimensions vs. frequency

For more detailed information on the potential array antenna size operating at the considered frequency of 15GHz, Figure 6.5.2.1-2 below presents several example cases with different antenna arrangements accounting for the typical spacing between elements of the array antenna of half wavelength. As can be seen from the figure for a typical inter-element antenna spacing, the total antenna array size becomes much larger comparing to the FR2 sizes.

Figure 6.5.2.1-2: Estimated antenna size at 15GHz

Figure 6.5.2.1-3 below illustrates an example of a survey of the internal area used by four different commercially available 5G devices for FR1 (based on individual company contributions in RAN4).  The FR2 area is calculated based a single 4x1 reference design, the configuration used as the basis for current FR2 spherical coverage requirements.  Additionally, an estimate of the required 12-15GHz antenna array area is provided for comparison, similarly based on a 4x1 microstrip patch array with half-wavelength spacing. So, assuming dedicated apertures, an estimated 4x1 array at 12-15 GHz would represent an additional area growth on the order of 30-70% relative to antenna area usage currently needed to support FR1.

Figure 6.5.2.1-3: A survey of internal area usage for FR1, FR2 and estimated 13-15GHz antennas

In addition, support of analog beamforming in 15GHz might impose a substantial growth and volume requirements dedicated for cellular antennas.  Furthermore, the nature of such an array might constrain the dimensions leading to implementation challenges in densely integrated consumer devices that can be of different form factors. For instance, Figure 6.5.2.1-4 shows an example of the PCB of a foldable phone, from which one can see that it might be challenging to mount single patch antennas on four edges or the backside of the smartphone considering the larger array size and the already-packed smartphone space. In other words, it can be difficult for a hand-held to assume that there will be extra space in all required physical dimensions making it challenging, at least from the packing perspective, to put FR2-like panels supporting the 15GHz range.

Figure 6.5.2.1-4: Main PCB of a foldable mobile phone.

#### 6.5.2.2	Design with patch antennas

A possible design approach is the adoption of patch antennas (i.e., FR1-like design) which gives more flexibility to the OEM vendors because even if a device implements more than one Tx, there is an additional flexibility on where Tx antennas can be placed. Furthermore, more than one Tx antenna can attain coherent EIRP gain through the corresponding placement of co-directional and co-located antennas when coherent Tx is implemented. Each antenna in such FR1-like system is linked to its own digital chain, allowing for easy support of coherent Tx. It can also reduce the PA power requirement for each antenna element, thus enhancing the overall power consumption of the UE at 15GHz, and it can help increase the coverage distance between the base station and the UE. For instance, Figure 6.5.2.2-1 presents an example of a general mobile device form factor with two patch antennas placed within the device as presented at the distance corresponding to approximately three wavelengths.

Figure 6.5.2.2-1: Exemplary placement of two patch antennas within a mobile device.

In this example, each patch antenna gain is around 2dB with the Tx power 23dBm. Additionally, Figure 6.5.2.2-2 below presents separately distribution of antenna and EIRP gains for the considered example antennas, where the baseline curve is only one antenna. Gain values are obtained by probing all points in 3D space around considered antennas, which allows us to find the best beam at each point of space independent of the base station location. In other words, these figures illustrate possible gain values, including the best one, regardless of the phone orientation with respect to the base station. As can be seen from this example, from the antenna gain perspective, the average gain comparing to one Tx is around 2.5dB. From the EIRP perspective, the average gain comparing to one Tx is around 5.5dB, which comprises 2.5dB antenna gain and 3dB coming from the fact that there are two Tx antennas each transmitting at 23dBm.

Figure 6.5.2.2-2: Antenna and EIRP gain for the considered two Tx antennas.

An additional aspect for the FR1-like antenna design is the resulting radiation pattern. It is worth noting that unlike at the antenna and EIRP gain, which are not very sensitive for the actual placement of Tx antennas, the radiation pattern may vary a lot. For instance, Figure 6.5.2.2-3 below presents an example resulting beam pattern from two Tx antennas physical location of which is the same as in Figure 6.5.2.2-1. For the sake of better clarity, we present the resulting radiation pattern in 2D and 3D spaces. Since two Tx antennas are placed at the back side of the phone, the resulting radiation pattern has a big negative gain in the opposite direction.

Figure 6.5.2.2-3: Radiation pattern for two Tx antennas presented in Figure 6.5.2.x-2.

On the Rx side, more than one discrete/ FR1-like antenna with digital beamforming can maximize the received signal power to improve the cell edge performance. For a cell-center user, instead of maximizing the received signal power, all antennas can be used jointly to achieve spatial multiplexing gain.

#### 6.5.2.3	Design with metal frame antennas

A simulation with two general metal frame Tx antennas was conducted to estimate the antenna gain and performance for cases of single-Tx and multi-Tx without beam steering assumptions. The antenna placement for the evaluation is captured in 6.5.2.3-1.

Figure 6.5.2.3-1: Exemplary placement of metal frame antennas within a mobile device.

With the assumption above, simulation results of the above example related to antenna performance are summarized in Figure 6.5.2.3-2 and Table 6.5.2.3-1, which also capture radiation efficiency and directivity in terms of peak antenna gains of each scenario.

Figure 6.5.2.3-2: CDF of peak antenna gains for metal antennas

Table 6.5.2.3-1: Peak antenna gains for one-Tx and two-Tx

|  | Ant1 | Ant2 | Ant3 |
| --- | --- | --- | --- |
| Gain (dB) | 3.9 | 4.8 | 2.7 |
| Rad. Efficiency (dB) | -6.3 | -4.8 | -6.3 |
| Directivity (dB) | 10.2 | 9.6 | 9.0 |
|  | Ant1 + Ant2 | Ant1 + Ant 3 | Ant2 + Ant3 |
| Combined gain (dB) | 3.8 | 2.6 | 3.9 |
| Gain drop (dB) | -1.0 | -1.3 | -0.9 |

As seen from Figure 6.5.2.3-2 and Table 6.5.2.3-1, in terms of the peak antenna gain with metal frame antennas, it is shown that for the considered example, the single-Tx antenna gain can reach up to 4.8 dB with its directivity and efficiency characteristics at the frequency range around 15 GHz. However, for the case of two-Tx, it is shown that the peak gain of combined two Tx antennas does not show better performance, but rather drops compared to the single-Tx case for the antenna placement in Figure 6.5.2.3-1. This would be coming from two different radiation patterns as depicted in Figure 6.5.2.3-3.

Figure 6.5.2.3-3: Simulated radiation patterns for one-Tx and two-Tx cases

The radiation pattern of antennas in a practical device has different peak direction compared with the boresight direction since the UE ground has much longer electric strength at this frequency range as shown in Figure 6.5.2.3-3. For example, Ant2 would form its peak direction toward Ant3, and similarly, Ant1 has the peak gain at the opposite direction toward the bottom side of UE at this case. On the other hand, if two Tx antennas are adjacent with the same orientation, it may have increased peak antenna gain thanks to the same radiation pattern between two Tx antennas although it would face a different issue such as antenna isolation to keep the fundamental performance.

In this regard, for FR1-like design with metal frame antennas, similar to the design approach with FR1-like patch antenna case, it should be noted that multi-Tx antennas may not help improving peak gain performance unless their placements are not designed carefully in terms of both radiation patterns and isolation.

<End of Change>

# Reference

[1] RP-242945 TR 38.922 v1.0.0

[2] R4-2420385, “LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson