3GPP TSG-RAN WG4 Meeting #114	R4-2502935

Athens, Greece, 17-21 Feb 2025

Title: 	TP into TR 36.719-01-01, addition of CA_3A-20A-28A-40A

Agenda Item:	6.4.2

Source: 	Nokia, Telefonica

Document for:	Approval

# Introduction

This TP for TR 36.719-01-01 adds CA_3A-20A-28A-40A.

****************************************** Start of TP ******************************************

## 5.2	LTE-A inter-band CA for x (x>1) bands DL with 1 band UL

### 5.2.x	CA_3A-20A-28A-40A

#### 5.2.x.1	Channel bandwidths per operating band for CA

Table 5.2.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 5.2.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_3A-20A-28A-40A | - | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |

#### 5.2.x.2	Co-existence studies

Not applicable for this CA configuration.

#### 5.2.x.3	∆TIB and ∆RIB values

Since, there is only single band UL, ∆TIB,c is set to zero.

Table 5.2.x.3-1: IB,c

| CA_3-20-28-40 | 3 | 0 |
| --- | --- | --- |
|  | 20 | 0 |
|  | 28 | 0 |
|  | 40 | 0 |

Table 5.2.x.3-2: R IB,c

| CA_3-20-28-40 | 3 | 0 |
| --- | --- | --- |
|  | 20 | 0.1 |
|  | 28 | 0.1 |
|  | 40 | 0 |

#### 5.2.x.4	REFSENS requirements

Not applicable for this CA configuration.

****************************************** End of TP ******************************************