3GPP TSG-RAN WG4 Meeting #114	R4-2502980

Athens, Greek, 17th – 21st February, 2025

Title: 	WF on UE requirements for PHS protection

Agenda Item:	4.1.1

Source: 	KDDI, OPPO

Document for:	Approval

- Introduction

The moderator copies agreements and approved way forwards during the online session into this contribution in agenda 4.1.1. On top of that, we capture additional way forwards into this contribution.

Sub-topic 1-1 PHS protection update

- < Issue 1-1-1: A-MPR region and value for NS_05 PC3 >

Way forward:

- A-MPR region definition and values for NS_05 PC3 are as follows.

Table 1: A-MPR regions for NS_05 (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A1 |  | Region A2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LCRB | RBstart | LCRB | RBstart |
| 5 |  | N/A | N/A | N/A | N/A |
| 10 | < 1930.7 | ≥ 1.35*RBstart + 4.32MHz/12/scs | >0 | ≤ 2.16MHz/12/scs | ≤ 1.8MHz/12/scs |
| 15 | 1927.5 ≤ fc < 1932.5 | ≥ RBstart*1.2 + 4.68MHz/12/scs | >0 | ≤ 3.96MHz/12/scs | ≤ 3.6MHz/12/scs |
| 15 | 1932.5 ≤ fc < 1938.2 | ≥ 8.28/12/scs | ≤ 0.54/12/scs | ≤ 2.16MHz/12/scs | ≤ 2.16MHz/12/scs |
| 20 | < 1945.7 | ≥ RBstart*1.5 + 4.14MHz/12/scs; | >0 | ≤ 5.76MHz/12/scs; ≤ RBstart*1.5 + 4.14MHz/12/scs; | ≤ 4.68MHz/12/scs |

Table 2: A-MPR values for NS_05 (Power Class 3, 10MHz, Fc<1930.7MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3 | ≤ 2 |
|  | QPSK | ≤ 4 | ≤ 3 |
|  | 16 QAM | ≤ 4 | ≤ 3 |
|  | 64 QAM | ≤ 4.5 | ≤ 3.5 |
|  | 256 QAM | ≤ 5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 5 | ≤ 4.5 |
|  | 16 QAM | ≤ 5 | ≤ 4.5 |
|  | 64 QAM | ≤ 5 | ≤ 4.5 |
|  | 256 QAM | ≤ 6 | ≤ 6 |

Table 3: A-MPR values for NS_05 (Power Class 3, 15MHz, 1927.5≤fc<1932.5)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3 | ≤ 2.5 |
|  | QPSK | ≤ 4 | ≤ 3 |
|  | 16 QAM | ≤ 4.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 6 | ≤ 5 |
|  | 16 QAM | ≤ 6 | ≤ 5 |
|  | 64 QAM | ≤ 6 | ≤ 5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6 |

Table 4: A-MPR values for NS_05 (Power Class 3, 15MHz, 1932.5≤Fc<1938.2MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | ≤ 2 |
|  | QPSK | ≤ 3 | ≤ 2.5 |
|  | 16 QAM | ≤ 3 | ≤ 3 |
|  | 64 QAM | ≤ 3 | ≤ 3 |
|  | 256 QAM | ≤ 4.5 | ≤ 4 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 64 QAM | ≤ 4.5 | ≤ 4.5 |
|  | 256 QAM | ≤ 6 | ≤ 6 |

Table 5: A-MPR values for NS_05 (Power Class 3, 20MHz, Fc<1945.7MHz)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 3 |
|  | QPSK | ≤ 4 | ≤ 3.5 |
|  | 16 QAM | ≤ 5 | ≤ 4 |
|  | 64 QAM | ≤ 5 | ≤ 4 |
|  | 256 QAM | ≤ 5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 6 | ≤ 5.5 |
|  | 16 QAM | ≤ 6 | ≤ 5.5 |
|  | 64 QAM | ≤ 6 | ≤ 5.5 |
|  | 256 QAM | ≤ 7 | ≤ 6 |

- < Issue 1-1-2: A-MPR region and value for NS_05 PC2 >

Way forward:

- Continue further discussion based on more inputs in next meeting.

- < Issue 1-1-3: A-MPR region and value for NS_05U PC3 >

Way forward:

- Continue further discussion based on more inputs in next meeting.

- < Issue 1-1-4: A-MPR region and value for NS_05U PC2 >

Way forward:

- Continue further discussion based on more inputs in next meeting.

- < Issue 1-1-5: How to align the modifiedMPR-Behaviour among releases >

Way forward:

- Continue further discussion in next meeting.

- Reference

[1] AMPR region definition for NS_05 PC3 v3.2_AfterAd-hoc.pptx, OPPO, RAN4#114