3GPP TSG-RAN4 Meeting #114	R4-25002995

# Athens, Greece, 17th- 21th, Feb, 2025

Source:	Samsung, Boost Mobile Network, Qualcomm, Fujitsu

Title:	TP for TR 38.750 CA_n29-n71 with PC2 FDD single UL

Agenda item:			6.8.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.750 to include CA_n29-n71 according to the request in [1].

PC3 is already in spec.

2. Reference

[1] RP-242866 Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	CA_n29-n71

### 5.x.1	UE maximum output power

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz)(NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n29A-n71A | n718 | n29 | 5, 10 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

NOTE 8:   Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

### 5.x.2	Reference sensitivity requirements

#### 5.x.2.0	General

There is cross band isolation issue due to the proximity of two bands.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n71

REFSENS for PC2 1Tx and 2TX is calculated assuming 6dB imbalance between PRX and DRX interference, adding 3dB into PC3 interferences for 1TX case, and adding 0.5dB RevIM for 2TX case.

Table 5.x.2.1-1: Reference sensitivity exceptions of cross band isolation

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 20.46 23.37 | ACLR2 |
| NOTE 6: Applicable to UE’s supporting PC2 with 1 Tx NOTE 7: Applicable to UE’s supporting PC2 with 2 Tx |  |  |  |  |  |  |  |  |  |

### 5.x.3	∆TIB and ∆RIB values (Void)

<End of Text Proposal>