3GPP TSG-RAN WG4 Meeting #114		R4-2503012

Athens, Greece, 17th – 21st February, 2025

Agenda item:	7.19.5

Source: 	Qualcomm Incorporated

Title: 	AI/ML ad-hoc meeting minutes-1

Document for:	Approval

# Introduction

The discussion on the AI/ML study is organized under two threads [131] and [132] in RAN4#114. The ad-hoc meeting will discuss some of the topics from the moderator summary in [1].

# Discussion

## Requirements for Beam management

### Sub-topic 2-1

Beam prediction KPIs

Different metrics/KPIs have been discussed and were captured in the TR:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered

-	Option 1: RSRP accuracy

-	Option 2: Beam prediction accuracy

-Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

-Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

-Related measurement accuracy can be considered to determine x

-	Option 4: combinations of above options

Issue 2-1: Metrics/KPIs for beam prediction

Proposals

  - Option 1:

    - In case of the AI/ML model with label (beam ID) output type

      - Beam prediction accuracy, i.e., Option 2 + Option 3 is adopted.

    - In case of the AI/ML model with label (beam ID) + RSRP output type

      - Beam prediction accuracy and RSRP accuracy, i.e., Option1, 2 an 3, are adopted simultaneously.

  - Option 2: Option 2 with a combination of Top-K/1 and Top 1/K

    - Option 2a: update option 2 to Top-N/K, the percentage of "the Top-N genie-aided beam is one of the Top-K predicted beams".

  - Option 3: Option 3 as it does not have the issue of UE failing if there are multiple beams within a small RSRP margin

  - Option 4: For beam prediction accuracy: A beam ranking metric should be used that considers the cumulative beam ranking other. Potential options are:

    - (1)   Kendall/Spearman correlation

    - (2)   Normalized Discounted Cumulative Gain (nDCG)

    - (3)   Rank-biased overlap

  - Option 5:combinations of options, concrete proposals should be disucssed

- Recommended WF

  - To be discussed

The number of options should be downselected to make some progress or a clear path to choose an option should be discussed/agreed

Discussion:

Nokia: options are not mutually exclusive. Option 1 is ok, it should be beam ID prediction accuracy. Option 2 would also be ok. Option 1 and Option 2 are both ok.

Moderator: beam prediction accuracy refers to beam ID prediction

Qualcomm: Option 1 is the option 1 from the proposals, not the original option 1 already agreed. We have 2 options for beam ID, one is about the RSRP difference between prediction and ieal measurement. If we only focus on prediction ID, the UE could fail a test if the beams are close. If we set X dB margin properly, this enables a good requirement.

E///:option 2 means we have to test both 1/k and K/1 at the same time? RAN4 can choose one of them.

Oppo: we had the original options mostly taken from RAN1, we could do some downselection as needed. No need necessarily to combine. for proposed Option 1, it might conflict with original options. For Option 3, this is ground truth, not predictied

QC: yes, it is ground truth.

Vivo: option 2 + option 3 should be clarified. Option 3 should solve the problem when several beams are close. For the new option 2, why do we need top 1/K and K/1? Combination is not needed in our opinion. Option 2a can solve the problem when several beams are close.

Samsung: we share similar view as oppo and vivo, combining options 2 and 3 might not work. Shouldn’t we wait for RAN1 conclusion on this? No need for us to define.

Xiaomi: Option 2 have the same K, we do not need the same number. A larger K will relax the requirement. Option 2 and Option 3 both test beam IDs.

Nokia: Option 3 basically verifies what is the success rate of top 1 beam with the X dB margin. This is not sufficient because it only verifies a single beam, not all. It should be in combination with all sorts

Qualcomm: in terms of beam ID prediction, the 2nd sub-bullet was not really investigate.

DCM: in order to satisfy this new 2nd sub-bullet. Option 1 can solve this problem. Option 3 can satisfy the difference between best beam and other beams. Option 1 can satisfy the difference between prediction and ground truth.

Apple: why do we need to define a different framework compared to legacy. If we are talking about beam index, we can get top 1/K or K/1. We only have an accuracy requirement. Overall proposal from us, we should take the current requirement as baseline.

Tejas: we do agree with Nokia’s second comment, the KPI should also verify prediction for other beams. Prediction should be consistent with the ideal value.

Qualcomm: if we only define RSRP and no beam ID, there could be a problem that UE only reports a beam which is not one of the strongest but RSRP prediction is accurate. RAN1 already agreed to have some beam ID reported for monitoring.

Nokia: absolute RSRP would be enough if RSRP is always predicted. This might not be the case

Apple: this situation QC described can happen for existing requirements. Do we need to define so many requirements.

Oppo: can we do downselection? Remove option 3 and add margin to option 2.

Samsung: for beam ID prediction, it is useful for performance monitoring.

Agreement:

- Option 1 (RSRP accuracy) will be one of the KPIs if predicted RSRP is reported

  - How option 1 is applied is FFS (if it applies to beams that UE should have predicted or not)

- FFS whether a combination of Option 2 and/or Option 3 or modified Option 2/3 will also be a KPI

To be checked until end of meeting

### Sub-topic 2-2

Relative RSRP accuracy

Relative RSRP accuracy has been discussed for a few meetings without any clear agreement

Issue 2-2: Relative RSRP accuracy

- Proposals

  - Option 1: Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest RSRP among the predicted beams.

- Option 2: Relative RSRP accuracy=predicted L1-RSRP of beam index i - ground truth of the beam index n ,

where the beam index n owns the largest value

  - Option 3: Relative RSRP is the RSRP difference between the predicted RSRP values, FFS what the accuracy should be

  - Option 4: Others

- Recommended WF

  - To be discussed

Discussion:

### Sub-topic 2-3

Measurement period for inference

Requirements for the measurement period are discussed by multiple companies and should be defined as core requirements

Issue 2-3: Measurement period for inference

- Proposals

  - Option 1: Use “legacy” L1-RSRP measurement as baseline

    - FFS whether same values for M,N,P should be reused

  - Option 2: Differentiate case 1 and case 2

    - Reuse L1 measurement period for Case 1

    - Introduce configurable number of measurement occasions for case 2

  - Option 3: Study the measurement period based on simulations

  - Option 4: Others

- Recommended WF

  - Option 1

Discussion:

### Sub-topic 2-4

QCL source RS for TCI states

In order to further discuss the conditions for a predicted TCI state to be known/unknown, QCL conditions should be clarified

Issue 2-4: QCL Source RS for TCI states

- Proposals

  - Option 1: send LS to RAN1 to ask whether the RS resource configured in Set A (but not in Set B) can be used as the QCL source RS for a TCI state

  - Option 2: wait for RAN1 discussion without sending any LS

  - Option 3: this information is not needed

- Recommended WF

  - Option 1

Discussion:

Agreement: Option 2

RAN1 is discussing this aspect

### Sub-topic 2-5

Requirements based on relative difference

In R4-2501957 it is proposed to define a set of requirements based on some relative difference between side conditions to ensure that the UE measurements are consistent.

Issue 2-5: Feasibility Intermediate KPIs for CSI requirements or LCM

- Proposals

  - Option 1: RAN4 to study how to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

  - Option 2: RAN4 should not spend time on such a study

  - Option 3: Companies are invited to bring a more concrete proposal on how such requirement would work before RAN4 would start a study

  - Option 4: others

- Recommended WF

  - Option 3

Discussion:

### Sub-topic 2-6

Simulation results

Issue 2-6:	Simulation results

- Proposals

  - Option 1: Discuss the simulation results based on summary – to be prepared by vivo

- Recommended WF

  - To be discussed

Discussion:

QC: we suggest to align the error model and go ahead with another round of simulation

Agreement:

Discussion offline to further align the error model and other assumptions/conditions

Interested companies to bring simulation results in future meetings based on the updated assumptions

Vivo to lead offline discusssons

### Sub-topic 2-7

Measurement error modeling

Several companies brought proposals on how to model the RF and baseband measurement error, the model should be further discussed

Issue 2-7: Measurement error modeling

- Proposals

  - Option 1: Measurement error to be discussed offline together with other simulation assumptions

- Recommended WF

  - To be discussed

Discussion:

## Beam management testability

Baseline test setup

The test setup has been discussed for multiple meetings without much progress. Some decisions should be made to limit the number of candidate setups.

Issue 4-1: Baseline test setup

- Proposals

  - Option 1: Single AoA setup (different Tx beams emulated only with power difference?)

    - Is it possible to test if UE can also make a prediction when set B beams come from different directions?

    - How many Tx beams can be emulated when the spatial domain properties are lost?

    - Will UE need just a simple interpolation model or is a more complex model necessary to pass such a test?

    - Can additional (L1-)RSRP measurement accuracy requirement (e.g. spherical coverage) be used to complement Single AoA testing with directional/spatial UE performance?

  - Option 2: Single AoA setup with “two stage approach”

    - Will a UE which uses Rx beam information in the prediction model have to also implement a model which does not use the Rx beam information?

    - Are there any implementation constraints on a UE to execute/pass this test?

    - How will the TE know the UE Rx beam pattern?

  - Option 3: multi AoA setup– RRM test setup as baseline (including enhancement for up to 4AoA active probes)

    - How many Tx beams can be emulated with such a setup, what is the relationship between number of probes and number of set B Tx beams?

    - Can AoAs between the probes also be emulated (is it possible to “make it look like” signals are coming from other angles than where the probes are placed?)

    - Can a simplified CDL be emulated or only TDL from each probe?

  - Option 4: multi AoA setup – 3D MPAC system as baseline

    - How many Tx beams can be emulated with the baseline setup?

    - 4Tx beams with dual pol(set B?) can be emulated with 2 additional probes

Pros&cons of different test setups

|  | Single AoA | Single AoA with two stage | Multi AoA – RRM test setup as baseline | Multi AoA 3D MPAC |
| --- | --- | --- | --- | --- |
| Tests that UE uses spatial information(UE actually makes measurement in multiple directions to make a prediction) | No | Maybe? | Yes | Yes |
| Relevance for field performance | Low | Maybe? | Good/High | Good/High |
| Simplified CDL emulation | No | Yes |  | Yes |
| Test setup complexity | Low | Low | Medium | Medium/High? |
| Test method complexity | Low | High | Medium | Medium/High? |
| Reuse of test setup for other tests | Medium (multi AoA testing already exists in RRM) |  | High? | Low/medium? Can only reuse the test setup for FR2 MIMO OTA? |

Companies are invited to propose further criteria which are relevant to BM prediction testing.

- Recommended WF

  - Do not further discuss single AoA test setups, continue to study the two multi AoA candidate setups

One option to continue the discussion proposed by R＆S is the following:

Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability

    - AI/ML BM performance centric, not CDL channel emulation centric.

    - Define test system candidates which are already feasible

Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance.

Discussion:

Option 1:

Apple: even for single AoA, UE would still need to do some beam sweeping.

Vivo: there could be different implications. UE may not do Rx beam sweeping.

Nokia: we have agreed that a UE would be rotated. UE needs to do sweeping.

Samsung: TE vendors can provide a dynamic range and we do some simulations to see how many beams can be emulated

Qualcomm: question to Samsung, this is dynamic range from the Tx side.

Samsung: yes.

Qualcomm: with option 2,will beam lock be used?

R&S: for PC3, the dynamic range is 35dB from the Tx side.  Single AoA can have multiple flavors, it depends on how the UE beam pattern is added or emulated in the test. This is feasible, it can go all the way to measuring the beam pattern.

Samsung: without beam lock UE would still need to beam lock.

Option 2:

R&S: this can have many flavors. This two stage can actually emulate CDL best.

### Sub-topic 4-2

Channel model

The channel model to be used has also been discussed for several meetings but not much progress has been made

Issue 4-2: Channel model

- Proposals

  - Option 1: simplified CDL

  - Option 2: TDL

  - Option 3: others

- Recommended WF

  - To be discussed

If simplified CDL is to be further considered, more detailed proposals on what to study/analyze further should be discussed (e.g. RX PAS, number of relevant clusters to emulate, dependency on number of clusters and number of probes in the chamber, etc)

Discussion:

### Sub-topic 4-3

Set B and Set A beams

The number of beams which needs to be emulated will be a requirement on the test setup

Issue 4-3: Set B and Set A beams

- Proposals

  - Option 1: set B: 16 beams, set A 128 beams. How many beams are need simultaneously?

  - Option 2: RAN4 perform system-level simulation to determine the value of X and Y (in Proposal 7):

    - To determine the value of X, by allowing a reasonable AI/ML prediction performance degradation (e.g., nearly 0dB absolute error degradation for RSRP accuracy) by only measuring  X strongest beams from the whole Set A beambook in the ideal Measurement Phase.

    - To determine the value of Y, by allowing a reasonable AI/ML prediction performance degradation (e.g., 1dB absolute error degradation for RSRP accuracy) by only measuring Y strongest beams from the whole Set B beambook.

    - Reasonable dynamic range of the beam signal strength level shall also be assumed.

  - Option 3: Others

- Recommended WF

  - To be discussed

Discussion:

### Sub-topic 4-4

Test setup requirements

The test setup has been discussed for multiple meetings without much progress. Some decisions should be made to limit the number of candidate setups

Issue 4-4: Option 4 for 2-sided model

- Proposals

  - Option 1: UE rotation during the test

    - multiple TRP (or reflections from another direction)

    - AoA – to be discussed later

    - AoD – to be discussed later

  - Option 2:

    - Other paramters

- Recommended WF

  - To be discussed

Discussion:

### Sub-topic 4-5

Positioning test feasibility for case 1

There is a debate on whether requirements for positioning case 1 should be defined or not, the testability of case 1 is also part of this discussion

Issue 4-5: Positioning case 1 testing

- Proposals

  - Option 1: Case 1 cannot be tested because UE does not report the position

  - Option 2: Case 1 cannot be tested because the ground truth(actual UE position) cannot be derived

  - Option 3: Testing is feasible by using PRUs

  - Option 4: others

- Recommended WF

  - To be discussed

Discussion:

# References

[1] R4-2500683, “Topic summary for [114][131] NR_AIML_air_part1”, Moderator(Qualcomm Incorporated), RAN4#114